| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0182-v3 |
| --- | --- |

| Title: | CE10-related: Simplification of local illumination compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chun-Chia Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, chunchia.chen, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes methods to simplify the local illumination compensation (LIC) in BMS2.2. Three tests are conducted. In Test 1, to reduce encoding time, two methods are included: 1) reusing the LIC-off motion search results for testing LIC-on, 2) disallowing bi-prediction with unequal coding unit (CU) weights and subblock-based temporal motion vector prediction (SbTMVP) when LIC is on, where the first method is encoder-only while the second method is normative. In Test 2, to alleviate the latency issue for LIC in hardware decoders, LIC is applied only with the offset (i.e., the weight is fixed to 1.0). In addition, Test 1 is included in Test 2. In Test 3, to improve the coding efficiency of Test 2 and keep the latency issue alleviated, top-and-left template, top template, and left template are used to derive the linear model parameters of LIC at the top left corner, top boundary, and left boundary of the current coding tree unit (CTU), and only the offset parameter is derived if the current template within the current CTU. Besides, Test 3 also includes Test 1. Results of these three tests compared with VTM3.0 are reported as follows:

Test 1: LIC fast encoding + disallowing bi-prediction with unequal CU weights when LIC is on
RA BD-rates = -0.40% (Y), -0.23% (U), -0.28% (V); EncT=105%; DecT=100%
LB BD-rates = -0.42% (Y), -0.36% (U), -0.33% (V); EncT=105%; DecT=100%

Test 2: Test 1 + offset-only LIC
RA BD-rates = -0.14% (Y), -0.22% (U), -0.24% (V); EncT=105%; DecT=101%
LB BD-rates = -0.05 % (Y), -0.31 % (U), -0.29 % (V); EncT=105%; DecT=101%

Test 3: Test 1 + adaptive template shape LIC at CTU boundary + offset-only LIC within CTU
RA BD-rates = -0.14% (Y), -0.17% (U), -0.15% (V); EncT=105%; DecT=101%
LB BD-rates = -0.11% (Y), -0.30% (U), -0.42% (V); EncT=106%; DecT=101%

# Introduction

In CE10.5 [1], LIC is tested. In LIC, templates are used to derive the linear model parameters for inter prediction. The current template in the current picture is composed of a top current template and a left current template, where the top current template and the left current template are composed of top and left neighbouring CU reconstructed samples, respectively. The reference template in the reference picture is composed of the current template’s motion compensated samples using the current CU motion vector (MV). Although LIC shows non-trivial coding gain, there are two problems caused by LIC: long encoding time in BMS2.2 encoder and long latency issue in hardware decoders.

The latency issue is stated as follows. In pipelined decoders, predicting a block (intra prediction or inter prediction) and reconstructing a previous block (inverse quantization, inverse transform, reconstruction) are usually done by different hardware circuits at the same time. An important principle for parallel processing is that there is no interaction in the same time slot between processing one block at one pipeline stage and processing its previous block at the next pipeline stage. The processing latency of reconstructing the previous block can be “hidden” by the pipeline design. In LIC, to derive the linear model parameters for inter prediction, the current CU has to wait for reconstructed samples in a previous CU, which violates the principle and the processing clock cycles of reconstructing previous CU samples cannot be “hidden” in a pipeline fashion in hardware decoders. This is a serious issue for LIC to be adopted.

This contribution proposes methods to mitigate the two problems.

# Proposed methods

## Test 1: Reducing encoding time

In BMS2.2, sum of absolute differences (SAD) and mean removed SAD (MR-SAD) are employed during the motion search for LIC-off and LIC-on cases, respectively [3]. It is proposed to reuse the motion search results of LIC-off for LIC-on. This change is encoder-only.

In addition to reusing motion search results, it is proposed to disallow bi-prediction with unequal coding unit (CU) weights (also named generalized bi-prediction, GBi, before; also named bi-prediction with weighted averaging, BWA, in JVET-L1002) and SbTMVP when LIC is on. This change is normative.

## Test 2: Test 1 + offset-only LIC

Test 2 is built on top of Test 1, so the two encoding time reduction methods from Test 1 are included.

To alleviate the latency issue, it is proposed to apply LIC only with the offset (i.e., the weight is fixed to 1.0). The offset β is calculated as the average of differences between reconstructed samples of the current template and the reference template. As formulated in Eq. (1), the offset β is

| β=C(n)-RnN | (1) |
| --- | --- |

where C(n) and Rn are the current template samples and the reference template samples, respectively, and N is the number of samples used for LIC parameter derivation in the template.

As depicted in Table 1, the complexity of the proposed offset-only LIC is significantly reduced. Note that the division operation in offset derivation can be replaced with a shift. Though the principle for parallel processing is still violated, the processing cycles for deriving parameters are significantly reduced, which can be helpful.

Table 1.  Number of operations used in the linear model derivation

| Operations | LIC in BMS2.2 | Offset-only LIC |
| --- | --- | --- |
| Multiplications | 2N + 2 + 2 | 0 |
| Additions | 4N + 3 | 2N + 1 |
| Divisions | 1 | 0 |

## Test 3: Test 1 + adaptive template shape LIC at CTU boundary + offset-only LIC within CTU

Test 3 is built on top of Test 1, so the two encoding time reduction methods from Test 1 are included.

To alleviate the latency issue and improve the coding efficiency of Test 2, it is proposed to apply LIC with different linear model derivation methods according to the spatial position of the current template:

- If the top current template is in the above CTU row and the left current template is in the left CTU column, both the top and left templates are used to derive the weight and offset parameters.

- Else if the top current template is in the above CTU row, only the top template is used to derive the weight and offset parameters.

- Else if the left current template is in the left CTU column, only the left template is used to derive the weight and offset parameters.

- Else (the top current template is not in the above CTU row and the left current template is not in the left CTU column), only the offset parameter is derived by using the top and left templates.

For example, as shown in Figure 1, the CUs, “c” and “d”, use top template to derive the weight and offset parameters. The CUs, “b”, “g”, and “i”, use left template to derive the weight and offset parameters. The CU, “a”, use both top and left templates to derive the weight and offset parameters. For those CUs marked with slash lines, “e”, “f”, “h”, “j”, and “k”, only the offset parameter is derived. With this method, since the reconstructed samples from the above CTU row and left CTU column are already ready when the current CTU is processed in hardware designs, the latency issue on reconstructed samples is effectively solved.

Figure 1.  An example of applying LIC with different linear model derivation methods according to the spatial position of the current template

# Experimental results

The proposed simplification methods have been implemented on VTM3.0 [2]. Table 1, Table 2, and Table 3 show the results of Test 1, Test 2, and Test 3, respectively. The simulations were performed following the JVET common test conditions (CTC) [4].

Table 1.  Results of Test 1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.60% | -0.47% | -0.68% | 103% | 98% |
| Class A2 | -0.28% | -0.06% | -0.08% | 106% | 100% |
| Class B | -0.45% | -0.17% | -0.21% | 105% | 100% |
| Class C | -0.27% | -0.26% | -0.23% | 106% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.40% | -0.23% | -0.28% | 105% | 100% |
| Class D | 0.01% | -0.14% | -0.05% | 107% | 105% |
| Class F | -0.94% | -0.60% | -0.53% | 103% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.64% | -0.79% | -0.36% | 104% | 100% |
| Class C | -0.33% | -0.08% | -0.35% | 106% | 101% |
| Class E | -0.16% | -0.03% | -0.27% | 104% | 99% |
| Overall | -0.42% | -0.36% | -0.33% | 105% | 100% |
| Class D | -0.04% | -0.07% | -0.40% | 107% | 105% |
| Class F | -2.51% | -1.98% | -1.89% | 104% | 101% |

Table 2.  Results of Test 2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.35% | -0.44% | -0.47% | 103% | 98% |
| Class A2 | -0.08% | -0.09% | -0.05% | 106% | 101% |
| Class B | -0.17% | -0.15% | -0.22% | 105% | 102% |
| Class C | 0.02% | -0.23% | -0.22% | 105% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -0.22% | -0.24% | 105% | 101% |
| Class D | 0.09% | -0.17% | -0.13% | 107% | 101% |
| Class F | -0.32% | -0.24% | -0.25% | 103% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.20% | -0.60% | -0.29% | 105% | 100% |
| Class C | 0.02% | -0.16% | -0.26% | 107% | 102% |
| Class E | 0.11% | -0.03% | -0.32% | 105% | 102% |
| Overall | -0.05% | -0.31% | -0.29% | 105% | 101% |
| Class D | 0.15% | -0.55% | -0.60% | 109% | 106% |
| Class F | -0.31% | 0.03% | 0.39% | 106% | 103% |

Table 3.  Results of Test 3

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.36% | -0.42% | -0.47% | 103% | 99% |
| Class A2 | -0.09% | -0.04% | 0.01% | 106% | 100% |
| Class B | -0.18% | -0.12% | -0.17% | 105% | 101% |
| Class C | 0.02% | -0.16% | -0.01% | 106% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -0.17% | -0.15% | 105% | 101% |
| Class D | 0.14% | -0.03% | 0.08% | 108% | 102% |
| Class F | -0.41% | -0.29% | -0.28% | 104% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.25% | -0.58% | -0.34% | 105% | 99% |
| Class C | 0.01% | 0.00% | -0.29% | 107% | 102% |
| Class E | -0.02% | -0.24% | -0.73% | 105% | 101% |
| Overall | -0.11% | -0.30% | -0.42% | 106% | 101% |
| Class D | 0.22% | -0.37% | -0.15% | 107% | 103% |
| Class F | -0.99% | -0.37% | -0.52% | 105% | 102% |

# Conclusions

This contribution proposes methods to simplify LIC. The encoding time for LIC is effectively reduced. The latency issue in hardware decoders can be effectively mitigated.

# References

- C.-W. Hsu, M. Winken, “Description of Core Experiment 10 (CE10): Combined and multi-hypothesis prediction,” Document of Joint Video Experts Team, JVET-L1030.

- VTM3.0: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- H. Liu, Y. Chen, J. Chen, L. Zhang, M. Karczewicz, “Local illumination compensation,” ITU-T SG16/Q6 Document, VCEG-AZ06.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).