| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0183-v1 |
| --- | --- |

| Title: | CE10-related: Simplification of MPM generation for CIIP |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {man-shu.chiang, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the most probable mode (MPM) generation for combined inter intra prediction (CIIP) in two aspects. Aspect 1 is that when generating the MPM list for CIIP, the default mode of the referenced-neighbouring coding unit (CU) is aligned with that for non-CIIP intra mode, i.e., changed from DC to planar. Aspect 2 is to further simplify the intra mode signalling for CIIP based on Aspect 1. The number of MPMs for CIIP is reduced from three to one. It is reported that, compared to VTM3.0, the proposed method results in -0.01% and 0.03% luma BD-rates for RA and LB, respectively.

# Introduction

CIIP [1], adopted into VTM3.0, is to improve the coding efficiency of a merge-coded CU by combining the existing merge prediction with the intra prediction, generated as that for non-CIIP intra mode. In a merge CU, one flag (mh_intra_flag in JVET-L1001-v7.docx; suggested to renamed as ciip_flag) is signalled for merge mode to select an intra mode from an intra candidate list when the flag is true. For luma, the intra candidate list is derived from four intra prediction modes including DC, planar, horizontal, and vertical modes, and the size of the intra candidate list can be three or four depending on the block shape. When the CU width is larger than the double of CU height, horizontal mode is excluded; when the CU height is larger than the double of CU width, vertical mode is excluded. One intra prediction mode selected by the intra mode index and one merge indexed prediction selected by the merge index are combined using weighted averaging. For chroma, direct mode (DM) is always applied without extra signalling. Moreover, the intra prediction mode used for CIIP can be saved for being referenced by the following neighbouring CUs. The signalling design of the intra mode for CIIP follows the intra mode signalling for non-CIIP intra mode in VTM2.0, where three MPMs, including LEFT and ABOVE are used. As the intra prediction mode of LEFT or ABOVE is not available, DC mode is used as the default mode.

However, at the last meeting, a method of six MPMs [2] is also adopted into VTM3.0 [3], and the signalling design of intra mode signalling for non-CIIP intra mode is changed accordingly. In VTM3.0, when generating the MPM list, if the intra prediction mode of LEFT or ABOVE is not available, planar mode is used as the default mode. The behavior of intra mode signalling for CIIP is suggested to be aligned with that for non-CIIP intra mode on top of VTM3.0.

# Proposed method

This proposal includes two aspects. Aspect 1 is to align CIIP with non-CIIP intra mode when generating the MPM list. If the intra prediction mode of a referenced-neighbouring CU is not available, planar mode is used as the default mode. Aspect 2 is modified based on Aspect 1 and is to further simplify intra mode signalling for CIIP according to the following observation. When signalling the intra prediction mode for CIIP, one flag is required to indicate whether the intra prediction mode is one of the MPMs. If the flag is true, one additional index is needed to select the intra prediction mode from the MPM list; otherwise, the intra prediction mode can be inferred to be the remaining one which is not in the MPM list. So, for a CIIP-coded CU, the syntax for the MPM is not signalled efficiently and requires more bins than non-MPM. In the proposed Aspect 2, the number of MPMs is reduced to 1, where only the intra prediction mode from LEFT is the MPM, and non-MPMs are signalled with truncated unary in this order {planar, DC, HOR, VER}. Note that, the intra prediction mode existing in the MPM list is removed from {planar, DC, HOR, VER}. Take an 8x8 CIIP- coded CU as an example. If the MPM is planar mode, the non-MPMs are DC, HOR, and VER. The corresponding code word is shown in Table 1.

Table 1.  The code word of signalling the intra prediction mode for an 8x8 CIIP-coded CU (assuming that the MPM is planar mode)

| Intra prediction mode | Code word |
| --- | --- |
| planar | 0 |
| DC | 10 |
| HOR | 110 |
| VER | 111 |

# Experimental results

The proposed method has been implemented on VTM3.0. Table 2 and Table 3 show the results for Aspect 1 and Aspect 2, respectively. The simulations were performed following the JVET common test conditions (CTC) [4].

For investigating the intra mode selection of CIIP, the statistic information of each intra mode for CIIP on VTM3.0 is shown in Table 4. The number means the per-class percentage of each intra mode for CIIP.

Table 2.  Results of Aspect 1: CIIP alignment with non-CIIP intra mode when generating the MPM list

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | -0.05% | 101% | 100% |
| Class A2 | -0.01% | -0.07% | 0.06% | 100% | 101% |
| Class B | -0.01% | 0.06% | 0.05% | 100% | 101% |
| Class C | -0.02% | -0.01% | 0.02% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.01% | 0.02% | 100% | 101% |
| Class D | 0.00% | -0.05% | -0.03% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.18% | 0.28% | 100% | 99% |
| Class C | 0.00% | 0.14% | 0.27% | 101% | 102% |
| Class E | -0.03% | 0.26% | -0.03% | 100% | 102% |
| Overall | 0.01% | 0.04% | 0.20% | 101% | 101% |
| Class D | -0.02% | 0.04% | -0.06% | 101% | 101% |
| Class F | 0.13% | 0.13% | 0.24% | 100% | 100% |

Table 3.  Results of Aspect 2: CIIP simplification of intra mode signalling based on Aspect 1

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | -0.05% | 96% | 102% |
| Class A2 | -0.01% | 0.03% | 0.07% | 96% | 99% |
| Class B | -0.01% | 0.10% | -0.03% | 98% | 100% |
| Class C | -0.02% | 0.01% | -0.01% | 98% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.04% | -0.01% | 97% | 99% |
| Class D | 0.01% | -0.19% | -0.01% | 98% | 98% |
| Class F | 0.00% | -0.02% | -0.02% | 101% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.25% | 0.11% | 99% | 97% |
| Class C | 0.04% | 0.25% | 0.21% | 97% | 97% |
| Class E | 0.03% | -0.09% | -0.39% | 103% | 101% |
| Overall | 0.03% | -0.04% | 0.02% | 99% | 98% |
| Class D | 0.00% | 0.14% | 0.29% | 100% | 99% |
| Class F | 0.02% | 0.03% | 0.18% | 98% | 96% |

Table 4.  Statistic information of each intra mode for CIIP on VTM3.0

|  | Random Access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VTM-3.0 (%) |  |  |  |  |  |  |  |
|  | Planar |  | DC |  | Hor |  | Ver |  |
| Class A1 | 43 |  | 26 |  | 15 |  | 15 |  |
| Class A2 | 40 |  | 25 |  | 16 |  | 19 |  |
| Class B | 39 |  | 24 |  | 17 |  | 20 |  |
| Class C | 39 |  | 23 |  | 21 |  | 18 |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 40 |  | 25 |  | 17 |  | 18 |  |
| Class D | 37 |  | 22 |  | 22 |  | 19 |  |
| Class F (optional) | 36 |  | 19 |  | 24 |  | 20 |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | VTM-3.0 (%) |  |  |  |  |  |  |  |
|  | Planar |  | DC |  | Hor |  | Ver |  |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 37 |  | 21 |  | 18 |  | 24 |  |
| Class C | 35 |  | 21 |  | 22 |  | 21 |  |
| Class E | 33 |  | 20 |  | 23 |  | 23 |  |
| Overall | 35 |  | 21 |  | 21 |  | 23 |  |
| Class D | 37 |  | 16 |  | 28 |  | 20 |  |
| Class F (optional) | 34 |  | 20 |  | 25 |  | 22 |  |

# Conclusions

This contribution proposes to simplify the most probable mode (MPM) generation for CIIP in two aspects. Compared against VTM3.0, the proposed method results in -0.01% and 0.03% luma BD-rates for RA and LB, respectively.

# References

- M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “CE10.1.1: Multi-hypothesis prediction for improving AMVP mode, skip or merge mode, and intra mode,” Document of Joint Video Experts Team, JVET-L0100.

- L. Li, J. Heo, J. Choi, J. Choi, S. Yoo, J. Lim, “CE3-6.2.1: Extended MPM list,” Document of Joint Video Experts Team, JVET-L0165.

- VTM3.0: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).