- 8.3.2.3 Derivation process for spatial merging candidates

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are as follows, with X being 0 or 1:

- the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2 of the neighbouring coding units,

- the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1 and refIdxLXB2 of the neighbouring coding units,

- the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and predFlagLXB2 of the neighbouring coding units,

- the motion vectors in 1/16 fractional-sample accuracy mvLXA0, mvLXA1, mvLXB0, mvLXB1 and mvLXB2 of the neighbouring coding units,

- the bi-prediction weight indices gbiIdxA0, gbiIdxA1, gbiIdxB0, gbiIdxB1, and gbiIdxB2.

For the derivation of availableFlagA1, refIdxLXA1, predFlagLXA1 and mvLXA1 the following applies:

- The luma location ( xNbA1, yNbA1 ) inside the neighbouring luma coding block is set equal to ( xCb − 1,  yCb + cbHeight − 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA1, yNbA1 ) as inputs, and the output is assigned to the block availability flag availableA1.

- The variables availableFlagA1, refIdxLXA1, predFlagLXA1 and mvLXA1 are derived as follows:

- If availableA1 is equal to FALSE, availableFlagA1 is set equal to 0, both components of mvLXA1 are set equal to 0, refIdxLXA1 is set equal to −1 and predFlagLXA1 is set equal to 0, with X being 0 or 1, and gbiIdxA1 is set equal to 0.

- Otherwise, availableFlagA1 is set equal to 1 and the following assignments are made:

mvLXA1 = MvLX[ xNbA1 ][ yNbA1 ]		(8-228)

refIdxLXA1 = RefIdxLX[ xNbA1 ][ yNbA1 ]		(8-229)

predFlagLXA1 = PredFlagLX[ xNbA1 ][ yNbA1 ]	(8-230)

gbiIdxA1 = GbiIdx[ xNbA1 ][ yNbA1 ]			(8-231)

For the derivation of availableFlagB1, refIdxLXB1, predFlagLXB1 and mvLXB1 the following applies:

- The luma location ( xNbB1, yNbB1 ) inside the neighbouring luma coding block is set equal to ( xCb + cbWidth − 1, yCb − 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB1, yNbB1 ) as inputs, and the output is assigned to the block availability flag availableB1.

- The variables availableFlagB1, refIdxLXB1, predFlagLXB1 and mvLXB1 are derived as follows:

- If one or more of the following conditions are true, availableFlagB1 is set equal to 0, both components of mvLXB1 are set equal to 0, refIdxLXB1 is set equal to −1 and predFlagLXB1 is set equal to 0, with X being 0 or 1, and gbiIdxB1 is set equal to 0:

  - availableB1 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB1, yNbB1 ) have the same motion vectors and the same reference indices.

- Otherwise, availableFlagB1 is set equal to 1 and the following assignments are made:

mvLXB1 = MvLX[ xNbB1 ][ yNbB1 ]		(8-232)

refIdxLXB1 = RefIdxLX[ xNbB1 ][ yNbB1 ]		(8-233)

predFlagLXB1 = PredFlagLX[ xNbB1 ][ yNbB1 ]		(8-234)

gbiIdxB1 = GbiIdx[ xNbB1 ][ yNbB1 ]			(8-235)

For the derivation of availableFlagB0, refIdxLXB0, predFlagLXB0 and mvLXB0 the following applies:

- The luma location ( xNbB0, yNbB0 ) inside the neighbouring luma coding block is set equal to ( xCb + cbWidth, yCb − 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB0, yNbB0 ) as inputs, and the output is assigned to the block availability flag availableB0.

- The variables availableFlagB0, refIdxLXB0, predFlagLXB0 and mvLXB0 are derived as follows:

- If one or more of the following conditions are true, availableFlagB0 is set equal to 0, both components of mvLXB0 are set equal to 0, refIdxLXB0 is set equal to −1 and predFlagLXB0 is set equal to 0, with X being 0 or 1, and gbiIdxB0 is set equal to 0:

  - availableB0 is equal to FALSE.

  - availableB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB0, yNbB0 ) have the same motion vectors and the same reference indices.

- Otherwise, availableFlagB0 is set equal to 1 and the following assignments are made:

mvLXB0 = MvLX[ xNbB0 ][ yNbB0 ]		(8-236)

refIdxLXB0 = RefIdxLX[ xNbB0 ][ yNbB0 ]		(8-237)

predFlagLXB0 = PredFlagLX[ xNbB0 ][ yNbB0 ]		(8-238)

gbiIdxB0 = GbiIdx[ xNbB0 ][ yNbB0 ]		(8-239)

For the derivation of availableFlagA0, refIdxLXA0, predFlagLXA0 and mvLXA0 the following applies:

- The luma location ( xNbA0, yNbA0 ) inside the neighbouring luma coding block is set equal to ( xCb − 1,  yCb + cbWidth ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA0, yNbA0 ) as inputs, and the output is assigned to the block availability flag availableA0.

- The variables availableFlagA0, refIdxLXA0, predFlagLXA0 and mvLXA0 are derived as follows:

- If one or more of the following conditions are true, availableFlagA0 is set equal to 0, both components of mvLXA0 are set equal to 0, refIdxLXA0 is set equal to −1 and predFlagLXA0 is set equal to 0, with X being 0 or 1, and gbiIdxA0 is set equal to 0:

  - availableA0 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbA0, yNbA0 ) have the same motion vectors and the same reference indices.

- Otherwise, availableFlagA0 is set equal to 1 and the following assignments are made:

mvLXA0 = MvLX[ xNbA0 ][ yNbA0 ]		(8-240)

refIdxLXA0 = RefIdxLX[ xNbA0 ][ yNbA0 ]		(8-241)

predFlagLXA0 = PredFlagLX[ xNbA0 ][ yNbA0 ]	(8-242)

gbiIdxA0 = GbiIdx[ xNbA0 ][ yNbA0 ]		(8-243)

For the derivation of availableFlagB2, refIdxLXB2, predFlagLXB2 and mvLXB2 the following applies:

- The luma location ( xNbB2, yNbB2 ) inside the neighbouring luma coding block is set equal to ( xCb − 1, yCb − 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB2, yNbB2 ) as inputs, and the output is assigned to the block availability flag availableB2.

- The variables availableFlagB2, refIdxLXB2, predFlagLXB2 and mvLXB2 are derived as follows:

- If one or more of the following conditions are true, availableFlagB2 is set equal to 0, both components of mvLXB2 are set equal to 0, refIdxLXB2 is set equal to −1 and predFlagLXB2 is set equal to 0, with X being 0 or 1, and gbiIdxB2 is set equal to 0:

  - availableB2 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB2, yNbB2 ) have the same motion vectors and the same reference indices.

  - availableB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB2, yNbB2 ) have the same motion vectors and the same reference indices.

  - availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4.

- Otherwise, availableFlagB2 is set equal to 1 and the following assignments are made:

- mvLXB2 = MvLX[ xNbB2 ][ yNbB2 ]		(8-244)

- refIdxLXB2 = RefIdxLX[ xNbB2 ][ yNbB2 ]		(8-245)

- predFlagLXB2 = PredFlagLX[ xNbB2 ][ yNbB2 ]		(8-246)

- gbiIdxB2 = GbiIdx[ xNbB2 ][ yNbB2 ]		(8-247)

Derivation process for triangle motion vector components and reference indices

General

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vectors in 1/16 fractional-sample accuracy mvLA and mvLB,

- the chroma motion vectors in 1/32 fractional-sample accuracy mvCLA and mvCLB,

- the reference indices refIdxLA and refIdxLB,

- the prediction list flags predFlagLA and predFlagLB.

The derivation process for luma motion vectors for triangle merge mode as specified in clause 8.3.3.2 is invoked with the luma location ( xCb, yCb ), the variables cbWidth and cbHeight as inputs, and the output being the luma motion vectors mvLA, mvLB, the reference indices refIdxLA, refIdxLB and the prediction list flags predFlagLA and predFlagLB.

The derivation process for chroma motion vectors in clause 8.3.2.13 is invoked with mvLA and refIdxLA as input, and the output being mvCLA.

The derivation process for chroma motion vectors in clause 8.3.2.13 is invoked with mvLB and refIdxLB as input, and the output being mvCLB.

Derivation process for luma motion vectors for merge triangle mode

This process is only invoked when merge_triangle_flag[ xCb ][ yCb ] is equal to 1, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vectors in 1/16 fractional-sample accuracy mvLA and mvLB,

- the reference indices refIdxLA and refIdxLB,

- the prediction list flags predFlagLA and predFlagLB.

The motion vectors mvLA and mvLB, the reference indices refIdxLA and refIdxLB and the prediction flags predFlagLA and predFlagLB are derived by the following ordered steps:

- The derivation process for merging candidates as specified in clause 8.3.2.2 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, and the luma coding block height cbHeight as inputs, and the output being the refIdxLXN, predFlagLXN, mvLXN[ 0 ], mvLXN[ 1 ], and gbiIdxN from the mergeCandList with X being 0 or 1 and N being 0 to 5.

- The variables m and n, being the candidates in the merging candidate list mergeCandList, are derived using merge_triangle_idx[ xCb ][ yCb ] as specified in Table 8-8.

- The following assignments are made with M and N being the candidates at position m and n in the merging candidate list mergeCandList ( M = mergeCandList[ m ] and N = mergeCandList[ n ] ):

predFlagLA = ( predFlagL0M = = 1 ) ? 0 : 1	(8-380)

predFlagLB = ( predFlagL1N = = 1 ) ? 1 : 0	(8-381)

refIdxLA = ( predFlagL0M = = 1 ) ? refIdxL0M : refIdxL1M	(8-382)

refIdxLB = ( predFlagL1N = = 1 ) ? refIdxL1N : refIdxL0N	(8-383)

mvLA[ 0 ] = ( predFlagL0M = = 1 ) ? mvL0M[ 0 ] : mvL1M[ 0 ]	(8-384)

mvLA[ 1 ] = ( predFlagL0M = = 1 ) ? mvL0M[ 1 ] : mvL1M[ 1 ]	(8-385)

mvLB[ 0 ] = ( predFlagL1N = = 1 ) ? mvL1N[ 0 ] : mvL0N[ 0 ]	(8-386)

mvLB[ 1 ] = ( predFlagL1N = = 1 ) ? mvL1N[ 1 ] : mvL0N[ 1 ]	(8-387)

gbiIdxM = 2	(8-xxx)

gbiIdxN = 2	(8-xxx)

- The variables refPicPocA and refPicPocB are derived as follows:

refPicPocA = (predFlagLA = = 0 )  ?  RefPicList0[ refIdxLA ]  :  RefPicList1[ refIdxLA ]	(8-xxx)

refPicPocB = (predFlagLB = = 0 )  ?  RefPicList0[ refIdxLB ]  :  RefPicList1[ refIdxLB ]	(8-xxx)

- If the refPicPocA is equal to refPicPocB, the mvLA[ 0 ] is equal to mvLB[ 0 ], and mvLA[ 1 ] is equal to mvLB[ 1 ], the following applies:

mvLB[ 0 ] += 4

-

-

-

-

-

-

-

-

-

-

  - -

-

-

-

-

-

-

-

-

Table 8-8 – Specification of m and n using merge_triangle_idx[ xCb ][ yCb ]

| merge_triangle_idx[ xCb ][ yCb ] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | 1 | 0 | 0 | 0 | 2 | 0 | 0 | 1 | 3 | 4 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 2 |
| n | 0 | 1 | 2 | 1 | 0 | 3 | 4 | 0 | 0 | 0 | 2 | 2 | 2 | 4 | 3 | 3 | 4 | 4 | 3 | 1 |
| merge_triangle_idx[ xCb ][ yCb ] | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| m | 2 | 2 | 4 | 3 | 3 | 3 | 4 | 3 | 2 | 4 | 4 | 2 | 4 | 3 | 4 | 3 | 2 | 2 | 4 | 3 |
| n | 0 | 1 | 3 | 0 | 2 | 4 | 0 | 1 | 3 | 1 | 1 | 3 | 2 | 2 | 3 | 1 | 4 | 4 | 2 | 4 |

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

