| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0190 |
| --- | --- |

| Title: | CE10-related: Redundant syntax reduction for triangle prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjo Ahn Donggyu Sim | Tel: Email: | +82-2-941-6470 yjahn@digitalinsights.co.kr dgsim@digitalinsights.co.kr |
| Source: | Digital Insights Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to reduce redundant syntax of triangle prediction when the MMVD or multi hypothesis prediction is enabled. It has been observed that the flag for triangle prediction is still signaled when the MMVD or multi hypothesis prediction is enabled. In this contribution, the triangle prediction flag is skipped when the MMVD or multi hypothesis prediction is enabled.

# Introduction

In the last meeting, three inter prediction tools were adopted such as merge with MVD (MMVD), multi hypothesis prediction, and triangle prediction [1]. However, redundant syntax and mismatch between draft and software have been observed. In this contribution, redundant syntax removal is proposed to solve the problems.

# Proposed method

To reduce redundant syntax, this contribution proposes a changing condition for a merge_triangle_flag parsing as Table 1. It means that merge_triangle_flag is skipped when the current coding unit is coded by multi hypothesis prediction.

Table 1. Proposed changes in Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mh_intra_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| mh_intra_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| mh_intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| mh_intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( sps_triangle_enabled_flag && slice_type = = B && cbWidth * cbHeight >= 64 && !mh_intra_flag[ x0 ][ y0 ] ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) |  |
| merge_triangle_idx[ x0 ][ y0 ] | ae(v) |
| else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |

In addition, the current parsing process of merge_triangle_flag has mismatch between the draft and the software when the current CU is coded by MMVD. This contribution fixes the mismatch between the draft text and the reference software.

# Performance evaluation

The proposed methods were evaluated under JVET common test conditions (CTC) [2]. The proposed methods were integrated on VTM-3.0. Table 2 shows BD-rate reduction of the proposed method. In this test, the proposed methods can achieve 0.07% and 0.19% BD-rate reduction in random access and low-delay B configuration, respectively.

Table 2. BD-rate reduction of the proposed method

|  | Over VTM-3.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.07% | -0.06% | -0.09% | 95% | 96% | -0.19% | 0.03% | -0.14% | 97% | 103% |
| Class E |  |  |  |  |  | -0.13% | 0.03% | -0.17% | 94% | 74% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.10% | -0.20% | -0.07% | 96% | 94% | -0.17% | 0.13% | 0.13% | 97% | 101% |

# References

- B. Bross, J. Chen, and S. Liu, “Versatile video coding (Draft 3),” JVET-L1001, 12th JVET meeting, Macau, CN, Oct. 2018.

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th JVET meeting, Macau, CN, Oct. 2018.

# Patent rights declaration(s)

Digital Insights Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).