| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0192-v1 |
| --- | --- |

| Title: | CE2-related: MV Derivation for Affine Chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anish Tamse Min Woo Park Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | tamse.anish@samsung.com m.w.park@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to simplify the MV derivation process for chroma sub-block of an affine CU. The size of chroma sub-block is 4x4 samples, which corresponds to 8x8 luma samples. To obtain the MV for the chroma sub-block, the MVs of each of the four 4x4 luma sub-blocks are averaged and rounded. Furthermore, the rounding process employed for computing the average of the four MVs is not symmetric for positive and negative numbers, because the current implementation uses right shifting by 2 after adding an offset of 2 irrespective of sign of the input. This contribution proposes several options simplify the MV derivation process and address the asymmetric rounding issue: 1) Use only the top-left luma sub-block’s MV as the representative MV 2) Use the average of the top-left and bottom-right luma sub-blocks’ MVs as the representative MV 3) Modify the current averaging process of four luma sub-block MVs to a symmetric one 4) Use the average of the top-left and bottom-right luma sub-blocks’ MVs as the representative MV while using symmetric rounding. All the proposed methods show marginal performance change.

# Introduction

In VVC [1], the sizes of both luma and chroma sub-blocks in an affine CU are 4x4 sample square. A 4x4 chroma sub-block corresponds to 8x8 luma block, which consists of four luma sub-blocks. The positions of luma and chroma pixel for an 8x8 luma block is as shown below.

The luma sub-blocks for an affine block and the corresponding MVs are shown in the figure below.

To compute the MV for the chroma sub-block, the MVs of the four luma sub-blocks are averaged as follows.

Mv curMv = (MV1 + MV2 + MV3 + MV4 + Mv(2, 2));

curMv.set(curMv.getHor() >> 2, curMv.getVer() >> 2);

The averaging process accesses four MVs. Also, the rounding process used is not symmetric. That is, if the MV obtained after adding the four MVs is (6,-6) it is rounded as

((6+2)>>2, (-6+2)>>2)

Which equals (2,-1).

To simplify the averaging process and address the asymmetric rounding issue, this contribution proposes several options.

# Proposed Methods

Each 4x4 chroma sub-block corresponds to four luma sub-blocks, top-left, top-right, bottom-left and bottom right. Proposed options basically use a subset of these MVs for the chroma MV computation process.

The asymmetric rounding issue is addressed at many other locations in VVC by computing the absolute value first, performing the shifting operation and then assigning the correct sign back. This is performed while rounding the MVP to the correct AMVR precision for example. The symmetric rounding method is performed as shown.

curMv.setHor(curMv.getHor() >= 0 ? ((curMv.getHor() + rndOffset) >> 2) : -((-curMv.getHor() + 2) >> 2));

curMv.setVer(curMv.getVer() >= 0 ? ((curMv.getVer() + rndOffset) >> rndShift) : -((-curMv.getVer() + rndOffset) >> rndShift));

Proposed options to simplify the MV computation process for chroma sub-blocks of affine CU and address the asymmetric rounding are as follows:

- Method 1) Use only top-left luma sub-block’s MV

- Method 2) Use the average of top-left and right-bottom luma sub-block’s MVs

- Method 3) Make the rounding symmetric

- Method 4) Use the average of top-left and right-bottom luma sub-block’s MVs with symmetric rounding.

# Experimental Results

The simulation was carried out under the JVET common test condition [2]. The software was compiled with GCC version 7.3 and the AVX2 SIMD instructions. Table 1 shows summary of all results.

Table 1. Summary of all results

|  | Test | BD-Rate (Y) | BD-Rate (U) | BD-Rate (V) | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| RA | Method 1 | 0.03% | 0.25% | 0.18% | 100% | 99% |
|  | Method 2 | -0.01% | -0.02% | 0.01% | 101% | 103% |
|  | Method 3 | 0.00% | 0.07% | 0.01% | 97% | 96% |
|  | Method 4 | 0.00% | 0.06% | 0.01% | 98% | 99% |
| LDB | Method 1 | 0.05% | 0.01% | 0.14% | 100% | 101% |
|  | Method 2 | 0.02% | 0.01% | 0.26% | 101% | 104% |
|  | Method 3 | 0.03% | 0.06% | 0.01% | 98% | 95% |
|  | Method 4 | 0.03% | 0.01% | 0.12% | 100% | 99% |

Table 2. Results for Method 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | 0.08% | 100% | 99% |
| Class A2 | 0.09% | 0.76% | 0.60% | 99% | 100% |
| Class B | 0.01% | 0.27% | 0.09% | 99% | 99% |
| Class C | 0.03% | 0.05% | 0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.25% | 0.18% | 100% | 99% |
| Class D | 0.00% | 0.04% | 0.04% | 100% | 99% |
| Class F (optional) | 0.02% | 0.31% | 0.33% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.14% | 0.06% | 100% | 102% |
| Class C | 0.04% | 0.10% | 0.05% | 99% | 98% |
| Class E | 0.09% | 0.13% | 0.38% | 100% | 102% |
| Overall | 0.05% | 0.01% | 0.14% | 100% | 101% |
| Class D | 0.03% | -0.29% | 0.19% | 100% | 99% |
| Class F (optional) | 0.02% | 0.46% | 0.62% | 98% | 96% |

Table 3. Results for Method 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | 0.01% | 101% | 101% |
| Class A2 | -0.01% | -0.03% | 0.07% | 101% | 101% |
| Class B | 0.00% | 0.04% | 0.07% | 101% | 102% |
| Class C | -0.01% | -0.08% | -0.09% | 101% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.02% | 0.01% | 101% | 103% |
| Class D | 0.01% | -0.06% | -0.07% | 101% | 103% |
| Class F (optional) | 0.01% | -0.01% | 0.00% | 101% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.18% | 0.47% | 101% | 101% |
| Class C | 0.02% | 0.29% | 0.20% | 101% | 105% |
| Class E | 0.01% | -0.03% | -0.01% | 102% | 108% |
| Overall | 0.02% | 0.01% | 0.26% | 101% | 104% |
| Class D | 0.03% | 0.42% | 0.19% | 101% | 107% |
| Class F (optional) | 0.13% | -0.28% | 0.11% | 100% | 102% |

Table 4. Results for Method 3

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.04% | 95% | 99% |
| Class A2 | -0.02% | 0.07% | 0.10% | 98% | 98% |
| Class B | -0.02% | 0.13% | -0.02% | 97% | 96% |
| Class C | 0.02% | 0.04% | -0.06% | 97% | 93% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.07% | 0.01% | 97% | 96% |
| Class D | 0.01% | -0.07% | -0.05% | 98% | 93% |
| Class F (optional) | 0.02% | 0.01% | 0.05% | 99% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.16% | -0.13% | 98% | 97% |
| Class C | 0.02% | 0.23% | 0.13% | 100% | 96% |
| Class E | 0.03% | 0.19% | 0.07% | 97% | 90% |
| Overall | 0.03% | 0.06% | 0.01% | 98% | 95% |
| Class D | -0.01% | 0.22% | 0.20% | 100% | 96% |
| Class F (optional) | -0.09% | 0.20% | 0.01% | 99% | 92% |

Table 5. Results for Method 4

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.00% | 0.00% | 96% | 101% |
| Class A2 | 0.00% | 0.10% | 0.10% | 99% | 100% |
| Class B | -0.01% | 0.07% | -0.01% | 98% | 99% |
| Class C | 0.01% | 0.07% | -0.03% | 98% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.06% | 0.01% | 98% | 99% |
| Class D | 0.00% | -0.09% | -0.01% | 98% | 96% |
| Class F (optional) | 0.00% | -0.01% | 0.01% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.22% | -0.04% | 100% | 100% |
| Class C | 0.06% | 0.24% | 0.36% | 100% | 100% |
| Class E | 0.00% | 0.10% | 0.08% | 98% | 97% |
| Overall | 0.03% | 0.01% | 0.12% | 100% | 99% |
| Class D | -0.01% | -0.18% | 0.78% | 99% | 99% |
| Class F (optional) | -0.04% | -0.11% | -0.43% | 100% | 101% |

# Conclusion

In this contribution, it is proposed to simplify the MV derivation for chroma sub-blocks of affine CU by using fewer luma sub-block MVs. It is also proposed to make the rounding process employed during averaging symmetric. The proposed methods show marginal coding performance change and it is suggested to adopt the proposed method into the next VVC working draft text.

# References

[1] “Versatile Video Coding (Draft 3),” JVET-L1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, October 2018.

[2] “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao,  October 2018.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).