| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-Mxxxx_WD |
| --- | --- |

| Title: | Proposed WD for CE4-related: xxx |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anish Tamse Min Woo Park | Email: | tamse.anish@samsung.com m.w.park@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Proposed Working Draft Changes

This document provides the proposed text of VVC Working Draft. The changes are highlighted in green and strikethrough.

#### Derivation process for pairwise average merging candidates

Inputs to this process are:

- a merging candidate list mergeCandList,

- the reference indices refIdxL0N and refIdxL1N of every candidate N in mergeCandList,

- the prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N in mergeCandList,

- the motion vectors in 1/16 fractional-sample accuracy mvL0N and mvL1N of every candidate N in mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the number of elements numOrigMergeCand within the mergeCandList before pairwise average merging candidate derivation process.

Outputs of this process are:

- the merging candidate list mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the reference indices refIdxL0avgCandk and refIdxL1avgCandk of every new candidate avgCandk added into mergeCandList during the invocation of this process,

- the prediction list utilization flags predFlagL0avgCandk and predFlagL1avgCandk of every new candidate avgCandk added into mergeCandList during the invocation of this process,

- the motion vectors in 1/16 fractional-sample accuracy mvL0avgCandk and mvL1avgCandk of every new candidate avgCandk added into mergeCandList during the invocation of this process.

The variables numCutoff, avgIdx, avgStop, numRefLists are derived as follows:

numCutoff = Min( numCurrMergeCand, 4 2 ) 	(8-229)

avgIdx = 0	(8-230)

avgStop = (avgIdx >= ( numCutoff * ( numCutoff − 1 ) / 2 )  ||  	(8-231)
	numCurrMergeCand  = =  MaxNumMergeCand )

numRefLists  =  ( slice_type = = B )  ?  2 : 1	(8-232)

The following ordered steps are repeated until avgStop is equal to TRUE:

- The variables p0CandIdx and p1CandIdx are derived using avgIdx as specified in Table 8-7.

- The following assignments are made, with p0Cand being the candidate at position p0CandIdx and p1Cand being the candidate at position p1CandIdx in the merging candidate list mergeCandList:

p0Cand = mergeCandList[ p0CandIdx ]	(8-233)

p1Cand = mergeCandList[ p1CandIdx ]	(8-234)

- The candidate avgCandk with k equal to ( numCurrMergeCand − numOrigMergeCand ) is added at the end of mergeCandList, i.e., mergeCandList[ numCurrMergeCand ] is set equal to avgCandk, and the reference indices, the prediction list utilization flags and the motion vectors of avgCandk are derived as follows and numCurrMergeCand is incremented by 1:

- For each reference picture list LX with X ranging from 0 to ( numRefLists − 1 ), the following applies:

  - If predFlagLXp0Cand is equal to 1 and predFlagLXp1Cand is equal to 1, the variables refIdxLXavgCandk, predFlagLXavgCandk, mvLXavgCandk[ 0 ], and mvLXavgCandk[ 1 ] are derived as follows:

refIdxLXavgCandk = refIdxLXp0Cand	(8-235)

predFlagLXavgCandk = 1	(8-236)

mvLXavgCandk[ 0 ] = ( mvLXp0Cand[ 0 ] + mvLXp1Cand[ 0 ] ) / 2	(8-237)

mvLXavgCandk[ 1 ] = ( mvLXp0Cand[ 1 ] + mvLXp1Cand[ 1 ] ) / 2	(8-238)

  - Otherwise, if predFlagLXp0Cand is equal to 1 and predFlagLXp1Cand is equal to 0, the variables refIdxLXavgCandk, predFlagLXavgCandk, mvLXavgCandk[ 0 ], mvLXavgCandk[ 1 ] are derived as follows:

refIdxLXavgCandk = refIdxLXp0Cand	(8-239)

predFlagLXavgCandk = 1	(8-240)

mvLXavgCandk[ 0 ] = mvLXp0Cand[ 0 ] 	(8-241)

mvLXavgCandk[ 1 ] = mvLXp0Cand[ 1 ] 	(8-242)

  - Otherwise, if predFlagLXp0Cand is equal to 0 and predFlagLXp1Cand is equal to 1, the variables refIdxLXavgCandk, predFlagLXavgCandk, mvLXavgCandk[ 0 ], mvLXavgCandk[ 1 ] are derived as follows:

refIdxLXavgCandk = refIdxLXp1Cand	(8-243)

predFlagLXavgCandk = 1	(8-244)

mvLXavgCandk[ 0 ] = mvLXp1Cand[ 0 ] 	(8-245)

mvLXavgCandk[ 1 ] = mvLXp1Cand[ 1 ] 	(8-246)

  - Otherwise, if predFlagLXp0Cand is equal to 0 and predFlagLXp1Cand is equal to 0, the variables refIdxLXavgCandk, predFlagLXavgCandk, mvLXavgCandk[ 0 ], mvLXavgCandk[ 1 ] are derived as follows:

refIdxLXavgCandk = −1	(8-247)

predFlagLXavgCandk = 0	(8-248)

mvLXavgCandk[ 0 ] = 0	(8-249)

mvLXavgCandk[ 1 ] = 0	(8-250)

- When numRefLists is equal to 1, the following applies:

refIdxL1avgCandk = −1	(8-251)

predFlagL1avgCandk = 0	(8-252)

- The variable avgIdx is incremented by 1.

- When avgIdx is equal to ( numCutoff * ( numCutoff − 1 ) / 2 ) 1 or numCurrMergeCand is equal to MaxNumMergeCand, avgStop is set equal to TRUE.

Table 8-7 – Specification of p0CandIdx and p1CandIdx

| avgIdx | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| p0CandIdx | 0 | 0 | 1 | 0 | 1 | 2 |
| p1CandIdx | 1 | 2 | 2 | 3 | 3 | 3 |

| avgIdx | 0 |
| --- | --- |
| p0CandIdx | 0 |
| p1CandIdx | 1 |

