| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0193_WD_r1 |
| --- | --- |

| Title: | Proposed WD for CE4-related: CE4-related: Pairwise Average Candidate Reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anish Tamse Min Woo Park | Email: | tamse.anish@samsung.com m.w.park@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Proposed Working Draft Changes

This document provides the proposed text of VVC Working Draft. The changes are highlighted in green and strikethrough.

#### Derivation process for luma motion vectors for merge mode

This process is only invoked when merge_flag[ xCb ][ yPb ] is equal to 1, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ],

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ],

- the bi-prediction weight index gbiIdx.

The bi-prediction weight index gbiIdx is set equal to 0.

The motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], the reference indices refIdxL0 and refIdxL1 and the prediction utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are derived by the following ordered steps:

- The derivation process for merging candidates from neighbouring coding units as specified in clause 8.4.2.3 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, and the luma coding block height cbHeight as inputs, and the output being the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1 and refIdxLXB2, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and predFlagLXB2, and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1 and mvLXB2, with X being 0 or 1, and the bi-prediction weight indices gbiIdxA0, gbiIdxA1, gbiIdxB0, gbiIdxB1, gbiIdxB2.

- The reference indices, refIdxLXCol, with X being 0 or 1, and the bi-prediction weight index gbiIdxCol for the temporal merging candidate Col are set equal to 0.

- The derivation process for temporal luma motion vector prediction as specified in in clause 8.4.2.11 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the variable refIdxL0Col as inputs, and the output being the availability flag availableFlagL0Col and the temporal motion vector mvL0Col. The variables availableFlagCol, predFlagL0Col and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col		(8-219)

predFlagL0Col = availableFlagL0Col		(8-220)

predFlagL1Col = 0		(8-221)

- When tile_group_type is equal to B, the derivation process for temporal luma motion vector prediction as specified in clause 8.4.2.11 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the variable refIdxL1Col as inputs, and the output being the availability flag availableFlagL1Col and the temporal motion vector mvL1Col. The variables availableFlagCol and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col  | |  availableFlagL1Col	(8-222)

predFlagL1Col = availableFlagL1Col		(8-223)

- The merging candidate list, mergeCandList, is constructed as follows:

i = 0
if( availableFlagA1 )
	mergeCandList[ i++ ] = A1
if( availableFlagB1 )
	mergeCandList[ i++ ] = B1
if( availableFlagB0 )
	mergeCandList[ i++ ] = B0		(8-224)
if( availableFlagA0 )
	mergeCandList[ i++ ] = A0
if( availableFlagB2 )
	mergeCandList[ i++ ] = B2
if( availableFlagCol )
	mergeCandList[ i++ ] = Col

- The variable numCurrMergeCand and numOrigMergeCand are set equal to the number of merging candidates in the mergeCandList.

- When numCurrMergeCand is less than (MaxNumMergeCand − 1) and NumHmvpCand is greater than 0, the following applies:

- The derivation process of history-based merging candidates as specified in 8.4.2.6 is invoked with mergeCandList, and numCurrMergeCand as inputs, and modified mergeCandList and numCurrMergeCand as outputs.

- numOrigMergeCand is set equal to numCurrMergeCand.

- When numCurrMergeCand is less than MaxNumMergeCand and greater than 1 and neither or both of pNCand  (N is 0 or 1) use the current picture as the reference picture the following applies:

- The derivation process for pairwise average merging candidates specified in clause 8.4.2.4 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList, and numCurrMergeCand and numOrigMergeCand as inputs, and the output is assigned to mergeCandList, and numCurrMergeCand, the reference indices refIdxL0avgCandk and refIdxL1avgCandk, the prediction list utilization flags predFlagL0avgCandk and predFlagL1avgCandk and the motion vectors mvL0avgCandk and mvL1avgCandk of every new candidate avgCandk being added into mergeCandList. The bi-prediction weight index gbiIdx of every new candidate avgCandk being added into mergeCandList is set equal to 0. The number of candidates being added, numAvgMergeCand, is set equal to 1 ( numCurrMergeCand − numOrigMergeCand ). When numAvgMergeCand is greater than 0, k ranges from 0 to numAvgMergeCand − 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in clause 8.4.2.5 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList and numCurrMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm and the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added into mergeCandList. The bi-prediction weight index gbiIdx of every new candidate zeroCandm being added into mergeCandList is set equal to 0. The number of candidates being added, numZeroMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand − numAvgMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

- The variable mergeIdxOffset is set equal to 0.

- When mmvd_flag[ xCb ][ yCb ] is equal to 1, the variable mmvdCnt is set equal to 0 and tThe following applies until mmvdCnt is greater than ( merge_idx[ xCb ][ yCb ] + mergeIdxOffset ) or mmvdCnt is equal to MaxNumMergeCand:

- When candidate mergeCandList[ mmvdCnt ] uses the current decoded picture as its reference picture, mergeIdxOffset is incremented by 1.

- The variable mmvdCnt is incremented by 1.

- The following assignments are made with N being the candidate at position merge_idx[ xCb ][ yCb ] + mergeIdxOffset  in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xCb ][ yCb ] + mergeIdxOffset ] ) and X being replaced by 0 or 1:

refIdxLX = refIdxLXN		(8-225)

predFlagLX[ 0 ][ 0 ] = predFlagLXN		(8-226)

mvLX[ 0 ][ 0 ][ 0 ] = mvLXN[ 0 ]		(8-227)

mvLX[ 0 ][ 0 ][ 1 ] = mvLXN[ 1 ]		(8-228)

gbiIdx = gbiIdxN		(8-229)

- When mmvd_flag[ xCb ][ yCb ] is equal to 1, the following applies:

- The derivation process for merge motion vector difference as specified in 8.4.2.7 is invoked with the luma location ( xCb, yCb ), the luma motion vectors mvL0[ 0 ][ 0 ], mvL1[ 0 ][ 0 ], the reference indices refIdxL0, refIdxL1 and the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] as inputs, and the motion vector differences mMvdL0 and mMvdL1 as outputs.

- The motion vector difference mMvdLX is added to the merge motion vectors mvLX for X being 0 and 1 as follows:

mvLX[ 0 ][ 0 ][ 0 ] += mMvdLX[ 0 ]		(8-230)

mvLX[ 0 ][ 0 ][ 1 ] += mMvdLX[ 1 ]		(8-231)

#### Derivation process for pairwise average merging candidates

Inputs to this process are:

- a merging candidate list mergeCandList,

- the reference indices refIdxL0N and refIdxL1N of every candidate N in mergeCandList,

- the prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N in mergeCandList,

- the motion vectors in 1/16 fractional-sample accuracy mvL0N and mvL1N of every candidate N in mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the number of elements numOrigMergeCand within the mergeCandList before pairwise average merging candidate derivation process.

Outputs of this process are:

- the merging candidate list mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the reference indices refIdxL0avgCandk and refIdxL1avgCandk of every new candidate avgCandk added into mergeCandList during the invocation of this process,

- the prediction list utilization flags predFlagL0avgCandk and predFlagL1avgCandk of every new candidate avgCandk added into mergeCandList during the invocation of this process,

- the motion vectors in 1/16 fractional-sample accuracy mvL0avgCandk and mvL1avgCandk of every new candidate avgCandk added into mergeCandList during the invocation of this process.

The variable variables numCutoff, avgIdx, avgStop, numRefLists are is derived as follows:

numCutoff = Min( numCurrMergeCand, 4 ) 	(8-229)

avgIdx = 0	(8-230)

avgStop = (avgIdx >= ( numCutoff * ( numCutoff − 1 ) / 2 )  ||  	(8-231)
	numCurrMergeCand  = =  MaxNumMergeCand )

numRefLists  =  ( tile_group_type = = B )  ?  2 : 1	(8-232)

The following ordered steps are repeated until avgStop is equal to TRUE:

The following ordered steps are performed:

- The variables p0CandIdx and p1CandIdx derived using avgIdx as specified in Table 8-7.

- The following assignments are made, with p0Cand being the candidate at position 0 p0CandIdx and p1Cand being the candidate at position 1 p1CandIdx in the merging candidate list mergeCandList:

p0Cand = mergeCandList[ 0p0CandIdx ]	(8-233)

p1Cand = mergeCandList[ 1p1CandIdx ]	(8-234)

- If neither or both of pNCand  (N is 0 or 1) use the current picture as the reference picture, tThe candidate avgCandk with k equal to ( numCurrMergeCand − numOrigMergeCand ) is added at the end of mergeCandList, i.e., mergeCandList[ numCurrMergeCand ] is set equal to avgCandk, and the reference indices, the prediction list utilization flags and the motion vectors of avgCandk are derived as follows and numCurrMergeCand is incremented by 1:

- For each reference picture list LX with X ranging from 0 to ( numRefLists − 1 ), the following applies:

  - If predFlagLXp0Cand is equal to 1 and predFlagLXp1Cand is equal to 1, the variables refIdxLXavgCandk, predFlagLXavgCandk, mvLXavgCandk[ 0 ], and mvLXavgCandk[ 1 ] are derived as follows:

refIdxLXavgCandk = refIdxLXp0Cand	(8-235)

predFlagLXavgCandk = 1	(8-236)

mvLXavgCandk[ 0 ] = ( mvLXp0Cand[ 0 ] + mvLXp1Cand[ 0 ] ) / 2	(8-237)

mvLXavgCandk[ 1 ] = ( mvLXp0Cand[ 1 ] + mvLXp1Cand[ 1 ] ) / 2	(8-238)

  - Otherwise, if predFlagLXp0Cand is equal to 1 and predFlagLXp1Cand is equal to 0, the variables refIdxLXavgCandk, predFlagLXavgCandk, mvLXavgCandk[ 0 ], mvLXavgCandk[ 1 ] are derived as follows:

refIdxLXavgCandk = refIdxLXp0Cand	(8-239)

predFlagLXavgCandk = 1	(8-240)

mvLXavgCandk[ 0 ] = mvLXp0Cand[ 0 ] 	(8-241)

mvLXavgCandk[ 1 ] = mvLXp0Cand[ 1 ] 	(8-242)

  - Otherwise, if predFlagLXp0Cand is equal to 0 and predFlagLXp1Cand is equal to 1, the variables refIdxLXavgCandk, predFlagLXavgCandk, mvLXavgCandk[ 0 ], mvLXavgCandk[ 1 ] are derived as follows:

refIdxLXavgCandk = refIdxLXp1Cand	(8-243)

predFlagLXavgCandk = 1	(8-244)

mvLXavgCandk[ 0 ] = mvLXp1Cand[ 0 ] 	(8-245)

mvLXavgCandk[ 1 ] = mvLXp1Cand[ 1 ] 	(8-246)

  - Otherwise, if predFlagLXp0Cand is equal to 0 and predFlagLXp1Cand is equal to 0, the variables refIdxLXavgCandk, predFlagLXavgCandk, mvLXavgCandk[ 0 ], mvLXavgCandk[ 1 ] are derived as follows:

refIdxLXavgCandk = −1	(8-247)

predFlagLXavgCandk = 0	(8-248)

mvLXavgCandk[ 0 ] = 0	(8-249)

mvLXavgCandk[ 1 ] = 0	(8-250)

- When numRefLists is equal to 1, the following applies:

refIdxL1avgCandk = −1	(8-251)

predFlagL1avgCandk = 0	(8-252)

- gbiIdxavgCand is set to 0

- The variable avgIdx is incremented by 1.

- When avgIdx is equal to ( numCutoff * ( numCutoff − 1 ) / 2 ) 1 or numCurrMergeCand is equal to MaxNumMergeCand, avgStop is set equal to TRUE.

Table 8-7 – Specification of p0CandIdx and p1CandIdx

| avgIdx | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| p0CandIdx | 0 | 0 | 1 | 0 | 1 | 2 |
| p1CandIdx | 1 | 2 | 2 | 3 | 3 | 3 |

