| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET- M0195-v1 |
| --- | --- |

| Title: | CE1-related: Non-Residual Block on VPDU Boundary |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anish Tamse Min Woo Park Minsoo Park Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | tamse.anish@samsung.com m.w.park@samsung.com mss.park@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes a solution addressing the VPDU issue, which is, there cannot be any transform unit overlapping with the virtual 64x64 unit as it makes the hardware implementation difficult. VTM3.0 handles this issue by imposing partitioning restriction such that no CU overlaps with VPDU boundary. This contribution proposes to handle this issue by relaxing partitioning restriction introduced in VTM3.0 and instead only disallowing TUs for CUs which overlap with VPDU boundary. Two methods are proposed to this effect: 1) Allow only skip mode for CUs overlapping with VPDU boundary. 2) Allow all inter modes with cbf flag set to zero, i.e. no residuals are allowed. The performance improvement of method 1 is -0.11% in Y, -0.11% in U and -0.10% in V with 105% encoder runtime and that of method 2 is -0.22% in Y, -0.22% in U and -0.21% in V with 115% encoder runtime, when tested under RA conditions on VTM 3.0.

# Introduction

In VVC [1], partitioning restrictions are introduced which solves the 64x64 pipeline implementation issue, however it introduces a loss in performance. The loss is due to the reduced flexibility of the types of block partitions that are now possible.

The issue with 64x64 pipeline implementation is that transforms which spans multiple VPDUs cannot be processed easily. In the figure shown, the TUs shown in orange are not pipeline friendly. In this proposal instead of addressing this issue by completely disallowing CUs which overlap with VPDU boundary, or imposing additional split restrictions on the TUs that overlap with VPDU, it is proposed to disallow residuals for the CUs which overlap with the VPDU boundary.

The proposed method and its implementation is described in more detail in the next section.

# Proposed Methods

All the CUs which overlap with VPDU boundary except those mentioned below are disallowed from having residuals. The blocks which lie on VPDU boundary and can still have residuals should be one of the follows:

- CU size is 128x128 : In this case, the TUs are all aligned with 64x64 VPDUs

- CU size is 128x64 and the block touches the top or bottom edge of the CTU : In this case, the TUs are all aligned with 64x64 VPDUs

- CU size is 64x128 and the block touches the left or right edge of the CTU : In this case, the TUs are all aligned with 64x64 VPDUs

- CU belongs to a picture boundary CTU in which case VPDU restrictions are not required.

There are multiple ways to impose the condition of not having any residuals. One is to only allow skip prediction mode. Skip prediction mode doesn’t have any residual by definition. In this proposal, regular skip and MMVD skip is allowed. Another method is to allow all inter modes (intra modes generally have large residuals, so it is not preferable to have non-residual intra block) but disallow residuals by setting  cbf flag to be 0. This allows MVD to be coded in regular inter prediction mode for blocks overlapping with VPDU boundary.

Hence, the methods proposed in this contribution to address the 64x64 pipeline issue are as follows:

- Method 1) Allow only skip blocks at VPDU boundaries

- Method 2) Allow inter modes with residuals forced to 0 at VPDU boundaries

# Experimental Results

The simulation was carried out under the JVET common test condition [2]. The software was compiled with GCC version 7.3 and the AVX2 SIMD instructions. Table 1 shows summary of all results.

Table 1. Summary of all results

|  | Test | BD-Rate (Y) | BD-Rate (U) | BD-Rate (V) | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| RA | Method 1 | -0.11% | -0.11% | -0.10% | 105% | 100% |
|  | Method 2 | -0.22% | -0.22% | -0.21% | 115% | 100% |
| LDB | Method 1 | -0.03% | -0.05% | 0.06% | 100% | 100% |
|  | Method 2 | -0.18% | -0.21% | -0.16% | 109% | 100% |

Table 3. Results for Method 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.19% | -0.09% | 106% | 100% |
| Class A2 | -0.25% | -0.29% | -0.24% | 105% | 100% |
| Class B | -0.08% | -0.10% | -0.17% | 105% | 100% |
| Class C | -0.01% | 0.07% | 0.09% | 105% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.11% | -0.10% | 105% | 100% |
| Class D | -0.02% | -0.11% | -0.03% | 105% | 100% |
| Class F (optional) | -0.06% | -0.12% | -0.05% | 102% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.21% | 0.09% | 101% | 101% |
| Class C | 0.00% | 0.08% | 0.14% | 102% | 101% |
| Class E | -0.16% | 0.03% | -0.11% | 96% | 99% |
| Overall | -0.03% | -0.05% | 0.06% | 100% | 100% |
| Class D | 0.00% | 0.12% | 0.30% | 101% | 100% |
| Class F (optional) | -0.10% | -0.50% | -0.37% | 94% | 98% |

Table 2. Results for Method 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -0.28% | -0.26% | 122% | 100% |
| Class A2 | -0.47% | -0.38% | -0.38% | 117% | 100% |
| Class B | -0.19% | -0.20% | -0.30% | 115% | 100% |
| Class C | -0.04% | -0.06% | 0.07% | 109% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -0.22% | -0.21% | 115% | 100% |
| Class D | -0.05% | -0.13% | -0.05% | 108% | 99% |
| Class F (optional) | -0.15% | -0.18% | -0.14% | 111% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | -0.31% | 0.04% | 110% | 98% |
| Class C | -0.01% | 0.12% | 0.16% | 105% | 102% |
| Class E | -0.50% | -0.49% | -0.92% | 113% | 99% |
| Overall | -0.18% | -0.21% | -0.16% | 109% | 100% |
| Class D | -0.03% | 0.10% | 0.60% | 102% | 101% |
| Class F (optional) | -0.19% | -0.22% | -0.38% | 103% | 97% |

# Conclusion

In this contribution, it is proposed to solve the 64x64 pipeline implementation issue by disallowing residuals for CUs who’s TUs are not aligned with VPDUs. Two methods are proposed of disallowing residuals. One is to allow only skip blocks and the other method is to allow all inter modes but constrained to have zero residuals..

# References

[1] “Versatile Video Coding (Draft 3),” JVET-L1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, October 2018.

[2] “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao,  October 2018.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).