| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0196 |
| --- | --- |

| Title: | CE5-related : Counter-based multi-CABAC for partial context models |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yinji Piao Kiho Choi | Tel: Email: | [+82-10-6296-4261] [yj1003.piao@samsung.com] [kiho14.choi@samsung.com] |
| Source: | Samsung Electronics Co.,Ltd |  |  |

_____________________________

# Abstract

This contribution provides additional test results of CE5.1.8. Counter-based probability update method is applied only to the context models of non-transform coefficient syntax to remove unnecessary counter for the context models in coefficient coding. Additional 0.01% BD-rate gain can be achieved in RA configuration compared to CE5.1.8 with proposed restriction and the RAM size for the counter is reduced to 1/3.

# Introduction

In two-probability update model, there are two different window sizes for probability update. The final probability is estimated by linear combination of two probabilities with window size equal to N0=16 (short window) and N1=128 (long window):

P0 = (Y >> 4) + P0 – (P0 >> 4)

P1 = (Y >> 7) + P1 – ( P1>> 7)

P = (P0 + P1 + 1) >> 1

where Y = 2^15.

A counter-based two-probability CABAC model is tested in CE5.1.8 [1]. For each context model, a counter is used to count the coded bins. Only short-window probability is updated until the counter reaches 31. Long window model starts updating with the initial value borrowed from the short window mode once the counter reaches 31. To reduce the RAM size for the counter, we propose to apply counter-based update to the partial context models.

# Proposed method

In CE5.1.8, 5bit is required for the counter per context model. There are in total 366 context models in VTM3.0, among which 126 for syntaxes such as flags, modes and indices and 240 for coefficient coding. Compared to the normal context models, those for the coefficient coding get less benefit from the counter-based model since there are sufficient number of bins to be coded in coefficient coding. Therefore, counter-based model is proposed to apply to the partial context models excluding those for coefficient coding. Therefore, 1200bit (5bit*240) RAM size can be saved by this restriction.

# Results

Proposed restriction was implemented on top of VTM 3.0. Tests were conducted under VTM common test condition [2]. The summary result is shown in following table with the result of CE5.1.8 for comparison. More details can be found in accompanying excel sheets.

Table I. Test results of CE5.1.8 & proposed restriction

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE5.1.8 |  |  |  |  | CE5.1.8 + proposed |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.97% | -1.37% | -1.41% | 108% | 103% | -0.94% | -1.38% | -1.45% | 108% | 104% |
| Class A2 | -0.91% | -1.48% | -1.50% | 109% | 105% | -0.90% | -1.51% | -1.54% | 108% | 104% |
| Class B | -0.87% | -1.41% | -1.37% | 107% | 106% | -0.87% | -1.39% | -1.52% | 107% | 106% |
| Class C | -0.81% | -1.30% | -1.40% | 106% | 106% | -0.84% | -1.37% | -1.48% | 108% | 106% |
| Class E | -0.76% | -1.24% | -1.15% | 108% | 110% | -0.79% | -1.26% | -1.12% | 108% | 108% |
| Overall | -0.86% | -1.36% | -1.37% | 108% | 106% | -0.86% | -1.38% | -1.44% | 108% | 106% |
| Class D | -0.69% | -1.08% | -1.12% | 107% | 108% | -0.76% | -1.10% | -1.16% | 105% | 107% |
| Class F | -0.69% | -0.78% | -0.71% | 108% | 109% | -0.72% | -0.78% | -0.64% | 107% | 108% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | CE5.1.8 |  |  |  |  | CE5.1.8 + proposed |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.74% | -0.88% | -1.08% | 106% | 105% | -0.73% | -0.78% | -1.14% | 106% | 105% |
| Class A2 | -0.65% | -0.59% | -0.77% | 106% | 105% | -0.66% | -0.32% | -0.74% | 106% | 105% |
| Class B | -0.71% | -0.88% | -0.65% | 105% | 105% | -0.72% | -0.84% | -0.66% | 108% | 106% |
| Class C | -0.60% | -1.01% | -0.47% | 106% | 111% | -0.66% | -1.18% | -0.53% | 106% | 111% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.68% | -0.86% | -0.71% | 106% | 107% | -0.69% | -0.81% | -0.73% | 107% | 107% |
| Class D | -0.50% | -0.95% | -0.11% | 106% | 109% | -0.57% | -0.79% | -0.57% | 107% | 110% |
| Class F | -0.51% | -0.37% | -0.45% | 105% | 107% | -0.51% | -0.58% | -0.31% | 105% | 108% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE5.1.8 |  |  |  |  | CE5.1.8 + proposed |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.68% | -0.92% | 1.17% | 102% | 101% | -0.65% | -0.30% | 1.63% | 102% | 100% |
| Class C | -0.60% | -0.57% | 0.00% | 102% | 102% | -0.63% | -0.57% | 0.09% | 102% | 104% |
| Class E | -0.20% | -0.10% | 0.08% | 100% | 99% | -0.43% | 0.34% | 0.11% | 100% | 104% |
| Overall | -0.53% | -0.60% | 0.51% | 101% | 101% | -0.59% | -0.23% | 0.74% | 102% | 102% |
| Class D | -0.47% | 0.30% | 1.19% | 103% | 103% | -0.47% | 0.09% | 1.19% | 103% | 103% |
| Class F | -0.56% | 0.10% | -0.84% | 101% | 98% | -0.46% | -0.66% | -0.63% | 101% | 100% |

# Conclusions

This document proposes a counter-based two-probability CABAC model for partial context models. Since the proposed restriction reduces the counter size without performance loss, we suggest adoption of proposed counter-based model with multi-CABAC engine into next version of VTM s/w.

# Reference

- JVET, “CE5: Counter-based probability estimation”, JVET-M0199, 13th JVET Meeting, Marrakech, MA, 9–18 Jan. 2019

- JVET, “JVET common test conditions and software reference configurations”, JVET-L1010, 12th JVET Meeting, Macao, CN, 3-12 Oct., 2018

# Patent rights declaration(s)

Samsung Electronics Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).