| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0202 |
| --- | --- |

| Title: | CE6-realted: Simplification related to MTS with reduced modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiho Choi, Minsoo Park Min Woo Park Woongil Choi Mehdi Salehifar Moonmo Koo Jaehyun Lim Seung Hwan Kim Seongchon-gil 34, Seocho-gu, Seoul, 06765 Korea 10225 Willow Creek Rd. San Diego, CA | Tel: Email: | kiho14.choi@samsung.com m.w.park@samsung.com mss.park@samsung.com woongil.chio@samsung.com mehdi.salehifar@lge.com moonmo.koo@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | Samsung Electronics Ltd. and LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution presents a simplification related to MTS with reduced mode. Currently, MTS in VTM3.0 supports 4 modes with a combination of DST7 and DCT8 along with each direction. In order to reduce encoding complexity of using MTS, four testing results based on reduced modes of MTS with non-normative and normative changes are presented in this contribution. Testing results on the proposed method showed about 11% encoding time reduction with no coding loss in AI scenario, compared to VTM3.0.

Introduction

Currently, VTM3.0 uses three types of transform as DCT2, DCT8, and DST7 depending on supported block size from 4x4 to 32x32. If MTS is disabled, DCT2 is only supported in VTM3.0 with block size from 4x4 to 64x64.

For MTS usage, MTS in VTM3.0 supports 4 modes with a combination of DST7 and DCT8 along with each direction. The use of 4 modes in MTS is a burden to encoder due to the fact that the selected mode to be sent to decoder is decided after 4 times RDO process in the encoder. The 4 times RDO process increases huge encoding complexity especially for all intra scenario.

Proposed method

In order to reduce encoding complexity of using MTS, in this contribution, we present 4 testing results based on reduced modes for MTS with non-normative and normative changes as follows:

- Test1: Non-normative change with 3modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}, {H:DCT8, V:DST7}, {H: DCT8, V:DCT8}

- Test2:Non-normative change with 2modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}, {H:DCT8, V:DST7}, {H: DCT8, V:DCT8}

- Test3:Normative change with 3modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}, {H:DCT8, V:DST7}

- Test4:Normative change with 3modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}

Test1 and 2 conducted an encoder change only when evaluating 4modes for final MTS mode. Specifically, the last one mode or two modes as {H: DCT8, V:DCT8} for Test1 and {H:DCT8, V:DST7}, {H: DCT8, V:DCT8} for Test2 are not evaluated in encoder side. So, syntax structure and parsing process are exactly same to those in VTM3.0.

Test3 and 4 conducted the reduced modes with a syntax change according to the reduced mode number(s). Current VTM3.0 parses two bits for a MTS mode as one bit for horizontal kernel and the other bit for vertical kernel when MTS is decided to be used. For the Test 3 and 4, the truncated unary was used for those tests as shown in the Table 1 and 2.

Table 1. Syntax design for Test3

|  | Horizontal Kernel | Vertical Kernel | Code |
| --- | --- | --- | --- |
| Mode 0 | DST7 | DST7 | 0 |
| Mode 1 | DST7 | DCT8 | 10 |
| Mode 3 | DCT8 | DST7 | 11 |

Table 2. Syntax design for Test4

|  | Horizontal Kernel | Vertical Kernel | Code |
| --- | --- | --- | --- |
| Mode 0 | DST7 | DST7 | 0 |
| Mode 1 | DST7 | DCT8 | 1 |

Test result

In order to check performance change of the proposed method, VTM3.0 SW was used for these experiments, as specified in JVET-L1010 [1].

Test result on Set1

Table 3: Test results compared to VTM

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.02% | 0.02% | 89% | 100% |
| Class A2 | 0.08% | -0.05% | -0.04% | 90% | 100% |
| Class B | 0.05% | -0.05% | -0.03% | 89% | 101% |
| Class C | 0.05% | -0.04% | -0.08% | 89% | 101% |
| Class E | 0.05% | 0.03% | 0.01% | 88% | 98% |
| Overall | 0.05% | -0.03% | -0.03% | 89% | 100% |
| Class D | 0.06% | 0.06% | -0.12% | 90% | 106% |
| Class F (optional) | 0.02% | -0.08% | -0.07% | 90% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.01% | 0.05% | 101% | 103% |
| Class A2 | -0.03% | 0.12% | 0.11% | 99% | 100% |
| Class B | 0.01% | 0.15% | -0.12% | 99% | 102% |
| Class C | 0.05% | -0.25% | 0.04% | 95% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.01% | 0.00% | 98% | 100% |
| Class D | 0.02% | -0.26% | -0.08% | 96% | 97% |
| Class F (optional) | -0.01% | -0.02% | 0.13% | 97% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.32% | -0.14% | 99% | 102% |
| Class C | 0.06% | 0.19% | 0.37% | 99% | 101% |
| Class E | 0.08% | 0.59% | -0.33% | 98% | 89% |
| Overall | 0.04% | 0.07% | -0.02% | 99% | 98% |
| Class D | -0.02% | -0.03% | 0.28% | 100% | 106% |
| Class F (optional) | 0.02% | 0.31% | -0.12% | 100% | 101% |

Testing results on the reduced encoding time of the proposed method show 11%, 2%, and 1% on average for All Intra (AI), Random Access (RA), and Low Delayed B (LDB), respectively, compared to VTM3.0.

Coding performance changes on the proposed method show 0.05%, 0.02%, and 0.04% BD rate losses on average for All Intra (AI), Random Access (RA), and Low Delayed B (LDB), respectively, compared to VTM3.0.

Test result on Set2

Table 4: Test results compared to VTM

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.20% | -0.13% | -0.03% | 77% | 97% |
| Class A2 | 0.24% | -0.02% | -0.05% | 79% | 99% |
| Class B | 0.19% | -0.07% | -0.10% | 77% | 98% |
| Class C | 0.16% | -0.19% | -0.20% | 76% | 100% |
| Class E | 0.18% | -0.06% | -0.01% | 76% | 97% |
| Overall | 0.19% | -0.10% | -0.09% | 77% | 98% |
| Class D | 0.16% | -0.02% | -0.02% | 76% | 101% |
| Class F (optional) | 0.09% | -0.03% | 0.03% | 78% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.12% | 0.08% | 98% | 102% |
| Class A2 | 0.02% | -0.07% | 0.01% | 99% | 104% |
| Class B | 0.08% | 0.17% | 0.02% | 98% | 101% |
| Class C | 0.14% | -0.14% | 0.00% | 94% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.03% | 0.03% | 97% | 101% |
| Class D | 0.06% | -0.54% | -0.15% | 94% | 96% |
| Class F (optional) | 0.07% | -0.01% | -0.05% | 95% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.20% | 0.01% | 97% | 99% |
| Class C | 0.07% | 0.07% | 0.11% | 99% | 97% |
| Class E | 0.15% | 1.04% | -0.74% | 96% | 90% |
| Overall | 0.08% | 0.20% | -0.14% | 97% | 96% |
| Class D | 0.05% | 0.08% | 0.47% | 97% | 102% |
| Class F (optional) | 0.12% | -0.51% | -0.50% | 98% | 100% |

Testing results on the reduced encoding time of the proposed method show 23%, 3%, and 3% on average for All Intra (AI), Random Access (RA), and Low Delayed B (LDB), respectively, compared to VTM3.0.

Coding performance changes on the proposed method show 0.19%, 0.08%, and 0.08% BD rate losses on average for All Intra (AI), Random Access (RA), and Low Delayed B (LDB), respectively, compared to VTM3.0.

Test result on Set3

Table 5: Test results compared to VTM

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.08% | -0.03% | 89% | 99% |
| Class A2 | 0.03% | 0.02% | -0.03% | 90% | 99% |
| Class B | 0.01% | -0.05% | -0.04% | 88% | 100% |
| Class C | 0.00% | -0.04% | -0.23% | 89% | 99% |
| Class E | 0.00% | -0.01% | 0.09% | 88% | 96% |
| Overall | 0.01% | -0.03% | -0.06% | 89% | 99% |
| Class D | 0.01% | -0.08% | -0.16% | 89% | 103% |
| Class F (optional) | -0.01% | -0.05% | -0.04% | 90% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.12% | 0.08% | 99% | 102% |
| Class A2 | -0.01% | -0.01% | 0.03% | 99% | 101% |
| Class B | 0.00% | 0.23% | -0.11% | 99% | 101% |
| Class C | 0.00% | -0.20% | 0.13% | 96% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.02% | 98% | 100% |
| Class D | -0.03% | -0.52% | -0.40% | 98% | 99% |
| Class F (optional) | 0.02% | 0.10% | 0.10% | 97% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.07% | -0.09% | 98% | 101% |
| Class C | 0.06% | 0.31% | 0.20% | 98% | 97% |
| Class E | 0.11% | 0.18% | -0.15% | 95% | 88% |
| Overall | 0.04% | 0.12% | -0.01% | 97% | 96% |
| Class D | -0.09% | -0.33% | 0.17% | 97% | 97% |
| Class F (optional) | 0.05% | 0.24% | -0.29% | 98% | 95% |

Testing results on the reduced encoding time of the proposed method show 11%, 2%, and 3% on average for All Intra (AI), Random Access (RA), and Low Delayed B (LDB), respectively, compared to VTM3.0.

Coding performance changes on the proposed method show 0.01%, 0.00%, and 0.04% BD rate losses on average for All Intra (AI), Random Access (RA), and Low Delayed B (LDB), respectively, compared to VTM3.0.

Test result on Set4

Table 6: Test results compared to VTM

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.16% | -0.12% | 0.02% | 78% | 100% |
| Class A2 | 0.21% | -0.06% | -0.09% | 79% | 100% |
| Class B | 0.14% | -0.15% | -0.15% | 77% | 100% |
| Class C | 0.11% | -0.16% | -0.25% | 77% | 101% |
| Class E | 0.13% | -0.09% | -0.01% | 76% | 95% |
| Overall | 0.15% | -0.12% | -0.11% | 77% | 99% |
| Class D | 0.11% | -0.23% | -0.22% | 78% | 109% |
| Class F (optional) | 0.06% | -0.15% | -0.07% | 78% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.03% | 0.12% | 99% | 103% |
| Class A2 | 0.02% | 0.03% | 0.06% | 97% | 101% |
| Class B | 0.06% | 0.09% | -0.14% | 98% | 102% |
| Class C | 0.07% | -0.16% | 0.26% | 95% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | -0.01% | 0.06% | 97% | 101% |
| Class D | 0.06% | -0.25% | -0.13% | 95% | 98% |
| Class F (optional) | 0.08% | -0.20% | -0.05% | 93% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.40% | -0.15% | 98% | 101% |
| Class C | 0.03% | 0.08% | 0.16% | 96% | 98% |
| Class E | 0.13% | 1.49% | -1.04% | 96% | 90% |
| Overall | 0.06% | 0.23% | -0.27% | 97% | 97% |
| Class D | -0.01% | -0.09% | -0.12% | 98% | 102% |
| Class F (optional) | 0.07% | -0.39% | -0.16% | 98% | 97% |

Testing results on the reduced encoding time of the proposed method show 23%, 3%, and 3% on average for All Intra (AI), Random Access (RA), and Low Delayed B (LDB), respectively, compared to VTM3.0.

Coding performance changes on the proposed method show 0.15%, 0.05%, and 0.06% BD rate losses on average for All Intra (AI), Random Access (RA), and Low Delayed B (LDB), respectively, compared to VTM3.0.

Test result on Set1with MTS inter

In order to check testing results on Set1 with MTS inter=on, additional testing was conducted. For the testing, anchor was MTS intra and inter on as the setting of EMT=3 and EMTfast=3.

Table 3: Test results compared to VTM

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.02% | 0.02% | 89% | 100% |
| Class A2 | 0.08% | -0.05% | -0.04% | 90% | 100% |
| Class B | 0.05% | -0.05% | -0.03% | 89% | 101% |
| Class C | 0.05% | -0.04% | -0.08% | 89% | 101% |
| Class E | 0.05% | 0.03% | 0.01% | 88% | 98% |
| Overall | 0.05% | -0.03% | -0.03% | 89% | 100% |
| Class D | 0.06% | 0.06% | -0.12% | 90% | 106% |
| Class F (optional) | 0.02% | -0.08% | -0.07% | 90% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.08% | 0.07% | 97% | 104% |
| Class A2 | 0.19% | -0.07% | -0.02% | 97% | 101% |
| Class B | 0.13% | 0.02% | -0.13% | 96% | 102% |
| Class C | 0.12% | -0.08% | -0.07% | 96% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.13% | -0.05% | -0.05% | 97% | 103% |
| Class D | 0.16% | -0.34% | -0.12% | 95% | 101% |
| Class F (optional) | 0.06% | -0.14% | 0.10% | 95% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.19% | 0.07% | -0.38% | 94% | 96% |
| Class C | 0.20% | -0.04% | 0.22% | 96% | 103% |
| Class E | 0.18% | 0.69% | -0.34% | 98% | 101% |
| Overall | 0.19% | 0.19% | -0.17% | 96% | 99% |
| Class D | 0.31% | -0.03% | -0.69% | 98% | 112% |
| Class F (optional) | 0.12% | -0.32% | -0.65% | 96% | 95% |

Testing results on the reduced encoding time of the proposed method show 11%, 3%, and 4% on average for All Intra (AI), Random Access (RA), and Low Delayed B (LDB), respectively, compared to VTM3.0.

Coding performance changes on the proposed method show 0.05%, 0.13%, and 0.19% BD rate losses on average for All Intra (AI), Random Access (RA), and Low Delayed B (LDB), respectively, compared to VTM3.0.

Test result on Set3with MTS inter

In order to check testing results on Set3 with MTS inter=on, additional testing was conducted. For the testing, anchor was MTS intra and inter on as the setting of EMT=3 and EMTfast=3.

Table 3: Test results compared to VTM

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.08% | -0.03% | 89% | 99% |
| Class A2 | 0.03% | 0.02% | -0.03% | 90% | 99% |
| Class B | 0.01% | -0.05% | -0.04% | 88% | 100% |
| Class C | 0.00% | -0.04% | -0.23% | 89% | 99% |
| Class E | 0.00% | -0.01% | 0.09% | 88% | 96% |
| Overall | 0.01% | -0.03% | -0.06% | 89% | 99% |
| Class D | 0.01% | -0.08% | -0.16% | 89% | 103% |
| Class F (optional) | -0.01% | -0.05% | -0.04% | 90% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.03% | 0.03% | 97% | 100% |
| Class A2 | 0.10% | -0.13% | -0.09% | 97% | 100% |
| Class B | 0.05% | 0.02% | -0.18% | 96% | 102% |
| Class C | 0.02% | -0.14% | -0.02% | 96% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | -0.06% | -0.08% | 96% | 101% |
| Class D | 0.06% | -0.77% | -0.12% | 95% | 102% |
| Class F (optional) | 0.02% | 0.01% | 0.04% | 95% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.21% | -0.28% | 94% | 99% |
| Class C | 0.04% | -0.37% | 0.00% | 95% | 100% |
| Class E | -0.02% | 0.11% | -0.31% | 98% | 106% |
| Overall | 0.03% | -0.01% | -0.20% | 95% | 101% |
| Class D | 0.10% | -0.31% | -0.40% | 95% | 98% |
| Class F (optional) | 0.00% | 0.20% | 0.61% | 96% | 96% |

Testing results on the reduced encoding time of the proposed method show 11%, 4%, and 5% on average for All Intra (AI), Random Access (RA), and Low Delayed B (LDB), respectively, compared to VTM3.0.

Coding performance changes on the proposed method show 0.05%, 0.05%, and 0.03% BD rate losses on average for All Intra (AI), Random Access (RA), and Low Delayed B (LDB), respectively, compared to VTM3.0.

Conclusion

This contribution presents a simplification related to MTS with reduced modes. Currently, MTS in VTM3.0 supports 4 modes with a combination of DST7 and DCT8 along with each direction. For MTS usage, MTS in VTM3.0 supports 4 modes with a combination of DST7 and DCT8 along with each direction. The use of 4 modes in MTS is a burden to encoder due to the fact that the selected mode to be sent to decoder is decided after 4 times RDO process in the encoder. The 4 times RDO process increases huge encoding complexity especially for all intra scenario. In order to reduce encoding complexity of using MTS, four testing results based on reduced modes of MTS with non-normative and normative changes are presented in this contribution. Testing results on the proposed method showed about 11% encoding time reduction with no coding loss in AI scenario, compared to VTM3.0. Therefore, it is recommend to adopt the proposed method into the next version of test model of VVC.

Reference

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, Macao, CN, Oct. 2018

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard.

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard.