| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0206 |
| --- | --- |

| Title: | CE4-related: MMVD improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungsoo Jeong Min Woo Park Kiho Choi Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | ss00.jeong@samsung.com m.w.park@samsung.com kiho14.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution proposes a method for improving the binarization of MMVD signalling. This includes three tests. The first test is the binarization method change of distance list index. There is no additional CTX model and other change. Experimental results shows -0.08% luma BD-rate change in RA and 0.01% luma BD-rate change in LDB. The second test is adding new distance group syntax with CTX model. Experimental results shows -0.10% luma BD-rate change in RA and 0.01% luma BD-rate change in LDB. The third test is context initial value change test of Test 1. Experimental results shows -0.08% luma BD-rate change in RA and 0.02% luma BD-rate change in LDB.

Introduction

In the process of distance index signalling, distance index is signalled with truncated unary coding method. It is described at Table 1.

Table 1. Distance list index binarization

| Distance list index | binarization |
| --- | --- |
| 0 | 0 |
| 1 | 10 |
| 2 | 110 |
| 3 | 1110 |
| 4 | 11110 |
| 5 | 111110 |
| 6 | 1111110 |
| 7 | 1111111 |

Proposed Method

Method 1: only binarization method change

Current binarization method has an advantage in distance index coding at small resolution. However, proposed binarization method has an advantage at small and large resolution. There is no additional syntax and CTX model.

Table 2. Proposed binarization method

| Distance list index | binarization |
| --- | --- |
| 0 | 00 |
| 1 | 01 |
| 2 | 10 |
| 3 | 110 |
| 4 | 1110 |
| 5 | 11110 |
| 6 | 111110 |
| 7 | 111111 |

Table 3. The result of method 1

Method 2: one more CTX model with Method 1

In this method, one more context model is used on top of Method 1. If first bin is ‘1’, second bin uses context model for better performance.

Method 3: CTX initial value change with Method 1

In this method, the context initial value of Method 1 is changed to 154.

Conclusion

This contribution proposes a method for improving the binarization of MMVD signalling. All the proposed methods show coding performance without no complexity increasement. Therefore, it is suggested to adopt proposed methods into the next VVC working draft text.

The simulation was carried out under the JVET common test condition [2]. The software was compiled with GCC version 7.3 and the AVX2 SIMD instructions. Table 1 shows summary of all results.

References

[1] “Versatile Video Coding (Draft 3),” JVET-L1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, October 2018.

[2] “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, October 2018.

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).