| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0212-v3 |
| --- | --- |

| Title: | CE3-related: Improved reference samples range for MDLM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuai Wan Qihong Ran Xinwei Li Yanzhuo Ma Junyan Huo Fuzheng Yang YuanFang Yu Yang Liu | Email: | swan@nwpu.edu.cn qhRan@stu.xidian.edu.cn xwli_xidian@stu.xidian.edu.cn yzma@mail.xidian.edu.cn jyhuo@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Northwestern Polytechnical University, Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

# Abstract

Multi-directional LM (MDLM) has been adopted into VVC at the previous JVET meeting. This contribution designs a method to reduce the reference samples range for MDLM parameter derivation. Specifically, we limit the reference samples range to the last three quarter of the top line or the left column. This method can get -0.02%, -0.11%, and -0.18% BD-rates on Y, Cb, and Cr components in All Intra configuration.

# Introduction

The current VVC includes a multi-directional LM (MDLM) mode. Apart from the above and left template which can be used to calculate the linear model coefficients together, MDLM introduces two other 2 LM modes, called LM_A and LM_L modes.

In the LM_A mode, only the above template is used to calculate the linear model parameters. To get more samples, the above template is extended to the size of (W+W).

Figure 1: LM_A mode

In the LM_L mode, only the left template is used to calculate the linear model parameters. To get more samples, the left template is extended to the size of (H+H).

Figure 2: LM_L mode

In the current VVC, a simplified line fitting method of the cross-component linear model (CCLM) has been adopted. This method uses two Luma samples and their corresponding chroma samples to obtain the linear model parameters α and β.

A

B

Figure 3:Illustration of the straight line between minimum and maximum Luma value

The linear model parameters α and β are obtained according to the following equations:

β= ∑Cn-α·∑LnN

α=yB-yAxB-xA                                        （1）

β= yA-αxA                                      （2）

# Proposed method

In our proposed method, the MDLM method removes the first quarter reference samples. The last three quarters of reference samples are used for MDLM, as shown in Figure 4 and Figure 5. This method can reduce the number of comparisons in the maximum and minimum luma determination process. Actually, the last three quarters of reference samples are more similar with the coding block than the first quarter if LM_A or LM_T applies.

Figure 4: LM_A mode with 3/4 reference samples range

Figure 5: LM_L mode with 3/4 reference samples range

# Experimental results

The proposed scheme is implemented based on VTM3.0. The coding performance of All Intra configuration is provided in Table 1 and Table 2.

Test1

Anchor: VTM3.0

Tested: The proposed method

Table 1. Summary of experimental results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.14% | -0.33% |  | 99% |
| Class A2 | -0.01% | -0.05% | -0.04% |  | 99% |
| Class B | -0.01% | -0.17% | -0.21% |  | 99% |
| Class C | -0.03% | -0.11% | -0.27% |  | 100% |
| Class E | -0.01% | -0.02% | 0.01% |  | 100% |
| Overall | -0.02% | -0.11% | -0.18% |  | 99% |
| Class D | -0.01% | -0.06% | -0.12% |  | 100% |
| Class F | 0.00% | 0.03% | -0.09% |  | 100% |

Test2

Anchor: VTM3.0

Tested: The proposed method combined with JVET_M0211

Table 2. Summary of experimental results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.51% | -0.54% |  | 99% |
| Class A2 | -0.03% | -0.21% | -0.11% |  | 100% |
| Class B | -0.04% | -0.36% | -0.59% |  | 100% |
| Class C | -0.08% | -0.23% | -0.43% |  | 99% |
| Class E | -0.03% | -0.07% | -0.13% |  | 99% |
| Overall | -0.06% | -0.28% | -0.39% |  | 99% |
| Class D | -0.04% | -0.21% | -0.14% |  | 98% |
| Class F | -0.05% | -0.11% | -0.21% |  | 100% |

# Conclusions

This contribution presents a modified MDLM method to reduce the reference samples range. We assume the last three quarters of reference samples are highly related with the coding block if LM_A or LM_L is selected. The proposed method gives 0.02%, 0.11%%, and 0.18% BD rate savings on Y, Cb and Cr components for All Intra configuration.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).