| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0216-v3 |
| --- | --- |

| Title: | CE10-related: syntax clean-up on triangle prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li Junghak Nam Naeri Park Hyeongmun Jang Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 | Tel: Email: | +82-2-6912-6561 {l.li, junghak.nam, naeri.park, hm.jang, jaehyun.lim, seunghwan3.kim }@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes syntax clean-up on triangle prediction. Two aspects are proposed:

1) merge_triangle_flag is not parsed when mmvd_flag is 1 or mh_intra_flag is 1.

2) Condition for parsing merge_triangle_flag: sps_triangle_enabled_flag && slice_type == B &&  cbWidth * cbHeight >= 64.

The experiment result reportedly shows overall -0.07% and -0.14% luma BD-rate changes for RA and LDB, respectively.

Proposed Method

At the last JVET-L meeting, several inter coding tools were adopted into VTM 3.0 [1], such as MMVD, MH_Intra and triangle prediction.

Compared VVC (L1001-v7) to VTM3.0, there are several issues as described in Table 1.

Table 1. Syntax related to triangle prediction

| aspects | VVC draft (L1001-v7) | VTM 3.0 |
| --- | --- | --- |
| merge_triangle_flag parsing regards to mmvd_flag | merge_triangle_flag is not parsed when mmvd_flag is 1 | merge_triangle_flag is parsed regardless of mmvd_flag |
| merge_triangle_flag parsing regards to mh_intra_flag | merge_triangle_flag is parsed regardless of mh_intra_flag | merge_triangle_flag is parsed regardless of mh_intra_flag |
| Condition for parsing merge_triangle_flag | sps_triangle_enabled_flag && slice_type = = B && cbWidth * cbHeight >= 16 | sps_triangle_enabled_flag && slice_type = = B && cbWidth * cbHeight >= 64 |

Thus, following methods are proposed to clean-up the above mentioned bugs.

- merge_triangle_flag is bypassed when mmvd_flag is 1 or mh_intra_flag is 1

- condition for parsing merge_triangle_flag:

sps_triangle_enabled_flag && slice_type = = B && cbWidth * cbHeight >= 64

Experimental Results

The proposed method is implemented on top of VTM 3.0, and evaluated for VTM configuration according to the common test conditions [2].

Table 2 Experiment result on random access (RA)

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.08% | -0.08% | 100% | 101% |
| Class A2 | -0.10% | -0.09% | -0.02% | 100% | 101% |
| Class B | -0.07% | -0.01% | -0.18% | 100% | 102% |
| Class C | -0.07% | -0.06% | -0.09% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.05% | -0.11% | 100% | 102% |
| Class D | -0.10% | -0.20% | -0.07% | 101% | 103% |
| Class F | -0.03% | -0.01% | -0.03% | 101% | 103% |

Table 3 Experiment result on low delay B (LDB)

|  | Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.10% | -0.32% | 0.00% | 101% | 106% |
| Class C | -0.19% | 0.03% | -0.14% | 101% | 103% |
| Class E | -0.13% | 0.03% | -0.17% | 101% | 105% |
| Overall | -0.14% | -0.12% | -0.09% | 101% | 105% |
| Class D | -0.17% | 0.13% | 0.13% | 101% | 105% |
| Class F | 0.00% | -0.14% | -0.54% | 101% | 104% |

Proposed Specification Text

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mh_intra_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| mh_intra_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| mh_intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| mh_intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( mh_intra_flag[ x0 ][ y0 ] = = 0 && sps_triangle_enabled_flag && slice_type = = B && cbWidth * cbHeight >= 16 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) |  |
| merge_triangle_idx[ x0 ][ y0 ] | ae(v) |
| else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |

Summary

The proposed methods shows overall -0.07% and -0.14% % luma BD-rate changes for RA and LDB. It is recommended to adopt proposed syntax clean-up to VVC.

References

- VTM 3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, Document of Joint Video Experts Team (JVET), JVET-L1010, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).