| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0217-v3 |
| --- | --- |

| Title: | CE2-related: Constructed affine merge candidate simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li Junghak Nam Naeri Park Hyeongmun Jang Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 | Tel: Email: | +82-2-6912-6561 {l.li, junghak.nam, naeri.park, hm.jang, jaehyun.lim, seunghwan3.kim }@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

Current VTM 3.0 can have up to six constructed affine merge candidates when 3 CPMV and TMVP are all available. In this proposal, TMVP is not considered as one of the CPMVs, thus up to 3 constructed affine merge candidates are used. The maximum number of MV comparison reduces from 20 to 8, and the maximum number of reference index comparison reduces from 52 to 22. The experiment result reportedly shows 0.01% and 0.09% bit rate changes for RA and LDB.

Proposed Method

For existing affine merge list construction, candidates are considered in following order.

- Sub-block based temporal motion vector prediction

- Inherited affine merge candidates

- Constructed affine merge candidates

- Zero MVs

In the process of considering constructed affine merge candidates, three spatial neighbors and one temporal neighbors are used for derivation of motion information for the control points as shown in Figure 1. CPMVs for each neighbor are derived:

- CPMV0 from checking B2->B3->A2

- CPMV1 from checking B1->B0

- CPMV2 from checking A1->A0

- CPMV3 from TMVP

After MVs of four control points are derived, constructed affine merge candidates are based on combination of CPMVs with pre-defined order. {CPMV0, CPMV1, CPMV2}, {CPMV0, CPMV1, CPMV3}, {CPMV0, CPMV2, CPMV3}, {CPMV1, CPMV2, CPMV3}, {CPMV0, CPMV1} and {CPMV0, CPMV2}. The worst case happens when all four CPMVs are available, then it has six constructed affine merge candidates to be included into affine merge list.

In this proposal, TMVP , which may be considered as CPMV3, is not considered. With only CPMV0, CPMV1 and CPMV2, the combinations could be {CPMV0, CPMV1, CPMV2}, {CPMV0, CPMV1} and {CPMV0, CPMV2}. Therefore, available constructed affine merge candidates reduces from six to three with only one 6-parameter and two 4-paramaters. The maximum number of MV comparison reduces from 20 to 8, and the maximum number of reference index comparison reduces from 52 to 22.

Figure 1. Locations of Candidates position for constructed affine merge

Experimental Results

The proposed method is implemented on top of VTM 3.0, and evaluated for VTM configuration according to the common test conditions [2].

Table 1 Experiment result for random access (RA)

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.03% | 0.03% | 100% | 100% |
| Class A2 | 0.01% | 0.01% | 0.11% | 100% | 101% |
| Class B | 0.01% | 0.07% | -0.03% | 100% | 103% |
| Class C | 0.02% | -0.03% | 0.00% | 100% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.02% | 100% | 102% |
| Class D | 0.02% | -0.03% | 0.03% | 101% | 104% |
| Class F | 0.03% | -0.01% | 0.04% | 101% | 104% |

Table 2 Experiment result for low delay B (LDB)

|  | Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.28% | -0.14% | 101% | 105% |
| Class C | 0.09% | 0.26% | -0.02% | 101% | 103% |
| Class E | 0.11% | 0.10% | 0.13% | 101% | 105% |
| Overall | 0.09% | 0.00% | -0.04% | 101% | 105% |
| Class D | 0.10% | 0.07% | -0.39% | 101% | 105% |
| Class F | 0.11% | 0.29% | -0.02% | 101% | 105% |

Summary

The proposed simplification on constructed affine merge candidates shows 0.01% and 0.09% bit rate changes for RA and LDB. It is recommended to adopt this proposal to VVC.

References

- VTM 3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, Document of Joint Video Experts Team (JVET), JVET-L1010, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).