| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0218-v1 |
| --- | --- |

| Title: | CE3 : Simplified MDMS (test 3.3.1 and test 3.3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Choi, Jin Heo, Sunmi Yoo, Ling Li, Jungah Choi, Jaehyun Lim, Seung-Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 { jangwon84.choi, jin78.heo, sunmi.yoo, l.li, jungah.choi, jaehyun.lim, seunghwan3.kim }@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of simplified MDMS, which are the test CE3-3.3.1 and test CE3-3.3.2. In these methods, chroma intra candidate modes are derived from 2-point DM searching. The mode coding of the chroma intra modes remains the same as VTM3.0.

Experimental results of test 3.3.1 which uses 5 chroma derived modes show -0.03%, -0.88%, and -0.86% BD-rates on Y, Cb, and Cr components, respectively in All Intra configuration, and -0.01%, -0.46%, and -0.50% BD-rates in Random Access configuration.

Experimental results of test 3.3.2 which uses 3 chroma derived modes show -0.02%, -0.29%, and -0.20% BD-rates with 97% enc-time on Y, Cb, and Cr components, respectively, in All Intra configuration, and 0.00%, -0.12%, and -0.05% BD-rates with 99% enc-time in Random Access configuration.

Proposed method

Figure 1 Example of corresponding luma sub-blocks for a chroma block in I slice.

As introduced in JVET-L0139[1], five or three chroma intra candidate modes are constructed with the two DM points (CR, and TL as shown in Figure 1). The chroma intra candidate mode list for five chroma intra modes (test 3.3.1) is generated as shown in Figure 2.

Figure 2 Flow chart of the proposed method (test3.3.1).

As similar to the VTM3.0, a luma intra mode of the center position DM(CR) is selected firstly for the chroma intra candidate mode. And then, the rest 4 chroma intra candidate modes are derived from the two DM points (CR, TL) condition check process. The mode coding including binarization, context (or bypass) coding and the parsing order of the chroma intra modes with CCLM remains the same as the VTM3.0.

In the test 3.3.2, three chroma intra modes are derived with the similar way to the test 3.3.1, as shown in Figure 3

Figure 3 Flow chart of the proposed method (test3.3.2).

Table 1 shows the detailed complexity comparison in the worst case of VTM3, JEM MDMS, and proposed two tests.

Table 1 Complexity analysis of VTM3, JEM MDMS, and proposed two test without CCLM prediction

|  | Max number of Luma DM point to access | Max number of neighbors to access | Max layers of if conditions | Max number of comparison operator | Max number of logical operators | Max number of assignment operators |
| --- | --- | --- | --- | --- | --- | --- |
| VTM3 | 1 | 0 | 1 | 5 | 0 | 15 |
| Test3.3.1 | 2 | 0 | 4 | 4 | 2 | 13 |
| Test3.3.2 | 2 | 0 | 3 | 3 | 1 | 12 |
| JEM MD MS | 5 | 5 | 4 | 39 | 28 | 80 |

|  | Max number of increments | Max number of bit operation | Number of Context coded bins | number of full RDO checks | Has LUT? | LUT size |
| --- | --- | --- | --- | --- | --- | --- |
| VTM3 | 4 | 2 | 1 | 5 | N | 0 |
| Test3.3.1 | 0 | 4 | 1 | 5 | N | 0 |
| Test3.3.2 | 0 | 2 | 1 | 3 | N | 0 |
| JEM MD MS | 5 | 2 | 5 | 5 | Y | 68 |

Experimental results

The proposed method has been implemented using VTM3.0 as software base according to the common test conditions defined in JVET-L1010 [2] and Core Experiment description in JVET-L1023[3].

Table 2 and Table 3 show the test results of the test 3.3.1 in AI and RA configurations.

Table 2 Experimental results of test 3.3.1 for all-intra (AI) test condition; anchor is VTM3.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.60% | -0.65% | 100% | 100% |
| Class A2 | -0.02% | -0.69% | -0.66% | 100% | 99% |
| Class B | -0.03% | -0.82% | -0.87% | 100% | 100% |
| Class C | -0.03% | -0.84% | -1.00% | 99% | 100% |
| Class E | -0.02% | -1.47% | -1.06% | 99% | 100% |
| Overall | -0.03% | -0.88% | -0.86% | 100% | 100% |
| Class D | -0.01% | -0.43% | -0.57% | 100% | 99% |
| Class F | -0.01% | -0.50% | -0.53% | 100% | 100% |

Table 3 Experimental results of test 3.3.1 for random-access (RA) test condition; anchor is VTM3.0

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.28% | -0.33% | 100% | 100% |
| Class A2 | -0.03% | -0.59% | -0.28% | 100% | 100% |
| Class B | -0.01% | -0.51% | -0.78% | 100% | 99% |
| Class C | 0.00% | -0.45% | -0.43% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.46% | -0.50% | 100% | 100% |
| Class D | 0.02% | -0.66% | -0.76% | 100% | 100% |
| Class F | 0.01% | -0.33% | -0.41% | 100% | 100% |

Also, Table 4 and 5 show the test results of test 3.3.2 in AI and RA configurations.

Table 4 Experimental results of test 3.3.2 for all-intra (AI) test condition; anchor is VTM3.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.11% | -0.28% | 97% | 100% |
| Class A2 | 0.01% | -0.10% | -0.24% | 96% | 99% |
| Class B | -0.02% | -0.34% | -0.18% | 98% | 99% |
| Class C | -0.04% | -0.25% | -0.27% | 97% | 99% |
| Class E | 0.01% | -0.63% | 0.00% | 98% | 100% |
| Overall | -0.02% | -0.29% | -0.20% | 97% | 100% |
| Class D | -0.02% | 0.02% | 0.02% | 97% | 98% |
| Class F | -0.03% | -0.05% | -0.07% | 97% | 100% |

Table 5 Experimental results of test 3.3.2 for random-access (RA) test condition; anchor is VTM3.0

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.14% | -0.19% | 99% | 100% |
| Class A2 | -0.01% | -0.15% | -0.03% | 99% | 99% |
| Class B | 0.00% | -0.07% | -0.12% | 99% | 100% |
| Class C | 0.01% | -0.13% | 0.13% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.12% | -0.05% | 99% | 100% |
| Class D | 0.01% | -0.09% | 0.07% | 99% | 101% |
| Class F | 0.00% | -0.10% | 0.07% | 99% | 100% |

Reference

- J. Choi, et al., “CE3-related : Simplified MDMS”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JVET-L0139, 12th Meeting, Macao, CN, 3-12 Oct. 2018.

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L1010, 12th Meeting, Macao, CN, 3-12 Oct. 2018.

- G. Van der Auwera, et al., “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L1023, 12th Meeting, Macao, CN, 3-12 Oct. 2018.

Patent rights declaration

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Specification of test 3.3.1

(section 8.2.3 in VVC specification)

Input to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived by the following ordered steps:

- The variables dmModeCR and dmModeTL are derived as follows:

- dmModeCR is set equal to IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ].

- dmModeTL is set equal to IntraPredModeY[ xCb ][ yCb ].

- For X being replaced by either 4 or 7, the variable X is derived as follows:

- If sps_cclm_enabled_flag is equal to 0, X is set equal to 4.

- If sps_cclm_enabled_flag is equal to 1, X is set equal to 7.

- The candModeListC[ x ] with x = 0..X is derived as follows:

candModeListC[ X ] = dmModeCR		(8-x)

- If dmModeCR is equal to dmModeTL, candModeListC[ x ] with x = 0..3 is derived as follows:

  - If dmModeCR is greater than 1, the following applies:

candModeListC[ 0 ] = 2 + ( ( dmModeCR + 61 ) % 64 )	(8-x)

candModeListC[ 1 ] = 2 + ( ( dmModeCR - 1 ) % 64 )	(8-x)

candModeListC[ 2 ] = 0		(8-x)

candModeListC[ 3 ] = 1		(8-x)

  - Otherwise (dmModeCR is not greater than 1), the following applies:

candModeListC[ 0 ] = 1 - dmModeCR	(8-x)

candModeListC[ 1 ] = 50		(8-x)

candModeListC[ 2 ] = 18		(8-x)

candModeListC[ 3 ] = 66		(8-x)

- Otherwise (dmModeCR is not equal to dmModeTL), candModeListC[ x ] with x = 0..3 is derived as follows:

candModeListC[ 0 ] = dmModeTL	(8-x)

  - If dmModeCR + dmModeTL is equal to 1, the following applies:

candModeListC[ 1 ] = 50		(8-x)

candModeListC[ 2 ] = 18		(8-x)

candModeListC[ 3 ] = 66		(8-x)

  - Otherwise if dmModeCR and dmModeTL are both greater than 1, the following applies:

candModeListC[ 1 ] = 0		(8-x)

candModeListC[ 2 ] = 1		(8-x)

  - If 2 + ( ( dmModeCR + 61 ) % 64 ) is equal to dmModeTL, the following applies:

candModeListC[ 3 ] = 2 + ( ( dmModeCR - 1 ) % 64 )	(8-x)

  - Otherwise (2 + ( ( dmModeCR + 61 ) % 64 ) is not equal to dmModeTL), the following applies:

candModeListC[ 3 ] = 2 + ( ( dmModeCR + 61 ) % 64 )	(8-x)

  - Otherwise, the following applies:

  - If dmModeCR && dmModeTL is equal to 1, the following applies:

candModeListC[ 1 ] = 0		(8-x)

candModeListC[ 2 ] = 2 + ( ( dmModeCR + dmModeTL + 60 ) % 64 )	(8-x)

candModeListC[ 3 ] = 2 + ( ( dmModeCR + dmModeTL - 2 ) % 64 )	(8-x)

  - Otherwise (dmModeCR && dmModeTL is equal to 0), the following applies:

candModeListC[ 1 ] = 1		(8-x)

candModeListC[ 2 ] = 2 + ( ( dmModeCR + dmModeTL + 61 ) % 64 )	(8-x)

candModeListC[ 3 ] = 2 + ( ( dmModeCR + dmModeTL – 1 ) % 64 )	(8-x)

- If sps_cclm_enabled_flag is equal to 1, candModeListC[ x ] with x = 4..6 is derived as follows:

candModeListC[ 4 ] = 81		(8-x)

candModeListC[ 5 ] = 82		(8-x)

candModeListC[ 6 ] = 83		(8-x)

- IntraPredModeC[ xCb ][ yCb ] is set equal to  candModeListC[ intra_chroma_pred_mode[ xCb ][ yCb ]].

The chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived using intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] as specified in Table 8-2 and Table 8-3.

Table 8-2 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] when sps_cclm_enabled_flag is equal to 0

| intra_chroma_pred_mode[ xCb ][ yCb ] | IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 0 | 50 | 18 | 1 | X |

Table 8-3 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] when sps_cclm_enabled_flag is equal to 1

| intra_chroma_pred_mode[ xCb ][ yCb ] | IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 81 | 81 | 81 | 81 | 81 |
| 5 | 82 | 82 | 82 | 82 | 82 |
| 6 | 83 | 83 | 83 | 83 | 83 |
| 7 | 0 | 50 | 18 | 1 | X |

Specification of test 3.3.2

(section 8.2.3 in VVC specification)

Input to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived by the following ordered steps:

- The variables dmModeCR and dmModeTL are derived as follows:

- dmModeCR is set equal to IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ].

- dmModeTL is set equal to IntraPredModeY[ xCb ][ yCb ].

- For X being replaced by either 2 or 5, the variable X is derived as follows:

- If sps_cclm_enabled_flag is equal to 0, X is set equal to 2.

- If sps_cclm_enabled_flag is equal to 1, X is set equal to 5.

- The candModeListC[ x ] with x = 0..X is derived as follows:

candModeListC[ X ] = dmModeCR		(8-x)

- If dmModeCR is equal to dmModeTL, candModeListC[ x ] with x = 0,1 is derived as follows:

  - If dmModeCR is greater than 1, the following applies:

candModeListC[ 0 ] = 2 + ( ( dmModeCR + 61 ) % 64 )	(8-x)

candModeListC[ 1 ] = 2 + ( ( dmModeCR - 1 ) % 64 )	(8-x)

  - Otherwise (dmModeCR is not greater than 1), the following applies:

candModeListC[ 0 ] = 1 - dmModeCR		(8-x)

candModeListC[ 1 ] = 50		(8-x)

- Otherwise (dmModeCR is not equal to dmModeTL), candModeListC[ x ] with x = 0, 1 is derived as follows:

candModeListC[ 0 ] = dmModeTL		(8-x)

  - If dmModeCR && dmModeTL is equal to 1, the following applies:

candModeListC[ 1 ] = 0		(8-x)

  - Otherwise (dmModeCR && dmModeTL is equal to 0), the following applies:

  - If dmModeCR + dmModeTL is greater than 1, the following applies:

candModeListC[ 1 ] = 1		(8-x)

  - Otherwise (dmModeCR + dmModeTL is not greater than 1), the following applies:

candModeListC[ 1 ] = 50		(8-x)

- If sps_cclm_enabled_flag is equal to 1, candModeListC[ x ] with x = 2..4 is derived as follows:

candModeListC[ 2 ] = 81		(8-x)

candModeListC[ 3 ] = 82		(8-x)

candModeListC[ 4 ] = 83		(8-x)

- IntraPredModeC[ xCb ][ yCb ] is set equal to  candModeListC[ intra_chroma_pred_mode[ xCb ][ yCb ]].

The chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived using intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] as specified in Table 8-2 and Table 8-3.

Table 8-2 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] when sps_cclm_enabled_flag is equal to 0

| intra_chroma_pred_mode[ xCb ][ yCb ] | IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 0 | 50 | 18 | 1 | X |

Table 8-3 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] when sps_cclm_enabled_flag is equal to 1

| intra_chroma_pred_mode[ xCb ][ yCb ] | IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 81 | 81 | 81 | 81 | 81 |
| 5 | 82 | 82 | 82 | 82 | 82 |
| 6 | 83 | 83 | 83 | 83 | 83 |
| 7 | 0 | 50 | 18 | 1 | X |

(section 9.5.3.7 in VVC specification)

Input to this process is a request for a binarization for the syntax element intra_chroma_pred_mode.

Output of this process is the binarization of the syntax element.

The binarization for the syntax element intra_chroma_pred_mode is specified in Table 9-7 and Table 9-8.

Table 9-7 – Binarization for intra_chroma_pred_mode when sps_cclm_enabled_flag is equal to 0

| Value of intra_chroma_pred_mode | Bin string |
| --- | --- |
| 2 | 0 |
| 0 | 10 |
| 1 | 11 |

Table 9-8 – Binarization for intra_chroma_pred_mode when sps_cclm_enabled_flag is equal to 1

| Value of intra_chroma_pred_mode | Bin string |
| --- | --- |
| 5 | 0 |
| 2 | 10 |
| 3 | 1110 |
| 4 | 1111 |
| 0 | 1100 |
| 1 | 1101 |

Table 9-7 – Binarization for intra_chroma_pred_mode when sps_cclm_enabled_flag is equal to 0

| Value of intra_chroma_pred_mode | Bin string |
| --- | --- |
| 4 | 0 |
| 0 | 100 |
| 1 | 101 |
| 2 | 110 |
| 3 | 111 |

Table 9-8 – Binarization for intra_chroma_pred_mode when sps_cclm_enabled_flag is equal to 1

| Value of intra_chroma_pred_mode | Bin string |
| --- | --- |
| 7 | 0 |
| 4 | 10 |
| 5 | 1110 |
| 6 | 1111 |
| 0 | 11000 |
| 1 | 11001 |
| 2 | 11010 |
| 3 | 11011 |

(section 9.5.4.2.1 Table 9-10 in VVC specification)

| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 0 | 0 | bypass | bypassna | na | na | na |
| --- | --- | --- | --- | --- | --- | --- |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 0 | 0 | 1 | 2 | bypass | bypassna | na |