| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0219-v2 |
| --- | --- |

| Title: | CE3-related : Reduced number of reference samples for CCLM parameter calculation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Choi, Jin Heo, Sunmi Yoo, Ling Li, Jungah Choi, Jaehyun Lim, Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 { jangwon84.choi, jin78.heo, sunmi.yoo, l.li, jungah.choi, jaehyun.lim, seunghwan3.kim }@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

The CCLM (cross-component linear model) method in the current VTM3 requires complex process to derive the linear model parameter α and β based on the straight line equation with reference pixel comparison step. Considering that the derivation process of the linear model parameters is also required in decoder side, it is necessary to reduce the required complexity. In this proposal, the number of reference sample pairs, which are used in the CCLM parameter determination is reduced to half based on pre-defined upper limit so that the worst pixel comparison complexity is reduced by half (i.e., from 4 samples to 2 samples).

Experimental results with 8 upper limit reference sample pairs show -0.01%, -0.13%, and -0.17% BD-rates on Y, Cb, and Cr components, respectively in All Intra configuration, and 0.00%, -0.08%, and -0.07% in Random Access configuration.

Proposed method

As similar to JVET-L0138 [1], the half number of reference samples in the determination of the CCLM parameters is used for CCLM and MDLM (multi-directional CCLM) so that the required computational complexity can be reduced significantly. In this method, the maximum number of reference sample pairs is also limited to 4 or 8, as shown in Table 1.

With the proposed method, the number of sample pairs for CCLM parameter calculation is reduced by half for worst case (i.e. 2x2 block) and up to by 1/8 or 1/16 (i.e. 32x32) for average case, and it brings the reduced number of pixel comparison operation in CCLM parameter derivation, as shown in Table 2..

Table 1 Number of sample pairs for CCLM parameter calculation in VTM3 and proposed method

| Chroma block size | VTM3 | Proposed method with 8 upper limit reference sample pairs | Proposed method with 4 upper limit reference sample pairs |
| --- | --- | --- | --- |
| 2 x n / n x 2 | 4 | 2 | 2 |
| 4 x n / n x 4 (n >=4) | 8 | 4 | 4 |
| 8 x n / n x 8 (n >=8) | 16 | 8 | 4 |
| 16 x n / n x 16 (n >=16) | 32 | 8 | 4 |
| 32 x 32 | 64 | 8 | 4 |

Table 2 Number of pixel comparison operation for CCLM parameter calculation in VTM3 and proposed method

| Chroma block size | VTM3 | Proposed method with 8 upper limit reference sample pairs | Proposed method with 4 upper limit reference sample pairs |
| --- | --- | --- | --- |
| 2 x n / n x 2 | 8 | 4 | 4 |
| 4 x n / n x 4 (n >=4) | 16 | 8 | 8 |
| 8 x n / n x 8 (n >=8) | 32 | 16 | 8 |
| 16 x n / n x 16 (n >=16) | 64 | 16 | 8 |
| 32 x 32 | 128 | 16 | 8 |

In this proposal, the subsampling pixels for the CCLM parameter calculation can be selected in the same way as introduced in [1].

Experimental results

The proposed method has been implemented using VTM3.0 as software base according to the common test conditions defined in JVET-L1010 [2] and Core Experiment description in JVET-L1023 [3].

Test 1: Proposed method with 8 upper limit reference sample pairs

In this test, the half number of reference samples in the determination of the CCLM parameters is used for CCLM and MDLM, and the maximum number of reference sample pairs is limited to 8. Table 3 and Table 4 show the test results in AI and RA configuration.

Table 3 Experimental results of Test1 for all-intra (AI) test condition; anchor is VTM3

|  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-3.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.34% | -0.33% | 100% | 100% |
| Class A2 | 0.02% | -0.05% | 0.07% | 100% | 100% |
| Class B | 0.00% | -0.19% | -0.36% | 100% | 100% |
| Class C | -0.04% | -0.03% | -0.17% | 100% | 100% |
| Class E | -0.01% | -0.06% | 0.06% | 100% | 101% |
| Overall | -0.01% | -0.13% | -0.17% | 100% | 100% |
| Class D | 0.01% | 0.14% | 0.13% | 101% | 100% |
| Class F | 0.02% | 0.06% | -0.03% | 101% | 100% |

Table 4 Experimental results of Test1 for random-access (RA) test condition; anchor is VTM3

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-3.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.31% | -0.23% | 100% | 99% |
| Class A2 | 0.00% | 0.14% | 0.17% | 100% | 100% |
| Class B | -0.01% | -0.05% | -0.31% | 100% | 100% |
| Class C | 0.01% | -0.11% | 0.16% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.08% | -0.07% | 100% | 100% |
| Class D | 0.00% | -0.29% | 0.10% | 100% | 101% |
| Class F | 0.02% | -0.04% | 0.04% | 100% | 101% |

Test 2: Proposed method with 4 upper limit reference sample pairs

In this test, the half number of reference samples in the determination of the CCLM parameters is used for CCLM and MDLM, and the maximum number of reference sample pairs is limited to 4. Table 5 and Table 6 show the test results in AI and RA configuration.

Table 5 Experimental results of Test1 for all-intra (AI) test condition; anchor is VTM3

|  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-3.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.05% | -0.08% | 100% | 100% |
| Class A2 | 0.08% | 0.39% | 0.25% | 100% | 100% |
| Class B | 0.03% | -0.13% | -0.06% | 100% | 100% |
| Class C | 0.01% | 0.16% | 0.04% | 100% | 100% |
| Class E | -0.02% | 0.09% | 0.00% | 100% | 101% |
| Overall | 0.02% | 0.09% | 0.02% | 100% | 100% |
| Class D | 0.00% | 0.13% | 0.22% | 101% | 100% |
| Class F | 0.06% | 0.26% | 0.06% | 101% | 100% |

Table 6 Experimental results of Test1 for random-access (RA) test condition; anchor is VTM3

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-3.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.03% | 0.05% | 100% | 99% |
| Class A2 | 0.01% | 0.31% | 0.26% | 100% | 100% |
| Class B | 0.02% | -0.01% | -0.12% | 100% | 100% |
| Class C | 0.04% | 0.08% | 0.29% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.08% | 0.10% | 100% | 100% |
| Class D | 0.02% | -0.06% | -0.07% | 100% | 101% |
| Class F | 0.06% | 0.27% | 0.11% | 100% | 101% |

Conclusions

To reduce the complexity of CCLM parameter derivation, the limitation of reference sample number for CCLM parameter calculation is proposed. This method can reduce the number of operations in the CCLM prediction without coding performance changes while the computation complexity is reduced significantly. It is recommended to adopt the proposed method to be included in the next VTM.

Reference

- J. Choi, et al., “CE3-related : Reduced number of reference samples for CCLM parameter calculation” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JVET-L0139, 12th Meeting, Macao, CN, 3-12 Oct. 2018.

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L1010, 12th Meeting, Macao, CN, 3-12 Oct. 2018.

- G. Van der Auwera, et al., “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L1023, 12th Meeting, Macao, CN, 3-12 Oct. 2018.

Patent rights declaration

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).