| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0221 |
| --- | --- |

| Title: | CE4.2.3: STMVP simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Yu Han, Dmytro Rusanovskyy, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

# Abstract

This contribution proposes a simplification of the non-sub-PU based spatial-temporal merge mode (STMVP), described in JVET-L0399 in the JVET meeting in Macao (3-12 October, 2018). The simplification shows negligible losses in coding gain over VTM3.0. It is reported that the BD-rate gain is 0.00 % and 0.11 % in RA and LB condition respectively.

# Introduction

A non-sub-PU STMVP technique was proposed in JVET-L0399. The STMVP motion vector is derived by taking the average of up to three motion vectors, which consists of two spatial and one temporal motion vector predictions. In this proposal, we propose a simplified STMVP, where motion scaling and STMVP candidates with non-local spatial neighbors are removed, and the average operation is simplified. Besides, for each spatial local neighbor (above or left), instead of searching up to two positions, only one position is searched. The modification reduces the complexity with negligible changes in coding efficiency.

# Proposed STMVP simplification

In this proposal, four modifications are proposed to simplify the complexity in the non-sub-PU STMVP in JVET_L0399, as described as follows:

- No MV scaling for the two spatial neighbors.

- STMVP candidates with non-local spatial neighbors are removed. As a result, only one STMVP is added into merge candidate list.

- For STMVP with local spatial neighbors, instead of searching up to two positions for above (left) spatial neighbors, only one position is checked, as shown in Figure 1. The positions are the same as the 1st and 2nd checked candidates in the non-sub-block merge candidate list.

Figure 1: spatial position checked (S0 and S1) to construct the STMVP candidate

Current PU

S0

S1

- If three candidates are available, i.e., the temporal candidate (MVT) on co-located block is available and the two spatial candidates are available with reference index equals to zero, the STMVP candidate is calculated as

MVSTMVP=MVS0+MVS14+MVT2

Note that the denominators in divisions are always of power of two, and therefore can be implemented using shift operations.

# Simulation results

The introduced STMVP candidate is added in the merge list right before above left spatial motion candidate. In our test, the pairwise averaged motion vector candidates are deactivated.  The performance is reported in Table 1.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.19% | -0.05% | #NUM! | #NUM! |
| Class A2 | 0.02% | 0.00% | -0.03% | #NUM! | #NUM! |
| Class B | 0.06% | 0.09% | -0.04% | #NUM! | #NUM! |
| Class C | -0.03% | 0.04% | 0.08% | #NUM! | #NUM! |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.01% | #NUM! | #NUM! |
| Class D | -0.22% | -0.13% | -0.06% | #NUM! | #NUM! |
| Class F (optional) | 0.08% | 0.05% | 0.07% | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.19% | -0.04% | 0.26% | #NUM! | #NUM! |
| Class C | 0.08% | 0.28% | 0.40% | #NUM! | #NUM! |
| Class E | 0.01% | 0.43% | 0.19% | #NUM! | #NUM! |
| Overall | 0.11% | 0.18% | 0.29% | #NUM! | #NUM! |
| Class D | -0.06% | -0.06% | 0.45% | #NUM! | #NUM! |
| Class F (optional) | 0.25% | 0.27% | 0.31% | #NUM! | #NUM! |

Table 1: performance of test 4.2.3.a over VTM-3.0

# Reference

- F. Bossen, J.Boyce, X. Li, V. Seregin and K. Suehring,  “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th meeting, Macao, CN, October 2018.

- Y. Han, W.-J Chien, H. Huang, M. Karczewicz, “CE4.4.6: Improvement on Merge/Skip mode”, JVET-L0399, 12th meeting, Macao, CN, October, 2018.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).