| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0224-v2 |
| --- | --- |

| Title: | CE10-related: Local illumination compensation simplifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Saurav Bandyopadhyay Xiaoyu Xiu Yuwen He | Email: | saurav.bandyopadhyay@interdigital.com yuwen.he@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

This document proposes simplifications to Local Illumination Compensation (LIC) based on CE 10.5.2. LIC is used to compensate for the illumination changes at the motion compensation stage that exists in-between the temporal reference block and the current coding block based on a linear model. Traditionally, the LIC parameters e.g. scaling factor and the offset are separately estimated for each prediction direction when bi-prediction is applied. In the simplified method, the bi-prediction reference samples of the template samples are first generated using bi-predictive weighted average (BPWA) based on the motion vectors of the current block. Then the LIC parameters are derived by minimizing the difference between the template reference samples and the template samples. In additional to disabling LIC for affine merge & triangle uni-prediction modes, which is the same as that in CE 10.5.2, the proposed simplifications further disable LIC for ATMVP sub-block merge mode.

The simplification results in PSNR-Y, U, V BD-rate variations of {-0.63%, -0.35%, -0.45%} gain in RA configuration over VTM3.0 and {-0.07%, -0.04%, -0.09%} gain over CE10.5.2 in RA configuration.

# Introduction

Local illumination compensation (LIC) is a coding tool which is used in the CE10.5.2 [1]   to address the issue of local illumination changes that exist in temporal neighboring pictures. The LIC is based on a linear model where a scaling factor and an offset are applied to the reference samples to obtain the prediction samples of a current block. Specifically, the LIC can be mathematically modeled by the following equation:

| Px,y= α∙Prx+vx,y+vy+β | (1) |
| --- | --- |

where Px,y is the prediction signal of the current block at the coordinate x,y; Prx+vx,y+vy is the reference block pointed by the motion vector vx,vy; α and β are the corresponding scaling factor and offset that are applied to the reference block. Figure 1 illustrate the LIC process. In Figure 1, when the LIC is applied for a video block, a least mean square error (LMSE) method is employed to derive the values of the LIC parameters (i.e., α and β) by minimizing the difference between the neighboring samples of the current block (i.e., the template T in Figure 3) and their corresponding reference samples in the temporal reference pictures (i.e., either T0 or T1 in Figure 1). Additionally, to reduce the computational complexity, both the template samples and the reference template samples are subsampled (adaptive subsampling) to derive the LIC parameters, i.e., only the shaded samples in Figure 1 are used to derive α and β.

Figure 1 Local illumination compensation

Moreover, when LIC is applied to bi-directional blocks (i.e., being predicted by two temporal prediction blocks), the LIC parameters are derived and applied for each prediction direction, i.e., L0 and L1, separately. As shown in Figure 1, based on the two motion vectors MV0 and MV1, two reference templates T0 and T1 can be obtained; by separately minimizing the distortions between T0 and T, and T1 and T, the corresponding pairs of LIC parameters in two directions can be derived. Afterwards, the final bi-directional prediction signal of the current block is generated by combining two LIC uni-prediction blocks, as indicated as

| Px,y= 1-w×α0∙Pr0x+vx0,y+vy0+β0+w×α1∙Pr1x+vx1,y+vy1+β1 | (2) |
| --- | --- |

where Px,y is the final prediction signal of the current block/sub-block; Pr0x+vx0,y+vy0 and Pr1x+vx1,y+vy1 are the two references of the current block/sub-block in L0 and L1, respectively, , α0 and β0 and α1 and β1 are the LIC parameters associated with the L0 and L1 motion vectors (i.e., vx0,vy0 and vx1,vy1) of the current block, (1-w) and w denote the BPWA weights applied to L0 and L1 reference samples.

# Proposed LIC simplifications

## Simplified LIC process for bi-prediction CUs

When bi-predictive weighted average (BPWA) is enabled, a weight index is signaled for explicit bi-prediction CUs. The traditional approach performs BPWA on top of LIC. Thus, first local illumination compensation is applied to the individual reference lists L0 and L1 for each of the two prediction blocks. Next, BPWA provided for each of the two illumination compensated prediction blocks for bi-prediction are used to combine the prediction templates.

Our proposed approach, as in Figure 2, firstly performs the generalized bi-prediction of the template samples from two prediction lists to produce BPWA based bi-prediction template reference samples. The LIC parameters are estimated using the BPWA based bi-prediction template reference samples and the template samples of the current CU. Thus, the proposed method is different in the sense that it performs BPWA first followed by LIC on bi-directional prediction signal of the current block/sub-block. It is mathematically shown below in (3). In the proposed LIC method, the bi-directional prediction signal of the template is firstly generated by weighted averaging two reference samples of the template in L0 and L1, i.e.,

| - Twx,y=1-w×T0x+vx0,y+vy0+w×T1x+vx1,y+vy1 | (3) |
| --- | --- |

where,  Twx,y denotes the bi-directional prediction signal of the template. T0x+vx0,y+vy0 and T1x+vx1,y+vy1 , are the L0 and L1 reference samples of the template as indicated by the bi-directional motion vectors vx0,vy0 and vx1,vy1 of the current block / sub-block respectively, (1-w) and w denote the BPWA weights applied to L0 and L1 reference samples. w=0.5 is the special case whereby, the LIC prediction signals are averaged to generate the final prediction signal of the current block/sub-block.

Figure 2 The proposed LIC method for bi-directional prediction

After that, the linear model parameter derivation, such as LMSE-based method, is employed to calculate the values of the scaling factor and the offset used for the LIC by minimizing the difference between the template samples and their bi-directional reference samples.

Finally, the derived LIC parameters are applied to the bi-directional prediction signal of the current block/sub-block based on the linear model, i.e.,

| Px,y= α∙ 1-w×Pr0x+vx0,y+vy0+w×Pr1x+vx1,y+vy1 +β | (4) |
| --- | --- |

where Px,y is the final prediction signal of the current block/sub-block; Pr0x+vx0,y+vy0 and Pr1x+vx1,y+vy1 are the two references of the current block/sub-block in L0 and L1, respectively, α and β are the local illumination compensation slope and offset parameters. Therefore, (4) applies LIC once (i.e., one LIC parameter estimation and one LIC-based sample adjustment) on bi-directional prediction signal of the current block/sub-block.

## Disabling LIC for ATMVP

It is proposed to disable LIC for the sub-block coding modes e.g. ATMVP. For the blocks coded by any of the sub-block modes, one block is further divided into multiple sub-blocks, each with unique motion vector in order to generate fine granularity motion field. Disabling LIC operation for the ATMVP sub-block mode will reduce the computational complexity.

# Experimental Results

The following experiments were conducted using the VTM-3.0 reference software and the JVET common test conditions [2] to collect the B-D rate, encoding and decoding time of the proposed techniques in the Random Access and Low Delay B configurations.

The results of the proposed implementation are shown against the VTM3.0 and the CE10.5.2 as anchor. The proposed simplification in Sections 2.1 & 2.2 is used as test. LIC for ATMVP sub-block mode is disabled for test as in Section 2.2. The experimental results show negligible impact due to the proposed simplification.

## Comparison with VTM-3.0 anchor

Table 1. Random Access

|  | Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.12% | -0.67% | -1.03% | 146% | 99% |
| Class A2 | -0.41% | -0.18% | -0.28% | 124% | 101% |
| Class B | -0.65% | -0.23% | -0.32% | 130% | 99% |
| Class C | -0.39% | -0.40% | -0.31% | 137% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.63% | -0.35% | -0.45% | 134% | 100% |
| Class D | -0.08% | -0.14% | -0.03% | 133% | 102% |
| Class F (optional) | -1.08% | -0.76% | -0.71% | 130% | 101% |

Low Delay B

|  | Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.83% | -0.83% | -0.89% | 144% | 98% |
| Class C | -0.56% | -0.56% | -0.74% | 157% | 99% |
| Class E | -0.07% | -0.21% | -0.03% | 105% | 101% |
| Overall | -0.55% | -0.58% | -0.62% | 137% | 99% |
| Class D | -0.19% | -0.37% | -0.50% | 147% | 103% |
| Class F (optional) | -2.93% | -1.70% | -2.20% | 146% | 99% |

## Comparison with CE10.5.2 test

Table 2. Random Access

|  | Over CE-10.5.2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.07% | -0.13% | 99% | 99% |
| Class A2 | -0.03% | -0.04% | -0.06% | 99% | 100% |
| Class B | -0.05% | -0.01% | -0.07% | 100% | 100% |
| Class C | -0.04% | -0.06% | -0.08% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.04% | -0.09% | 99% | 100% |
| Class D | -0.03% | -0.10% | 0.11% | 100% | 100% |
| Class F (optional) | -1.08% | -0.76% | -0.71% | 130% | 101% |

Low Delay B

|  | Over CE-10.5.2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | 0.02% | -0.09% | 100% | 97% |
| Class C | -0.07% | -0.34% | -0.21% | 100% | 97% |
| Class E | 0.01% | -0.16% | 0.24% | 100% | 98% |
| Overall | -0.04% | -0.15% | -0.04% | 100% | 97% |
| Class D | -0.08% | 0.07% | -0.10% | 100% | 101% |
| Class F (optional) | -2.93% | -1.70% | -2.20% | 146% | 99% |

# Conclusion

The proposed LIC simplifications reduce the complexity with a marginal variation on the LIC performance. One simplified LIC method is proposed to reduce the complexity of deriving the LIC parameters for the bi-predicted blocks. Unlike the existing LIC method which estimates the LIC parameters for each prediction direction separately, the proposed LIC method generates the bi-prediction reference samples of the template samples for the current block and estimate the LIC parameters only once. By this way, only one single LIC operation (i.e., one LIC parameter estimation and one LIC-based sample adjustment) needs to be performed for generating the prediction signal of the current block. The LIC for ATMVP sub-block mode is proposed to be disabled.

# Reference

K. Abe, T. Toma and J. Li, “CE-4 related: Low pipeline latency LIC”, JVET-L0120, Macao, CN, October 2018.

F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations”, JVET-L1010, Macao, CN, October 2018.

# Patent rights declaration(s)

InterDigital Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).