| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0225 |
| --- | --- |

| Title: | AHG8: On wrap around motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Weiwei Feng Shan Liu | Tel: Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes the following changes on the wrap-around motion compensation:

1. A general_non_packed_constraint_flag is proposed in the Profile syntax structure. The wrap around motion compensation is disabled when general_non_packed_constraint_flag is equal to 1.

2. The value of sps_ref_wraparound_offset is restricted. sps_ref_wraparound_offset shall be greater than pic_width_in_luma_samples / 2 - 1, and shall not be greater than pic_width_in_luma_samples.

# Problem statement

The horizontal wrap-around motion compensation (JVET-L0231) was adopted at the Macau meeting. The wrap-around motion compensation can reduce some visual artifact at the seam line in which the left boundary and the right one meet for 360 rendering. It has a marginal BD-rate saving (-0.22%), only when the padded equirectangular projection (PERP) is used. There is no report to provide a positive RD gain with normal video sequences. If the wrap-around motion compensation is allowed for normal video sequences without a restriction, it is asserted that undesired visual artifacts might appear at boundary by copying and using unconnected reverse-side pixel data for motion compensation.

Another problem is that there is no appropriate range of sps_ref_wraparound_offset in the current semantics. The allowed value if greater than 0 and smaller than a picture width  (pic_width_in_luma_samples) in the current VVC WD. If the value of sps_ref_wraparound_offset is not greater than pic_width_in_luma_samples/2 - 1, the same pixel values are circularly repeated for the wrap-around motion compensation. It is asserted that the circularly repeated motion compensation is not intended by the original proposal.

# Proposal

In order to resolve the above problems, the following changes are proposed:

- The constraint flag for non-packed content (general_non_packed_constraint_flag) is proposed. The wrap around motion compensation is disabled when general_non_packed_constraint_flag is equal to 1. Hence, sps_ref_wraparound_enabled_flag shall be equal to 0, when general_non_packed_constraint_flag is equal to 1. The constraint flag has similar semantics as in HEVC. The proposed semantics are same as proposed in JVET-M0101.

- The value of sps_ref_wraparound_offset is restricted. sps_ref_wraparound_offset shall be greater than pic_width_in_luma_samples / 2 - 1, and shall not be greater than pic_width_in_luma_samples. If the value of sps_ref_wraparound_offset is smaller than pic_width_in_luma_samples / 2, the motion-compensated pixel data is circularly repeated.

# Proposed Spec Text.

| profile_level( maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| … |  |
| general_non_packed_constraint_flag | u(1) |
| … |  |
| } |  |

general_non_packed_constraint_flag equal to 1 indicates that the cropped output pictures of the decoded bitstream are suitable to be displayed. general_non_packed_constraint_flag equal to 0 indicates that the cropped output pictures of the decoded bitstream may require further processing to be suitable for displaying.

NOTE 2 – Examples for content that requires additional processing before displaying are bitstreams containing one or more frame packing arrangement SEI messages, equirectangular projection SEI messages, or cubemap projection SEI messages.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| profile_level( maxNumSubLayersMinus1 ) |  |
| … |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset | ue(v) |
| ... |  |
| } |  |

sps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction. sps_ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is not applied. The value of sps_ref_wraparound_enabled_flag shall be equal to 0, when general_non_packed_constraint_flag is equal to 1.

sps_ref_wraparound_offset specifies the offset used for computing the horizontal wrap-around position in luma samples. sps_ref_wraparound_offset shall be greater than pic_width_in_luma_samples / 2 -1, shall not be greater than pic_width_in_luma_samples, and shall be an integer multiple of MinCbSizeY.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).