| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0227 |
| --- | --- |

| Title: | CE2: A second ATMVP candidate (test 2.5.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen Xianglin Wang | Email: | yiwenchen@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

Reported in this document is the results of CE2.5.2 on new ATMVP-like merge candidate. The derivation process of the new ATMVP is similar to the original ATMVP, with slightly different rules used in selecting motion vectors (MV) from corresponding reference sub-blocks. More specifically, the MV selection rules for TMVP from a reference block are used in deriving MVs for each subblock under the new ATMVP mode. Simulation results of CE2.5.2.c show that the new ATMVP mode provides -0.04% BD-rate saving under RA with negligible encoding/decoding run time difference. Simulation results of CE2.5.2.g show that replacing the original ATMVP mode with the new ATMVP mode introduces 0.26% BD-rate loss under RA.

# Introduction

In the original ATMVP, for each subblock within a CU, its motion vectors are derived from corresponding MVs in a collocated picture. Firstly based on a temporal vector identified, a corresponding reference subblock in the collocated picture is located, as shown in Figure 1. Then its motion vectors are derived based on procedures listed in Table 1 for selecting its reference MV, followed by appropriate MV scaling if needed.

Figure 1: ATMVP motion prediction for a CU

Table 1: Reference MV selection for ATMVP

If the current picture has no backward prediction,

If the L[X] MV of the reference subblock is available,

L[X] MV of the reference subblock is selected.

Otherwise,

L[1-X] MV of the reference subblock is selected.

Otherwise,

L[X] MV of the reference subblock is selected if available.

In the table, L[X] (with X being 0 or 1) denotes reference list X. According to Table 1, in case that the current picture has backward prediction, the derived MVs for a subblock can be either uni-directional or bi-directional.

# Proposed Implementation

In proposal JVET-L0105[1], we proposed to generate and insert a new ATMVP-like merge candidate into subblock merge candidate list, in addition to the original ATMVP. The derivation process of the new ATMVP is similar to the original ATMVP, except that slightly different rules are used in selecting the reference MVs.

As illustrated in Table 2, the reference MV selection for the proposed ATMVP is the same as that used for the conventional TMVP. In Table 2, listCol denotes the collocated picture list index.

It is worth noting that when a current picture has no backward prediction, the MV selection process of the new ATMVP and the original ATMVP are identical. Therefore, the proposed ATMVP is only derived and inserted into the merge list when the current picture has backward prediction.

Table 2: Reference MV selection for the new ATMVP

If current picture has no backward prediction,

If the L[X] MV of the reference block is available,

L[X] MV of the reference block is selected.

Otherwise,

L[1-X] MV of the reference block is selected.

Otherwise,

If the L[1- listCol] MV of the reference block is available,

L[1- listCol] MV of the reference block is selected.

Otherwise,

L[listCol] MV of the reference block is selected.

In CE tests CE2.5.2.c, the new ATMVP is inserted into the subblock merge candidate list in additional to the original ATMVP and redundancy checking is performed between the two ATMVP candidates. In CE tests CE2.5.2.g, the new AMTVP is inserted into the subblock merge candidate list to replace the original ATMVP.

# Results

The results of the proposed implementation are shown below against an anchor of VTM3.0[2] under CTC [3].

Table 3. Performance of CE2.5.2.c under Random Access

Table 4. Performance of CE2.5.2.g under Random Access

# Reference

- “ CE4-related: A second ATMVP candidate”, Y.-W. Chen, X. Wang, doc. JVET-L0105, October 2018.

- VTM-3.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-L1010, October 2018.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).