| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0228 |
| --- | --- |

| Title: | CE2-related: Affine mode modifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen Xianglin Wang | Email: | yiwenchen@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this proposal, two modifications are proposed to affine motion mode. Firstly, it is proposed when generating constructed affine merge candidates, the comparisons of the motion vectors between control points are removed. It is asserted that with this modification, up to 20 motion vector comparisons are removed for an affine merge coded block. Secondly, we propose to use the affine motion model to derive the motion vectors for each 4x4 chroma subblock instead of taking average of the corresponding luma motion vectors for design consistency.

# Introduction

In the current VVC, constructed affine merge candidates are derived as affine merge candidates for an affine merge block. They are constructed by combining the neighbor translational motion information of each control point. As shown in Figure 1, CPMVk (k=1, 2, 3, 4) represents the k-th control point. After MVs of the four control points are obtained, affine merge candidates are constructed based on the following combinations and order:

{CPMV1, CPMV2, CPMV3}, {CPMV1, CPMV2, CPMV4}, {CPMV1, CPMV3, CPMV4},
{CPMV2, CPMV3, CPMV4}, {CPMV1, CPMV2}, {CPMV1, CPMV3}

The combinations of three CPMVs are used to construct 6-parameter affine merge candidates and the combinations of two CPMVs 4-parameter affine merge candidates. To avoid constructing a translational motion field, when two CPMVs are identical under 4-parameters affine model (or three CPMVs identical under 6-parameters affine model), the associated combination of control point MVs is discarded. Therefore, during the derivation of the constructed affine merge candidates, MV comparisons are needed and there are 20 of them between control points.

Figure 1 –Control points for constructed affine merge mode

Currently, 4x4 sub-block is used for affine motion compensation, for both luma and chroma. For each luma sub-block, the motion vector of the center sample of the sub-block is calculated according to the affine motion model. For each chroma sub-block, the motion vector is calculated as the average of the MVs of the four corresponding 4×4 luma sub-blocks.

# Proposed Implementation

Two modifications are proposed in this contribution to affine motion mode.

Firstly, we propose to remove all the 20 MVs comparisons between control points for generating constructed affine merge candidates.

Secondly, we propose to derive the affine motion vectors for each 4x4 chroma sub-block using the affine motion model directly, instead of taking average of the MVs for the four corresponding luma sub-blocks. With the modification, both luma and chroma subblocks use the same function to derive their affine motion vectors. The proposed modification removes the necessity of luma motion buffer, and 32 lines of codes in VTM-3.0 without adding any into it.

# Results

The proposed modifications are implemented on top of VTM-3.0[1]. The results of the proposal under CTC [2] are shown below against VTM3.0 anchor.

Table 1. Performance of proposed MV comparison removal under Random Access

Table 2. Performance of proposed MV comparison removal under Low delay B

Table 3. Performance of proposed 4x4 chroma MV derivation under Random Access

Table 4. Performance of proposed 4x4 chroma MV derivation under Low delay B

# Conclusion

In this proposal, two modifications are proposed to the affine motion model. The first modification is to remove the 20 MV comparisons between control points when deriving the constructed affine merge candidates for a CU. The second modification is to derive the 4x4 chroma block using the affine motion model directly instead of taking average of the MVs from corresponding luma blocks. Both modifications introduce negligible coding performance impact.

# Reference

- VTM-3.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-L1010, October 2018.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).