| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0229 |
| --- | --- |

| Title: | CE3-related: Simplification of LM Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen Xianglin Wang | Email: | yiwenchen@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, a modification to the cross-component linear model (CCLM) deriving method is presented. To determine the α and β for the CCLM model, currently in VVC the minimum and maximum luma samples are searched among the down-sampled top and left neighbouring reconstructed luma samples. α and β are then determined by the linear model formed from the minimum/maximum down-sampled luma samples and the corresponding chroma samples. In this proposal, it is proposed to search the minimum and maximum luma samples among the top and left neighbouring reconstructed luma samples without the down-sampling process. After the minimum and maximum luma samples are identified, their corresponding down-sampled luma samples are then generated to derive the α and β. With the proposed modification, the number of the operations involved during the derivation of α and β are reduced by around 97% for a 32x32 chroma block. Simulation results reportedly show BD-rates on Y, Cb and Cr components are 0.03%, 0.14%, and 0.19% for AI configuration over VTM-3.0, with 100% encoding time and 98% decoding time.

# Introduction

In the current CCLM scheme, the minimum (min) and maximum (Max) luma values are searched among the neighboring down-sampled luma samples as depicted in the right part of Figure 1. After the min and Max luma samples are determined, the linear model parameters α and β are obtained according to the values of the min/Max luma samples(LumaMax,Lumamin) and the corresponding chorma samples (ChromaMax,Chromamin) using the following equation:

α=ChromaMax-ChromaminLumaMax-Lumamin, β=Chromamin-αLumamin

Figure 1: Down-sampled reconstructed luma samples (Rec’L) and the reconstructed chorma samples (RecC) used to derive α and β in VTM-3.0.

# Proposed Implementation

In current CCLM, a six-tap filter is applied to derive down-sampled luma samples. To reduce the computational complexity, we propose to use the corresponding reconstructed luma samples directly (without down-sample filtering) to find the min/Max luma samples as shown in the right part of Figure 2. After the min/Max luma samples are identified, their corresponding down-sampled luma samples are then generated to derive the α and β.

Figure 2: Reconstructed luma samples (RecL) and the reconstructed chorma samples (RecC) that are used directly to derive α and β in the proposed scheme.

In this case, the down-sampling operations are greatly reduced to only two, with one for the minimum luma sample and one for the maximum luma sample. The number of operations for α and β derivation in the VTM-3.0 and the proposed schemes are illustrated in Table 1. As shown in this table, the numbers of “multiplications”, “sums” and “shift” are reduced from “2N+1” to 5, “6N+3” to 15 and “N” to 2, respectively. It is noted that N is at least 4 (2x2 chroma block) and the proposed modification could reduce the number of operations by around 97% for a 32x32 chroma block.

Table 1 The comparisons of number of operations in deriving α and β (N >=4 in VTM-3.0)

| Operations | Number of operations for α and β derivation |  |
| --- | --- | --- |
|  | VTM-3.0 | Proposed |
| Multiplications | 2N+1 | 4+1 |
| Sums | 6N+3 | 12+3 |
| Shift | N | 2 |
| “Division” | 1 | 1 |
| Comparison | 2N | 2N |

# Results

The proposed modification is integrated into VTM-3.0 [1] and the results of the proposed implementation under CTC [2] are shown below against VTM3.0.

Table 1. Performance of the proposed modification under All Intra

Table 1. Performance of the proposed modification under Random Access

Table 1. Performance of the proposed modification under Low Delay B

# Conclusion

In this proposal, a simplification on CCLM mode is proposed to reduce computational complexity. The modification provides around 97% reduction of down-sample filtering operations needed for a 32x32 chroma block, with 0.03%, 0.14% and 0.19 BD-rate increase for Y, Cb and Cr components respectively. It is recommended to consider adoption of the proposed simplification into VVC for CCLM mode.

# Reference

- VTM-3.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “ CE4-related: A second ATMVP candidate”, Y.-W. Chen, X. Wang, doc. JVET-L0105, October 2018.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).