| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0236-v1 |
| --- | --- |

| Title: | CE1: Transform tiling for pipelined processing of CTUs (Test 1.2.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne, A. Dorrell, Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au andrew.dorrell@cisra.canon.com.au |
| Source: | Canon Information Systems Research Australia Pty Ltd. |  |  |

_____________________________

# Abstract

To support pipelined processing architectures with a 64x64 virtual pipeline data unit (VPDU) size while having a CTU size of 128x128, block structure restrictions have been adopted into VTM-2.0 and VTM-3.0. VTM-2.0 introduced a constraint to prohibit performing a ternary split of a coding tree node with a side length exceeding 64. This restriction prevents any TU from spanning a 64x64 grid boundary (in luma, with corresponding constraint in chroma). VTM-3.0 introduces additional split restrictions of JVET-L0081. This contribution proposes to extend the tiling operation of TUs, currently used for CUs larger than the maximum TU size, such that smaller TUs are used to prevent the resulting TUs from crossing the above-mentioned VPDU grid boundary, instead of imposing the ternary split restriction and restrictions of JVET-L0081. Coding results in RA are -0.34%, -0.36, and -0.27% and LDB are -0.26%, -0.27%, and -0.13% in Y, Cb, and Cr channels, respectively.

# Introduction

Implementations of HEVC, with a 64x64 CTU size, generally process data in a pipelined manner with a 32x32 processing region size, reducing on-chip memory requirement to one-quarter the amount that would be required were CTUs stored entirely on-chip. Due to the largest transform size being 32x32 and use of a quadtree split as the only split in the HEVC coding tree, TUs of HEVC are always fully contained within 32x32 regions. For VVC, with a 128x128 CTU size, implementers may wish to perform a similar optimisation, for example with a 64x64 processing region size. However, the presence of ternary splits may result in TUs spanning 64x64 processing region boundaries.

The transform is an operation that needs to be performed within a pipeline stage, due to the nature of internal dependencies within the transform. Other operations, such as intra prediction and inter prediction, may be performed in stages, i.e. a CU spanning multiple pipeline procession regions may be processed in stages, possibly with minor increases in intra-CTU line buffering to hold intermediate results for intra prediction.

VVC Draft 3 [1] includes a prohibition on ternary splits of blocks with a side length exceeding 64, discussed in review of JVET-K0556 [2]. Although this prohibition is sufficient to enable pipelined architectures, some coding loss is incurred.

# Description of the proposed method

This contribution reports the method and results of CE1 [3] Test 1.2.1. Proposed is a method to perform a TU tiling operation with a TU sized such that the resulting TUs are each fully contained within the boundaries of the VPDUs. This results in use of smaller TUs that used in VTM-2.0.1 however ternary splitting is no longer needs restriction. A common case is of a ternary split at the top of the coding tree, resulting in a centre CU of dimensions 64 and 128. In this case, the CU will be tiled with eight 32x32 TUs instead of two 64x64 TUs.

Also, the restrictions of JVET-L0081 [4] are lifted, namely:

1: For each VPDU containing one or multiple CUs, the CUs are completely contained in the VPDU.
	2: For each CU containing one or more VPDUs, the VPDUs are completely contained in the CU.

TU tiling is already performed for CUs exceeding available TU sizes, and the approach is extended to select smaller TUs where the larger TU would span the 64x64 pipeline procession region boundary.

This means that residual decoding operates at the CTU granularity, while downstream pipeline stages (inverse quant, inverse transformation etc, operate at the VPDU granularity). Coefficients of later VPDUs in a CTU need to be buffered in the entropy decoder before all coefficients of the current VPDU are obtained.

# Experimental results

Simulation results under the JVET common test conditions [5] are presented in Table 1.

Table 1. TU tiling for enabling pipeline processing results.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0_parallel_jvetl1010.tlist |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 102% | 106% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class D | 0.00% | 0.00% | 0.00% | 96% | 105% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-3.0_parallel_jvetl1010.tlist |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.52% | -0.71% | -0.50% | 132% | 98% |
| Class A2 | -0.66% | -0.59% | -0.45% | 123% | 100% |
| Class B | -0.28% | -0.26% | -0.24% | 122% | 101% |
| Class C | -0.03% | -0.05% | 0.00% | 113% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.34% | -0.36% | -0.27% | 122% | 101% |
| Class D | -0.05% | -0.08% | -0.03% | 113% | 105% |
| Class F | -0.17% | -0.22% | -0.16% | 120% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0_parallel_jvetl1010.tlist |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.20% | -0.47% | 0.03% | 118% | 97% |
| Class C | 0.00% | 0.24% | -0.12% | 106% | 107% |
| Class E | -0.70% | -0.61% | -0.40% | 123% | 99% |
| Overall | -0.26% | -0.27% | -0.13% | 115% | 101% |
| Class D | 0.00% | 0.29% | 0.21% | 103% | 96% |
| Class F | -0.35% | -0.21% | -0.36% | 110% | 104% |

# Conclusion

This input document presents a method to enable pipelined implementations of VVC, while retaining large CU sizes, in particular those spanning pipeline processing boundaries, as may be useful especially at lower bit rates. Coding results in RA are -0.34%, -0.36, and -0.27% and LDB are -0.26%, -0.27%, and -0.13% in Y, Cb, and Cr channels, respectively.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3)”, JVET-L1001, Macao, CN, October 2018.

- C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE1-related: Constraint for binary and ternary partitions”, JVET-K0556, Ljubljana, SI, July 2018.

- J. Ma, F. Le Léannec, M. W. Park, “Description of Core Experiment 1 (CE 1): Partitioning”, JVET-L1021, Macao, CN, October 2018.

- Y.-C. Su, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE1.2.1: Constraint for binary and ternary partitions”, JVET-L0081, Macao, CN, October 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, Macao, CN, October 2018.

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A. VVC Text

#### 7.3.4.7	Transform tree syntax

| transform_tree( x0, y0, tbWidth, tbHeight , treeType) { | Descriptor |
| --- | --- |
| MaxIntTrafoWidth = ( ( ( x0 & ( 1 << Log2TbGranularity) – 1 ) + tbWidth − 1 ) >> Log2TbGranularity ) ? tbWidth / 2 : MaxTbSizeY |  |
| MaxIntTrafoHeight = ( ( ( y0 & ( 1 << Log2TbGranularity) – 1 ) + tbHeight − 1 ) >> Log2TbGranularity ) ? tbHeight / 2 : MaxTbSizeY |  |
| if( tbWidth > MaxIntTrafoWidthMaxTbSizeY ¦ ¦ tbHeight > MaxIntTrafoHeightMaxTbSizeY ) { |  |
| trafoWidth = ( tbWidth > MaxIntTrafoWidthMaxTbSizeY ) ? (tbWidth / 2) : tbWidth |  |
| trafoHeight = ( tbHeight > MaxIntTrafoHeightMaxTbSizeY ) ? (tbHeight / 2) : tbHeight |  |
| transform_tree( x0, y0, trafoWidth, trafoHeight ) |  |
| if( tbWidth > MaxTbSizeY ) |  |
| transform_tree( x0 + trafoWidth, y0, trafoWidth, trafoHeight, treeType ) |  |
| if( tbHeight > MaxTbSizeY ) |  |
| transform_tree( x0, y0 + trafoHeight, trafoWidth, trafoHeight, treeType ) |  |
| if( tbWidth > MaxTbSizeY && tbHeight > MaxTbSizeY ) |  |
| transform_tree( x0 + trafoWidth, y0 + trafoHeight, trafoWidth, trafoHeight, treeType ) |  |
| } else { |  |
| transform_unit( x0, y0, tbWidth, tbHeight, treeType ) |  |
| } |  |
| } |  |

7.4.5.7	Transform tree semantics

The variable Log2TbGranularity specifies the log2 of the granularity of a grid in luma samples within which transform blocks are contained and is set equal to 6.