| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0242-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-M0191 (CE3-related: Construction of non-MPM mode list in intra prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jinho Lee Hahyun Lee Sung-Chang Lim Jungwon Kang 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr hanilee@etri.re.kr sclim@etri.re.kr jungwon@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution reports crosscheck results of non-MPM mode list construction methods in JVET-M0191. The non-MPM list is derived using neighbouring intra prediction modes (method1) or using angular modes in MPM list (method2). For both methods, number of non-MPM modes is adaptively decided for each coding block according to a threshold value. In case of the threshold value is 5, it is reported that the average luma BD-rate of the method1 and method2 for AI case is -0.13% and -0.08% compared to VTM 3.0, respectively.

- Algorithm description

In VTM3.0, 61 remaining intra prediction modes except 6 MPMs are signaled by truncated binary code. On the other hand, in JVET-M0191, the number of remaining intra prediction modes are adaptively decided by deriving non-MPM modes from neighbouring intra prediction modes (method1) or angular modes in MPM list (method2). For both methods, non-MPM modes are derived by applying offsets to one or two angular modes. The offset value is increased from one to a threshold value D (5 or 10 in JVET-M0191).

Experimental results

Experiments were performed based on VTM 3.0 software [1] under JVET common test condition [2].

From Table 1 to Table 4 show experimental results of the each proposed method compared to VTM 3.0.

T1: Method 1, D=5

T2: Method 1, D=10

T3: Method 2, D=5

T4: Method 2, D=10

Table 1. Results of T1 over VTM 3.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.23% | -0.07% | 104% | 102% |
| Class A2 | -0.10% | -0.08% | -0.12% | 104% | 102% |
| Class B | -0.11% | -0.05% | -0.14% | 105% | 103% |
| Class C | -0.13% | -0.13% | -0.21% | 104% | 104% |
| Class E | -0.17% | -0.19% | -0.07% | 105% | 103% |
| Overall | -0.13% | -0.12% | -0.13% | 104% | 103% |
| Class D | -0.14% | -0.23% | -0.05% | 104% | 105% |
| Class F | -0.25% | -0.32% | -0.24% | 105% | 105% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.01% | -0.01% | 100% | 101% |
| Class A2 | -0.07% | -0.09% | 0.10% | 100% | 101% |
| Class B | -0.06% | 0.19% | 0.03% | 100% | 100% |
| Class C | -0.04% | -0.10% | 0.05% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | 0.02% | 0.04% | 100% | 101% |
| Class D | -0.08% | -0.28% | -0.41% | 100% | 101% |
| Class F | -0.16% | 0.08% | -0.16% | 101% | 102% |

Table 2. Results of T2 over VTM 3.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.23% | -0.17% | 104% | 101% |
| Class A2 | -0.10% | -0.07% | -0.14% | 104% | 102% |
| Class B | -0.11% | -0.08% | -0.11% | 105% | 102% |
| Class C | -0.13% | -0.15% | -0.12% | 104% | 104% |
| Class E | -0.23% | -0.26% | -0.19% | 105% | 103% |
| Overall | -0.14% | -0.15% | -0.14% | 105% | 103% |
| Class D | -0.17% | -0.14% | -0.25% | 104% | 105% |
| Class F | -0.21% | -0.24% | -0.09% | 105% | 105% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.02% | 0.00% | 100% | 100% |
| Class A2 | -0.06% | -0.12% | 0.00% | 100% | 101% |
| Class B | -0.08% | 0.06% | -0.02% | 100% | 100% |
| Class C | -0.09% | -0.27% | -0.04% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.08% | -0.02% | 100% | 101% |
| Class D | -0.12% | -0.30% | -0.25% | 101% | 101% |
| Class F | -0.15% | -0.18% | 0.00% | 101% | 102% |

Table 3. Results of T3 over VTM 3.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.13% | -0.02% | 103% | 101% |
| Class A2 | -0.06% | -0.04% | -0.10% | 104% | 102% |
| Class B | -0.06% | -0.02% | -0.03% | 104% | 102% |
| Class C | -0.08% | -0.05% | -0.18% | 103% | 104% |
| Class E | -0.10% | -0.15% | -0.11% | 104% | 102% |
| Overall | -0.08% | -0.07% | -0.09% | 104% | 102% |
| Class D | -0.14% | -0.07% | 0.03% | 103% | 105% |
| Class F | -0.21% | -0.27% | -0.24% | 104% | 105% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.08% | 0.07% | 100% | 100% |
| Class A2 | -0.05% | -0.06% | 0.11% | 100% | 100% |
| Class B | -0.05% | 0.21% | -0.03% | 101% | 100% |
| Class C | -0.02% | -0.11% | 0.15% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.01% | 0.07% | 100% | 101% |
| Class D | -0.10% | -0.25% | -0.02% | 100% | 101% |
| Class F | -0.15% | -0.14% | -0.21% | 101% | 101% |

Table 4. Results of T4 over VTM 3.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.13% | -0.07% | 103% | 101% |
| Class A2 | -0.07% | -0.05% | -0.07% | 103% | 102% |
| Class B | -0.06% | -0.07% | -0.08% | 104% | 102% |
| Class C | -0.09% | -0.17% | -0.11% | 103% | 104% |
| Class E | -0.16% | -0.14% | -0.08% | 104% | 102% |
| Overall | -0.10% | -0.11% | -0.08% | 104% | 102% |
| Class D | -0.14% | -0.08% | -0.13% | 103% | 105% |
| Class F | -0.19% | -0.13% | -0.12% | 104% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.09% | -0.01% | 100% | 100% |
| Class A2 | -0.04% | 0.06% | 0.11% | 101% | 100% |
| Class B | -0.06% | 0.17% | 0.05% | 100% | 100% |
| Class C | -0.06% | 0.05% | 0.05% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | 0.06% | 0.05% | 100% | 100% |
| Class D | -0.05% | -0.43% | -0.25% | 100% | 101% |
| Class F | -0.12% | 0.00% | -0.01% | 101% | 101% |