| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0251 |
| --- | --- |

| Title: | Non-CE7: Last position coding for large block-size transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jung-Ah Choi, Jin Heo, Sunmi Yoo, Jangwon Choi, Ling Li, Jaehyun Lim, Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 {jungah.choi, jin78.heo, sunmi.yoo, jangwon84.choi, l.li, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In this proposal, the binarization of last significant coefficient position is modified to reduce the maximum number of context coded bins. Specifically, the number of context coded bins for large block (i.e., 64×64, 64×N, N×64) in worst case scenario is reduced from 11 to 9. Experimental results show 0.01%, -0.01%, and -0.02% BD-rate reductions on Y, Cb, and Cr components, respectively, with 99% decoding time compared to VTM3.0 in all-intra configuration, and 0.02%, 0.00%, and 0.06% BD-rate reductions with 100% decoding time in random access configuration.

Introduction

It is known that large block partitions typically present less residuals and the energy is more concentrated in low-frequency coefficients in the transform domain. In VTM3.0 [1], high frequency transform coefficients are zeroed out for the transform blocks with size (width or height, or both width and height) equal to 64. Thus, for a W × H transform block, where W indicates the block width and H the block height, only the top-left (W == 64 ? 32 : W) × (H == 64 ? 32 : H) transform coefficients are retained.

In [1], the prefix last_sig_coeff_x_prefix and last_sig_coeff_y_prefix are both context coded using truncated Rice binarization with cMax = (log2TbSize << 1) – 1. Here, if the syntax element to be parsed is last_sig_coeff_x_prefix, log2TbSize is set equal to log2TbWidth and otherwise, log2TbSize is set equal to log2TbHeight. That is, the maximum possible magnitude is determined by the transform block width or height. In the worst case scenario, the number of the bins that use context modelling is equal to 11. Table 1 shows the binarization for W = 64 or H = 64 in VTM3.0.

Table 1. Binarization for W = 64 or H = 64 in VTM3, where X means 0 or 1

| Magnitude of last position component | last_sig_coeff_x_prefix or last_sig_coeff_y_prefix (context coded bin) | last_sig_coeff_x_suffix or last_sig_coeff_y_suffix (bypass bin) |
| --- | --- | --- |
| 0 | 0 |  |
| 1 | 10 |  |
| 2 | 110 |  |
| 3 | 1110 |  |
| 4, 5 | 11110 | X |
| 6, 7 | 111110 | X |
| 8~11 | 1111110 | XX |
| 12~15 | 11111110 | XX |
| 16~23 | 111111110 | XXX |
| 24~31 | 1111111110 | XXX |
| 32~47 | 11111111110 | XXXX |
| 48~63 | 11111111111 | XXXX |

Proposed method

A last position coding scheme is proposed for large block-size transforms. Compared to VTM3.0, the proposed coding scheme uses less context coded bins in the worst case scenario. The codeword in the proposed scheme still starts with a truncated Rice code and followed by a fixed length code. After high-frequency zeroing, for a W×H transform block, only the top-left min(W, 32) × min(H, 32) transform coefficients are kept. Thus, the maximum possible codeword length of the prefix last_sig_coeff_x_prefix or last_sig_coeff_y_prefix is derived as:

cMax = (min(log2TbSize, 5) << 1) – 1.

Table 2 shows the binarization for W = 64 or H = 64. Here, the different parts are emphasized.

Table 2. Proposed binarization for W = 64 or H = 64, where X means 0 or 1

| Magnitude of last position component | last_sig_coeff_x_prefix or last_sig_coeff_y_prefix (context coded bin) | last_sig_coeff_x_suffix or last_sig_coeff_y_suffix (bypass bin) |
| --- | --- | --- |
| 0 | 0 |  |
| 1 | 10 |  |
| 2 | 110 |  |
| 3 | 1110 |  |
| 4, 5 | 11110 | X |
| 6, 7 | 111110 | X |
| 8~11 | 1111110 | XX |
| 12~15 | 11111110 | XX |
| 16~23 | 111111110 | XXX |
| 24~31 | 111111111 | XXX |
| 32~47 | N/A | N/A |
| 48~63 | N/A | N/A |

Table 3. Comparison of codeword length for the prefix (context coded bins) when W = 64 or H = 64

| Magnitude of last position component | VTM3.0 | Proposed method |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 2 | 2 |
| 2 | 3 | 3 |
| 3 | 4 | 4 |
| 4, 5 | 5 | 5 |
| 6, 7 | 6 | 6 |
| 8~11 | 7 | 7 |
| 12~15 | 8 | 8 |
| 16~23 | 9 | 9 |
| 24~31 | 10 | 9 |
| 32~47 | 11 | N/A |
| 48~63 | 11 | N/A |

Table 3 shows the comparison of the proposed method and VTM3.0. The context coded bins can be as long as 9 in the proposed method, while it is up to 11 bins in VTM3.0. Note that when the magnitude of the last position component in the range of 24-31, the number of context coded bins is reduced from 10 to 9.

Experimental results

The proposed method has been implemented on the VTM3.0 software. The simulations were performed following the common test conditions defined in JVET-L1010 [2]. In every case, the anchor is the VTM3.0 software.

Table 4. Experimental results for all-intra (AI) test condition; anchor is VTM3.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.11% | 0.00% | 101% | 101% |
| Class A2 | 0.03% | -0.01% | 0.00% | 100% | 100% |
| Class B | 0.01% | 0.04% | -0.03% | 100% | 98% |
| Class C | 0.00% | 0.02% | -0.03% | 100% | 98% |
| Class E | 0.03% | -0.02% | -0.04% | 100% | 98% |
| Overall | 0.01% | -0.01% | -0.02% | 100% | 99% |
| Class D | 0.01% | 0.00% | -0.01% | 101% | 99% |
| Class F | 0.00% | 0.03% | 0.03% | 99% | 97% |

Table 5. Experimental results for random-access (RA) test condition; anchor is VTM3.0

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.01% | 0.07% | 101% | 100% |
| Class A2 | 0.02% | 0.04% | 0.10% | 101% | 100% |
| Class B | 0.00% | -0.03% | 0.09% | 102% | 99% |
| Class C | 0.00% | 0.01% | -0.01% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.00% | 0.06% | 101% | 100% |
| Class D | 0.00% | 0.01% | 0.06% | 102% | 99% |
| Class F | -0.02% | -0.05% | -0.05% | 102% | 99% |

Reference

- B. Bross, et al., “Versatile Video Coding (Draft 3),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L1001, 12th meeting, Macao, CN, 3-12 Oct. 2018.

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L1010, 12th Meeting, Macao, CN, 3-12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Specification

(section 7.3.4.12 in VVC specification)

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| if( transform_skip_enabled_flag && ( cIdx ! = 0 ¦ ¦ tu_mts_flag[ x0 ][ y0 ] = = 0 ) && ( log2TbWidth <= 2 ) && ( log2TbHeight <= 2 ) ) |  |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |  |
| last_sig_coeff_x_prefix | ae(v) |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| log2SbSize = ( Min( log2TbWidth, log2TbHeight ) < 2 ? 1 : 2 ) |  |
| numSbCoeff = 1 << ( log2SbSize << 1 ) |  |
| lastScanPos = numSbCoeff |  |
| log2TbWidth = Min( log2TbWidth, 5 ) |  |
| log2TbHeight = Min( log2TbHeight, 5 ) |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − 2 * log2SbSize ) ) − 1 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ] [ lastSubBlock ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ] [ lastSubBlock ][ 1 ] |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| numSigCoeff = 0 |  |
| QState = 0 |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| startQStateSb = QState |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ] [ lastSubBlock ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ] [ lastSubBlock ][ 1 ] |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( ( i < lastSubBlock ) && ( i > 0 ) ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| firstSigScanPosSb = numSbCoeff |  |
| lastSigScanPosSb = −1 |  |
| remBinsPass1 = ( log2SbSize < 2 ? 6 : 28 ) |  |
| remBinsPass2 = ( log2SbSize < 2 ? 2 : 4 ) |  |
| firstPosMode0 = ( i = = lastSubBlock ? lastScanPos − 1 : numSbCoeff − 1 ) |  |
| firstPosMode1 = −1 |  |
| firstPosMode2 = −1 |  |
| for( n = ( i = = firstPosMode0; n >= 0 && remBinsPass1 >= 3; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| remBinsPass1− − |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |  |
| numSigCoeff++ |  |  |
| abs_level_gt1_flag[ n ] | ae(v) |
| remBinsPass1− − |  |
| if( abs_level_gt1_flag[ n ] ) { |  |
| par_level_flag[ n ] | ae(v) |
| remBinsPass1− − |  |
| if( remBinsPass2 > 0 ) { |  |
| remBinsPass2− − |  |
| if( remBinsPass2 = = 0 ) |  |
| firstPosMode1 = n − 1 |  |
| } |  |
| } |  |
| if( lastSigScanPosSb = = −1 ) |  |
| lastSigScanPosSb = n |  |
| firstSigScanPosSb = n |  |
| } |  |
| AbsLevelPass1[ xC ][ yC ] = sig_coeff_flag[ xC ][ yC ] + par_level_flag[ n ] + abs_level_gt1_flag[ n ] |  |
| if( dep_quant_enabled_flag ) |  |
| QState = QStateTransTable[ QState ][ AbsLevelPass1[ xC ][ yC ] & 1 ] |  |
| if( remBinsPass1 < 3 ) |  |
| firstPosMode2 = n − 1 |  |
| } |  |
| if( firstPosMode1 < firstPosMode2 ) |  |
| firstPosMode1 = firstPosMode2 |  |
| for( n = numSbCoeff − 1; n >= firstPosMode2; n− − ) |  |
| if( abs_level_gt1_flag[ n ] ) |  |
| abs_level_gt3_flag[ n ] | ae(v) |
| for( n = numSbCoeff − 1; n >= firstPosMode1; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( abs_level_gt3_flag[ n ] ) |  |
| abs_remainder[ n ] | ae(v) |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] + 2 * ( abs_level_gt3_flag[ n ] + abs_remainder[ n ] ) |  |
| } |  |
| for( n = firstPosMode1; n > firstPosMode2; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( abs_level_gt1_flag[ n ] ) |  |
| abs_remainder[ n ] | ae(v) |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |
| } |  |
| for( n = firstPosMode2; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| dec_abs_level[ n ] | ae(v) |
| if(AbsLevel[ xC ][ yC ] > 0 ) |  |
| firstSigScanPosSb = n |  |
| if( dep_quant_enabled_flag ) |  |
| QState = QStateTransTable[ QState ][ AbsLevel[ xC ][ yC ] & 1 ] |  |
| } |  |
| if( dep_quant_enabled_flag ¦ ¦ !sign_data_hiding_enabled_flag ) |  |
| signHidden = 0 |  |
| else |  |
| signHidden = ( lastSigScanPosSb − firstSigScanPosSb > 3 ? 1 : 0 ) |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] && ( !signHidden ¦ ¦ ( n != firstSigScanPosSb ) ) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| if( dep_quant_enabled_flag ) { |  |
| QState = startQStateSb |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 2 * AbsLevel[ xC ][ yC ] − ( QState > 1 ? 1 : 0 ) ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| QState = QStateTransTable[ QState ][ par_level_flag[ n ] ] |  |
| } else { |  |
| sumAbsLevel = 0 |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = AbsLevel[ xC ][ yC ] * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( signHidden ) { |  |
| sumAbsLevel += AbsLevel[ xC ][ yC ] |  |
| if( ( n = = firstSigScanPosSb ) && ( sumAbsLevel % 2 ) = = 1 ) ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = −TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if( tu_mts_flag[ x0 ][ y0 ] && ( cIdx = = 0 ) ) |  |
| mts_idx[ x0 ][ y0 ][ cIdx ] | ae(v) |
| } |  |  |

(section 7.4.5.12 in VVC specification)

last_sig_coeff_x_prefix specifies the prefix of the column position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_x_prefix shall be in the range of 0 to ( Min( log2TbWidth, 5 )  <<  1 ) − 1, inclusive.

last_sig_coeff_y_prefix specifies the prefix of the row position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_y_prefix shall be in the range of 0 to ( Min( log2TbHeight, 5 ) <<  1 ) − 1, inclusive.

(section 9.5.3.1 in VVC specification)

| Table 9-4 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| tile_group_data( ) | end_of_tile_group_flag | FL | cMax = 1 |
| coding_tree_unit( ) | alf_ctb_flag[ ][ ][ ] | FL | cMax = 1 |
| sao( ) | sao_merge_left_flag | FL | cMax = 1 |
|  | sao_merge_up_flag | FL | cMax = 1 |
|  | sao_type_idx_luma | TR | cMax = 2, cRiceParam = 0 |
|  | sao_type_idx_chroma | TR | cMax = 2, cRiceParam = 0 |
|  | sao_offset_abs[ ][ ][ ][ ] | TR | cMax = ( 1 << ( Min( bitDepth, 10 ) − 5 ) ) − 1, cRiceParam = 0 |
|  | sao_offset_sign[ ][ ][ ][ ] | FL | cMax = 1 |
|  | sao_band_position[ ][ ][ ] | FL | cMax = 31 |
|  | sao_eo_class_luma | FL | cMax = 3 |
|  | sao_eo_class_chroma | FL | cMax = 3 |
| coding_quadtree( ) | qt_split_cu_flag[ ][ ] | FL | cMax = 1 |
| multi_type_tree( ) | mtt_split_cu_flag | FL | cMax = 1 |
|  | mtt_split_cu_vertical_flag | FL | cMax = 1 |
|  | mtt_split_cu_binary_flag | FL | cMax = 1 |
| coding_unit( ) | cu_skip_flag[ ][ ] | FL | cMax = 1 |
|  | pred_mode_flag | FL | cMax = 1 |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | intra_luma_ref_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | intra_luma_mpm_flag[ ][ ] | FL | cMax = 1 |
|  | intra_luma_mpm_idx[ ][ ] | TR | cMax = 5, cRiceParam = 0 |
|  | intra_luma_mpm_remainder[ ][ ] | TB | cMax = 60 |
|  | intra_chroma_pred_mode[ ][ ] | 9.5.3.7 | - |
|  | merge_flag[ ][ ] | FL | cMax = 1 |
|  | inter_pred_idc[ x0 ][ y0 ] | 9.5.3.8 | cbWidth, cbHeight |
|  | inter_affine_flag[ ][ ] | FL | cMax = 1 |
|  | cu_affine_type_flag[ ][ ] | FL | cMax = 1 |
|  | ref_idx_l0[ ][ ] | TR | cMax = num_ref_idx_l0_active_minus1, cRiceParam = 0 |
|  | mvp_l0_flag[ ][ ] | FL | cMax = 1 |
|  | ref_idx_l1[ ][ ] | TR | cMax = num_ref_idx_l1_active_minus1, cRiceParam = 0 |
|  | mvp_l1_flag[ ][ ] | FL | cMax = 1 |
|  | avmr_flag[ ][ ] | FL | cMax = 1 |
|  | amvr_4pel_flag[ ][ ] | FL | cMax = 1 |
|  | gbi_idx[ ][ ] | TR | cMax = NoBackwardPredFlag ? 4: 2 |
|  | cu_cbf | FL | cMax = 1 |
| merge_data( ) | mmvd_flag[ ][ ] | FL | cMax = 1 |
|  | mmvd_merge_flag[ ][ ] | FL | cMax = 1 |
|  | mmvd_distance_idx[ ][ ] | TR | cMax = 7, cRiceParam = 0 |
|  | mmvd_direction_idx[ ][ ] | FL | cMax = 3 |
|  | ciip_flag[ ][ ] | FL | cMax = 1 |
|  | ciip_luma_mpm_flag[ ][ ] | FL | cMax = 1 |
|  | ciip_luma_mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | merge_subblock_flag[ ][ ] | FL | cMax = 1 |
|  | merge_subblock_idx[ ][ ] | TR | cMax = MaxNumSubblockMergeCand − 1, cRiceParam = 0 |
|  | merge_triangle_flag[ ][ ] | FL | cMax = 1 |
|  | merge_triangle_idx[ ][ ] | EG1 | - |
|  | merge_idx[ ][ ] | TR | cMax = MaxNumMergeCand − 1, cRiceParam = 0 |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater1_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_minus2[ ] | EG1 | - |
|  | mvd_sign_flag[ ] | FL | cMax = 1 |
| transform_unit( ) | tu_cbf_luma[ ][ ][ ] | FL | cMax = 1 |
|  | tu_cbf_cb[ ][ ][ ] | FL | cMax = 1 |
|  | tu_cbf_cr[ ][ ][ ] | FL | cMax = 1 |
|  | cu_qp_delta_abs | 9.5.3.9 | - |
|  | cu_qp_delta_sign_flag | FL | cMax = 1 |
|  | tu_mts_flag[ ][ ] | FL | cMax = 1 |
| residual_coding( ) | transform_skip_flag[ ][ ][ ] | FL | cMax = 1 |
|  | last_sig_coeff_x_prefix | TR | cMax = ( Min( log2TbWidth, 5 ) << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_y_prefix | TR | cMax = ( Min( log2TbHeight, 5 ) << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_x_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_x_prefix >> 1 ) − 1 ) − 1 ) |
|  | last_sig_coeff_y_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_y_prefix >> 1 ) − 1 ) − 1 ) |
|  | coded_sub_block_flag[ ][ ] | FL | cMax = 1 |
|  | sig_coeff_flag[ ][ ] | FL | cMax = 1 |
|  | par_level_flag[ ] | FL | cMax = 1 |
|  | abs_level_gt1_flag[ ] | FL | cMax = 1 |
|  | abs_level_gt3_flag[ ] | FL | cMax = 1 |
|  | abs_remainder[ ] | 9.5.3.10 | cIdx, current sub-block index i, x0, y0 |
|  | dec_abs_level[ ] | 9.5.3.11 | cIdx, x0, y0, xC, yC, log2TbWidth, log2TbHeight |
|  | coeff_sign_flag[ ] | FL | cMax = 1 |
|  | mts_idx[ ][ ][ ] | FL | cMax = 3 |