| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0258-v1 |
| --- | --- |

| Title: | CE3-related: Chroma intra candidates modification based on non-directional DM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo Jianglin Wang Xinwei Li Yanzhuo Ma Shuai Wan Fuzheng Yang YuanFang Yu Yang Liu | Email: | jyhuo@mail.xidian.edu.cn jlwang_2@stu.xidian.edu.cn xwli_xidian@stu.xidian.edu.cn yzma@mail.xidian.edu.cn swan@nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

Abstract

This contribution modifies chroma intra mode candidates when DM is non-directional. Specifically, if DM is Planar or DC, directionanl modes (mode 6,40,61) are used as the chroma intra mode.

The simulation results show -xxx%, -xxx%, and -xxx% BD-rates on Y, Cb, and Cr components, respectively, compared to VTM3.0 in All Intra configuration. The proposed method has negligible impact on the VTM encoder and decoder complexity. And the simulation results the proposed method with the method based on directional DM proposed in JVET- M0213 show -xxx%, -xxx%, and -xxx% BD-rates on Y, Cb, and Cr components, respectively, compared to VTM3.0 in All Intra configuration.

Introduction

In VVC draft, chroma intra coding modes are DM, LM and 4 additional default modes (Planar, Vertical, Horizontal and DC). When the chroma intra prediction mode is DM, the intra prediction mode of the co-located luma block is used. In case of DM being equal to any of the default modes, that default mode is replaced by VDIA_IDX (mode 66).

Proposed method

Because of the separate tree for luma and chroma, the current chroma block may have multiple co-located luma blocks. In chroma intra prediction, the mode from DM which uses the intra prediction mode of the corresponding luma block maybe a local optimal mode.

When DM is Planar or DC, the center region of the current chroma block is flat. If the whole region is flat, DC or Planar is used. Or else, there may exist some tiny directions.

This contribution proposes to select 3 tiny directional modes (mode 6, 40, 61) to replace Vertical, Horizontal and VDIA_IDX when DM is PLANAR or DC. The three angular modes (mode 6,40,61) are shown in Figure 1.

Figure 1 three angular modes (mode 6,40,61)

Chroma intra prediction modes of the proposed method is shown in Table 1.

Table 1: Chroma intra prediction modes of the proposed method.

| Chroma prediction mode | DM mode |  |  |
| --- | --- | --- | --- |
|  | PLANAR_IDX | DC_IDX | X ( 2 <= X <= 66 ) |
| 0 | 40 | PLANAR_IDX | PLANAR_IDX |
| 1 | 6 | 6 | VER_IDX |
| 2 | 61 | 61 | HOR_IDX |
| 3 | DC_IDX | 40 | DC_IDX |
| 4 | LM | LM | LM |
| 5 | LM_L | LM_L | LM_L |
| 6 | LM_T | LM_T | LM_T |
| 7 | PLANAR_IDX | DC_IDX | X |

The proposed method has negligible impact on the VTM encoder and decoder complexity.

Experimental results

The proposed schemes are implemented based on VTM3.0 under the JVET common test condition.

Test #1: Test 1 shows the performance of proposed method.

Table 1. Summary of experimental results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class C | -0.01% | -0.04% | -0.12% |  |  |
| Class E | #VALUE! | #VALUE! | #VALUE! |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | -0.02% | 0.06% | -0.04% |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! |  |  |

Test #2: Test 2 shows the performance of the proposed method with the method based on directional DM proposed in JVET-M0213.

Table 2. Summary of experimental results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class C | -0.05% | -0.25% | -0.42% |  |  |
| Class E | -0.05% | -0.27% | -0.35% |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | -0.07% | -0.25% | -0.27% |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! |  |  |

Conclusions

This contribution presents a method for chroma intra candidates modification based on non-directional DM. It is suggested to change Vertical, Horizontal and VDIA_IDX to 6, 61, 40 in chroma intra candidates of VVC.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).