| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0264 |
| --- | --- |

| Title: | Non-CE4: Harmonization between HMVP and GBi |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junru Li, Shanshe Wang, Wen Gao (Peking University) Li Zhang, Kai Zhang, Hongbin Liu, Jizheng Xu (Bytedance) Xiaoyu Xiu, Jiancong Luo, Yuwen He (InterDigital) | Tel: Email: | 1-858-999-7093 lizhang.idm@bytedance.com yuwen.he@interdigital.com |
| Source: | Peking University, Bytedance Inc. and InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

In current VVC, History-based Motion Vector Prediction (HMVP) and Generalized Bi-Prediction (GBi) are both employed. With GBi, each inter-coded block is further assigned one bi-prediction weight index in addition to the motion information. The bi-prediction weight index is generally carried through the merge motion vector prediction process, but it is not carried through the HMVP table. To align the design, it is proposed to store the coded bi-prediction weight index in addition to the motion information for all HMVP candidates. Simulation results reportedly show that the harmonization achieves 0.01% and 0.01% BD rate reduction for RA Main10 and LDB Main 10 configurations, respectively.

# Introduction

The History-based Motion Vector Prediction (HMVP) and Generalized Bi-Prediction (GBi) were both adopted in the 3rd VVC meeting [1][2]. In HMVP, a HMVP table is maintained to store non-duplicated motion information of previously coded blocks. When decoding a block, the HMVP candidates could be treated as merge candidates for the current block in addition to conventional spatial and temporal merge candidates. After decoding the block, the decoded motion information may be further added to the HMVP table with MV redundancy removal.

In GBi, one bi-prediction weight index may be coded or inherited from neighboring blocks. Generally speaking, the bi-prediction weight index is carried through the merge motion vector prediction process, but it is not currently carried through the HMVP table. For each HMVP candidate, the bi-prediction weight index is set to default (i.e. equal weight for bi-prediction) regardless of what the original bi-prediction index is.

# Proposed methods

To keep the design being consistent with other spatial merge candidates, for each HMVP candidate in the HMVP table, it is proposed to store the associated bi-prediction weight index in addition to the motion information. Meanwhile, when a HMVP candidate is added to the regular merge list, the associated bi-prediction weight index is also inherited.

# Experimental Results

The proposed method is implemented on VTM-3.0 [3]. The simulations were performed following the JVET CTC [4]. Results of Random Access (RA) configuration and Low-delay B (LB) configuration are summarized in the Table 1. More detailed results can be found in the attached spreadsheets.

Table 1. Results of proposed methods compared to VTM-3.0

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | -0.06% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.00% | -0.03% | 0.06% | 100% | 100% |  |  |  |  |  |
| Class B | -0.03% | -0.01% | -0.05% | 99% | 97% | -0.03% | -0.25% | -0.10% | 101% | 100% |
| Class C | 0.00% | 0.01% | 0.03% | 101% | 96% | -0.02% | 0.21% | 0.04% | 100% | 94% |
| Class E |  |  |  |  |  | 0.04% | -0.36% | -0.10% | 99% | 98% |
| Overall | -0.01% | -0.02% | -0.01% | 100% | 98% | -0.01% | -0.12% | -0.05% | 100% | 97% |
| Class D | 0.01% | -0.03% | -0.04% | 99% | 100% | -0.05% | -0.16% | -0.11% | 101% | 99% |
| Class F | 0.00% | -0.01% | 0.05% | 99% | 97% | -0.06% | 0.02% | -0.14% | 101% | 97% |

# Conclusions

This contribution proposes to inherit the bi-prediction weight index for each HMVP candidate. With such modification, the design of spatial merge and HMVP merge candidate is consistent. It is recommended to consider adoption of the proposed method to the next version of VTM.

# Reference

- “CE4: History-based Motion Vector Prediction (Test 4.4.7) ”, L. Zhang, K. Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong, Doc. JVET-L0266, Oct. 2018.

- “CE4-related: Generalized bi-prediction improvements combined from JVET-L0197 and JVET-L0296”, Y.-C. Su, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, Y. He, J. Luo, X. Xiu, Y. Ye, Doc. JVET-L0646, Oct. 2018.

- “JVET common test conditions and software reference configurations”, J. Boyce, K. Suehring, X. Li, V. Seregin, Doc. JVET-J1010, April 2018.

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

# Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).