| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0267-v2 |
| --- | --- |

| Title: | Non-CE4: Harmonization of MMVD and AMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Jizheng Xu, Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to harmonize MMVD and AMVR. To signal the MVD in MMVD mode, The AMVR information is signaled first, followed by a MVD signaled with the specified MV resolution. Simulation results reportedly show 0.14% BD-rate savings under Random Access (RA) configurations.

# Introduction

In VTM-3.0, there are two kinds of inter-coded mode explicitly signaling MVDs: the regular AMVP mode and the newly adopted Merge with MVD (MMVD) mode.

With the regular AMVP mode, the MVD is signaled with an Adaptive MV Resolution (AMVR). The AMVR information is signaled first, to indicate the MV resolution (1/4-pel, 1-pel or 4-pel). Then the MVD is signaled with the specified MV resolution.

With the MMVD mode, the MVD (a.k.a. distance) is signaled directly. A truncated unary code is signaled the chosen MVD from: {1/4-pel, ½-pel, 1-pel, 2-pel, 4-pel, 8-pel, 16-pel}.

# Harmonization of MMVD and AMVR

It is proposed that MVD is also signaled with AMVR for MMVD mode. To signal the MVD in MMVD mode, The AMVR information is signaled first, followed by a MVD signaled with the specified MV resolution. When the MV resolution is 1/4-pel, MVD is chosen from {1/4-pel, ½-pel}; when the MV resolution is 1/2-pel, MVD is chosen from {1-pel, 2-pel}; when the MV resolution is 1/2-pel, MVD is chosen from {4-pel, 8-pel, 16-pel}. MVD is still coded by a truncated unary code.

| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| --- | --- |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| amvr_mode[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx_in_subset[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } |  |

# Experimental Results

The proposed method has been implemented on top of VTM-3.0. Results of RA configurations are summarized in Table 1. The simulations were performed following the JVET CTC [1]. More detail results can be found in the attached spreadsheets.

Table 1: Results on VTM-3

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | -0.34% | -0.35% | 99% | 100% |
| Class A2 | -0.18% | -0.17% | -0.10% | 100% | 100% |
| Class B | -0.10% | -0.14% | -0.03% | 100% | 101% |
| Class C | -0.05% | 0.02% | -0.09% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -0.14% | -0.12% | 100% | 100% |
| Class D | -0.03% | -0.07% | -0.03% | 100% | 102% |
| Class F | -0.04% | -0.01% | 0.04% | 100% | 100% |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-L1010, “JVET common test conditions and software reference configurations for SDR video”, Oct. 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).