| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0278_v1 |
| --- | --- |

| Title: | Non-CE7 : Residual rearrangement for transform skipped blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Jungah Choi Jin Heo Jangwon Choi Ling Li Jaehyun Lim 19 Yangjaedaero-11gil, Seocho-gu Seoul 06772, South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 sunmi.yoo@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In this contribution, reordering residuals for intra predicted blocks is proposed. When a block is coded with an intra prediction mode, the residuals of the block have the similar direction of the predicted angle and the levels of the non-zero residuals become greater as it is far from the reference samples if the block is not transformed. On the other hand, general residual coding is designed for the transformed blocks so that the statistics of each syntax element may go undesirable when the transform-skipped block is coded, which degrades the coding efficiency. Therefore, it is proposed to flip a residual block horizontally or vertically according to the intra mode. The experimental result reportedly shows that the proposed scheme changes luma BD-rate by -0.03%, -0.02%, and X.XX% in AI, RA, and LDB, respectively, in the common test conditions. Especially, in Class F and SCC test sequences, which have more benefit the from transform skip, it changes the luma BD-rate by -0.94% and -1.38% in all intra configuration, respectively.

Problem Statement

In the current VVC standard [1], TU blocks smaller or equal to 4x4 blocks can be transform-skipped. If the current block is coded with one of intra prediction modes, the residual block has the similar direction of the predicted angle – the levels of the non-zero residuals in the block become greater as they are far from the reference samples. If the block is not transformed, greater residuals remain in the right or bottom area, whereas the transformed residuals are concentrated to the top left area. Because a general residual coding is designed for the transformed blocks, the statistics of each syntax element may become undesirable when those blocks are coded, which may decrease the coding efficiency.

Proposed method

To maintain the statistics of the syntax elements for residual coding, it is proposed to flip a residual block horizontally or vertically according to the intra mode. In this method, the intra prediction mode can be divided into three categories – using mainly top-reference modes, left-reference modes, and others.

- Top-reference modes : VER_IDX -3 to 80

- Left-reference modes : -14 to HOR_IDX + 3, Planar mode, DC mode

Figure 1 Categorized intra modes for proposed method

For example, if the block is predicted with a vertical prediction mode, it is determined as the top-reference group, and the block is horizontally flipped because its residual may be concentrated in the bottom of the block. When the intra mode is one in the left-reference group, the block is vertically flipped because its residual is more likely to be in the right side of the block. Especially, non-directional modes, planar and DC modes, are categorized in the left-reference group by experiment.

- Horizontal Flip

- Vertical Flip

Figure 2 Proposed horizontal and vertical flip

Experimental result

The proposed method has been implemented using VTM-3.0 [2] as software base and experimentally evaluated according to the common test conditions [3], in addition to SCC sequences used in CE8.

Table 1 Result of the proposed method (All intra)

Table 2 Result of the proposed method (Random access)

Table 3 Result of the proposed method (Low delay B)

Conclusion

To summarize, it is proposed to flip horizontally or vertically for intra predicted transform skip blocks. Generally, residual coding is designed for the transformed blocks so that the statistics of each syntax element may go unwanted way when the transform-skipped block is coded. The proposed scheme rearranges the residuals that have larger levels to left or top area of the block, which is similar with the transformed blocks, and it is proved that the proposed method is promising. Therefore, it is recommended that this technology is studied in the CE with different rearrangement methods, such as rotation and mirror.

Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” JVET-L1001, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).