| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MO, 9–18 Jan. 2019 | Document: JVET-M0282 v1 |
| --- | --- |

| Title: | CE2.2.8: Affine motion predictor pruning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Robert, F. Le Léannec, F. Galpin, T. Poirier | Email: | antoine.robert@technicolor.com fabrice.leleannec@technicolor.com franck.galpin@technicolor.com tangi.poirier@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution describes a simple motion predictor pruning process to be applied in Affine AMVP and Affine Merge modes to avoid redundancy in the lists of predictors and to be close to the AMVP processes. The proposed process has been implemented in the JVET VTM3.0. The reported simulation results show an average luma BD-rate gain of -0.02% in RA configuration and the same results in LDB configuration. The encoding and decoding times stay identical to the VTM-3.0 ones. The worth case complexity in terms of integer comparisons required by the pruning is increased to 18 comparisons per reference frame in Affine AMVP and by 14 comparisons in addition to the 78 actual ones in Affine Merge mode.
Combined with the proposal of JVET-M0281, corresponding to CE4.1.5, that performs motion predictor pruning for AMVP and Merge modes, the reported simulation results show an average luma BD-rate gain of -0.04% in RA configuration and of -0.03% in LDB configuration. The encoding and decoding times here also stay identical to the VTM-3.0 ones. And, the worth case complexity in terms of integer comparisons required by the pruning is further increased by 24 comparisons in addition to the 174 actual ones in Merge mode.

# Introduction

In the current Affine AMVP and Affine Merge motion predictor lists’ constructions, it is possible to encounter the same predictor several times. This reduces the achievable performances.

At the last meeting in Macao, several modifications concerning pruning have been adopted:

- In Affine AMVP, all pruning processes have been removed.

- In Affine Merge, the pruning processes have been limited to virtual candidates where the reference frame and the motion vector of each seed are compared (same reference frame but different motion vectors).

This contribution proposes a harmonization and some simple motion predictor pruning processes for Affine AMVP and Affine Merge modes that allow reducing the redundancy in the lists of predictors. It is also possible to clean and speed-up these predictor lists’ constructions.

# Description

In Affine AMVP, a motion predictor is a 4- or 6-parameters affine model described by 2 or 3 motion vectors respectively dedicated to a particular reference picture, and a list must contain 2 motion predictors. The process is repeated for each reference frame of each reference frame list.

It has been adopted to remove all the pruning in Affine AMVP, and to reduce the number of possible inherited candidates that are now picked in a way close to the one of spatial candidates in AMVP.

This contribution proposes to make the Affine AMVP motion predictor list construction even more close to the AMVP mode. In AMVP, rounding are performed on each candidate before pruning, here in Affine AMVP rounding are performed but without pruning. It is proposed:

- After getting the 2 spatial inherited candidates, if both are equal, then the second is removed (like in AMVP),

- To prune the virtual candidate (as the temporal candidate in AMVP),

- To add an early and simple pruning of the virtual translational candidates

  - If vLT and vRT exist and are equal, then vRT is set invalid for virtual translational candidate

  - If vLT and vLB exist and are equal or vRT and vLB exist and are equal, then vLB is set invalid for virtual translational candidate

Figure 1 represents the candidates considered during the Affine AMVP list creation, their number over the list size, and the worth case complexity analysis where the number of motion vectors involved in the pruning of motion predictors are counted to get the total number of integer comparisons (2 per motion vector).

The associated worth case complexity in terms of integer comparisons is increased to 18 comparisons per reference frame (compared to 16 in AMVP).

Figure 1: Proposed Affine AMVP list construction modification (and complexity in comparisons)
[black: VTM-3.0 – orange: proposed modification]

In Affine Merge mode, a motion predictor is one or two 4- or 6-parameters affine model(s) described by 2 or 3 motion vectors respectively with its associated reference picture, and a list must contain 5 motion predictors.

The pruning process have been limited to virtual candidates where the reference frame and the motion vector of each seed are compared (same reference frame but different motion vectors). In the list of motion predictors, the first candidates are still spatial inherited models picked from left and top positions as in Affine AMVP (and AMVP). Furthermore, at the end, the list is fulfilled by null models which are not considered in the following RDO.

This contribution then proposes to:

- Take into account the null models,

- Perform the same pruning of the spatial inherited candidates as proposed in Affine AMVP: after getting the 2 spatial inherited candidates, if both are equal, then the second is removed (like in AMVP).

Figure 2 represents the candidates considered during the Affine Merge list creation, their number over the list size, and the worth case complexity analysis where the number of motion vectors and associated reference frame indexes involved in the pruning of motion predictors are counted to get the total number of integer comparisons (2 per motion vector and 1 per reference frame index).

The associated worth case complexity in terms of integer comparisons is increased by only 14 comparisons on top of the 78 existing ones (+15%).

Figure 2: Proposed Affine Merge list construction modification (and complexity in comparisons)
[black: VTM-3.0 – orange: proposed modification]

# Simulation results

The simulations are conducted using the VTM configuration as described in JVET-L1010 [1]. VTM-3.0 reference software is used with VTM configuration files.

## CE2.2.8 test a): Affine AMVP

Table 1 and Table 2 show the simulation results of the proposed harmonization of the Affine AMVP predictor list construction in random access and low delay B configurations. In both configurations, the proposed harmonization allows slight improvements of the BD-rate performances with the same complexity in encoding/decoding time and with an increased number of integer comparisons requested by pruning (18 per reference frame instead of none).

Table 1: Simulation results of the proposed harmonization of the Affine AMVP motion predictor list in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | -0.05% | 100% | 101% |
| Class A2 | -0.01% | 0.04% | 0.06% | 100% | 100% |
| Class B | -0.01% | 0.07% | -0.02% | 100% | 101% |
| Class C | 0.03% | -0.08% | 0.03% | 100% | 99% |
| Overall | 0.00% | 0.01% | 0.00% | 100% | 100% |
| Class D | -0.03% | -0.12% | -0.08% | 99% | 98% |
| Class F (optional) | -0.02% | 0.04% | 0.02% | 100% | 100% |

Table 2: Simulation results of the proposed harmonization of the Affine AMVP motion predictor list in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.01% | -0.26% | -0.31% | 100% | 101% |
| Class C | 0.00% | 0.29% | 0.26% | 100% | 100% |
| Class E | -0.03% | 0.22% | -0.12% | 100% | 100% |
| Overall | -0.01% | 0.04% | -0.07% | 100% | 100% |
| Class D | 0.00% | 0.05% | -0.15% | 100% | 100% |
| Class F (optional) | -0.08% | -0.20% | -0.32% | 100% | 99% |

## CE2.2.8 test b): Affine Merge

Table 3 and Table 4 show the simulation results of the proposed pruning process of the Affine Merge inherited candidates in random access and low delay B configurations. In both configurations, the proposed pruning process improves a bit the BD-rate performances with the same complexity in encoding/decoding time and with an increased number of integer comparisons requested by pruning (14 on top of the 78 existing ones).

Table 3: Simulation results of the proposed pruning of the Affine Merge inherited candidates in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.17% | 0.01% | 101% | 101% |
| Class A2 | -0.03% | -0.06% | 0.07% | 101% | 101% |
| Class B | -0.02% | 0.06% | -0.12% | 101% | 101% |
| Class C | 0.01% | 0.07% | 0.10% | 100% | 99% |
| Overall | -0.01% | -0.01% | 0.00% | 101% | 100% |
| Class D | -0.02% | -0.07% | 0.02% | 100% | 98% |
| Class F (optional) | 0.03% | 0.04% | 0.07% | 101% | 101% |

Table 4: Simulation results of the proposed pruning of the Affine Merge inherited candidates in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.01% | -0.14% | 0.01% | 100% | 101% |
| Class C | -0.02% | 0.22% | 0.30% | 100% | 99% |
| Class E | -0.06% | -0.06% | -0.62% | 100% | 99% |
| Overall | -0.03% | 0.00% | -0.05% | 100% | 100% |
| Class D | -0.07% | -0.02% | -0.18% | 100% | 99% |
| Class F (optional) | 0.06% | 0.22% | 0.19% | 100% | 99% |

## CE2.2.8 test c): combined Affine AMVP and Affine Merge

Table 5 and Table 6 show the simulation results of the combination of Affine AMVP and Affine Merge modifications of motion predictor list constructions in random access and low delay B configurations. In both configurations, the proposed modifications allow improving the BD-rate performances for all sequence classes with the same complexity in encoding/decoding time and with an increased number of integer comparisons requested by pruning (18 per reference frame in Affine AMVP and 14 on top of the 78 existing ones in Affine Merge).

Table 5: Simulation results of the proposed Affine AMVP and Affine Merge modifications of predictor lists in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.15% | -0.02% | 101% | 101% |
| Class A2 | -0.02% | 0.03% | 0.06% | 100% | 100% |
| Class B | -0.03% | 0.06% | -0.10% | 101% | 101% |
| Class C | -0.01% | 0.02% | 0.09% | 100% | 100% |
| Overall | -0.02% | 0.00% | 0.00% | 100% | 100% |
| Class D | -0.02% | -0.10% | -0.04% | 100% | 99% |
| Class F (optional) | 0.02% | 0.03% | 0.02% | 101% | 101% |

Table 6: Simulation results of the proposed Affine AMVP and Affine Merge modifications of predictor lists in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.01% | -0.43% | 0.02% | 100% | 100% |
| Class C | -0.01% | 0.40% | 0.21% | 100% | 100% |
| Class E | 0.05% | 0.39% | 0.06% | 100% | 99% |
| Overall | 0.00% | 0.05% | 0.10% | 100% | 100% |
| Class D | -0.07% | -0.16% | -0.12% | 100% | 100% |
| Class F (optional) | 0.04% | 0.17% | 0.39% | 100% | 99% |

## CE2.2.8 test d): combined CE2.2.8 c) with CE4.1.5 c)

Table 7 and Table 8 show the simulation results in random access and low delay B configurations of the combination of CE2.2.8 test c) and CE4.1.5 test c) of JVET-M0281 [3] that performs motion predictor pruning for AMVP and Merge modes. In both configurations, the proposed modifications allow improving the BD-rate performances with the same complexity in encoding/decoding time and with a moderately increased number of integer comparisons requested by pruning (18 per reference frame in Affine AMVP, 14 on top of the 78 existing ones in Affine Merge and 24 on top of the 174 existing ones in Merge).

Table 7: Simulation results of the combination of CE2.2.8 c) and CE4.1.5 c) in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.09% | 0.01% | 101% | 100% |
| Class A2 | -0.06% | 0.02% | 0.00% | 100% | 100% |
| Class B | -0.05% | 0.08% | -0.08% | 101% | 102% |
| Class C | -0.04% | -0.09% | 0.04% | 100% | 99% |
| Overall | -0.04% | -0.01% | -0.01% | 100% | 100% |
| Class D | -0.03% | -0.10% | -0.01% | 99% | 98% |
| Class F (optional) | 0.02% | 0.05% | 0.04% | 101% | 101% |

Table 8: Simulation results of the combination of CE2.2.8 c) and CE4.1.5 c) in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.05% | -0.31% | -0.23% | 100% | 101% |
| Class C | -0.05% | 0.11% | 0.10% | 100% | 100% |
| Class E | 0.02% | -0.12% | -0.43% | 100% | 100% |
| Overall | -0.03% | -0.12% | -0.17% | 100% | 100% |
| Class D | -0.07% | -0.24% | 0.11% | 99% | 100% |
| Class F (optional) | -0.05% | -0.14% | -0.10% | 100% | 99% |

We want to kindly thank Canon for their cross-check.

# Conclusion

This contribution proposes an uniformization of the motion predictor list constructions pruning in the Affine AMVP mode like in AMVP and in the Affine Merge mode like the proposed Affine AMVP associated with some cleaning. Simulation results reportedly show that the proposed method can bring an average luma BD-rate gain of -0.02% for RA and the same results for LDB using the VTM configuration. The complexity in encoding and decoding time stays the same for RA and LDB. The worth case complexity in terms of integer comparisons required by the pruning is increased to 18 comparisons in Affine AMVP mode and by 14 comparisons in addition to the 78 actual ones in Affine Merge mode.
Combined with the proposal of JVET-M0281 CE4.1.5 [3] that performs motion predictor pruning for AMVP and Merge modes, simulation results reportedly show an average luma BD-rate gain of -0.04% for RA and -0.03% for LDB. Simulations also show that the complexity in encoding and decoding time stays the same for RA and LDB. In that case, the worth case complexity in terms of integer comparisons required by the pruning is further increased by 24 comparisons in addition to the 174 actual ones in Merge mode.
It is suggested that the uniformization of the motion predictor pruning proposed in this contribution should be adopted into the reference test model.

# References

[1] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1010, 12th Meeting, Macao, CN, October 2018.

[2] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 2 (CE2): Sub-block based motion prediction”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1022, 12th Meeting, Macao, CN, October 2018.

[3] A. Robert, F. Le Léannec, F. Galpin, T. Poirier, “CE4.1.5: Inter motion predictor pruning”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M0281, 13th Meeting, Marrakesh, MO, January 2019.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).