| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0284 |
| --- | --- |

| Title: | CE9-related: BDOF Modifications to Enable 64x64 VPDU |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen, Xiang Ma, Semih Esenlik, Haitao Yang, Jianle Chen | Tel: Email: | +49-162-210-6260 semih.esenlik@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution presents modifications to BDOF (Bi-Directional Optical Flow) in order to enable 64x64 square VPDU (Virtual Pipeline Data Unit). The document presents results of 4 alternative methods:

- Disabling BDOF for CUs greater than 64 luma samples in one dimension,

- Splitting BDOF application region into 2 or 4 smaller sub-regions, if the region crosses a 64x64 VPDU boundary

- Disabling 1 row (or 1 column) of BDOF sub-blocks that are adjacent to the VPDU boundaries, if the CU applying BDOF crosses a horizontal (or vertical) VPDU boundary. The subblocks that are disabled lay at the top (or left) of the VPDU boundary.

- Disabling 2 rows (or 2 column) of BDOF sub-blocks that are adjacent to the VPDU boundaries, if the CU applying BDOF crosses a horizontal (or vertical) VPDU boundary.

The simulation results show negligible loss in BD-rate performance.

# Introduction

During the 12th JVET meeting in Macau, the concept of VPDU was introduced into the VVC draft specification. The goal of VPDU is to guarantee completion of the processing of one 64x64 square region before starting the processing of other 64x64 square regions. This method is intended to reduce the memory footprint of the pipelined hardware implementations.

During the same meeting BDOF was included in the VVC draft specification. It is noted that in the current specification the maximum luma CU size that is allowed to apply BDOF is set to be equal to 128x128.

# Problem Statement

Assume a CTU is split horizontally into 2 leaf CUs, and the top CU applies BDOF. Since BDOF is applied to coding blocks that are larger than 64 luma samples in one dimension, two problems exist:

- The CU-based early termination decision requires that SAD matching cost is computed for the whole CU. Accordingly in order the finish the processing of the top-left VPDU, top-right VPDU also needs to be reconstructed. Hence the intended goal of sequential processing of VPDUs cannot be achieved.

- BDOF is applied on a 4x4 sub-block basis where gradients are computed for the 6x6 extended window surrounding the sub-block. Similarly to the first problem, the top-right VPDU needs to be reconstructed in order to enable the computation of gradients along the VPDU boundary, before the processing of top-left VPDU is finished.

# Proposed Solutions

Four solutions are proposed to solve the above mentioned problems.

## Disabling BDOF for large CUs

According to the first proposed solution, the BDOF is disabled for CUs which has one dimension greater than 64 samples. It is noted that this modification together with the CU split constraints imposed by the VPDU conditions guarantee that a BDOF application region never crosses a VPDU boundary.

## Implicit splitting of BDOF application region

According to the second proposed solution, if BDOF is applied at a CU which has one dimension that is greater than 64 samples, the CU is split into 64x64 square regions, where each region is considered to be an independent BDOF application region. In other words, The BDOF operation considers the CU to be composed of multiple sub-blocks of size 64x64, and the gradients are calculated using padded samples along the VPDU boundaries that divide the CU into sub-blocks.

## Disabling BDOF sub-blocks along the VPDU edges

According to the third proposed solution, if BDOF is applied at a CU which has one dimension that is greater than 64 samples, the BDOF subblocks along the VPDU edges are disabled. This is illustrated in the example below, where a CU is equal in size to 128x128.

Figure 1: Illustration of the region where BDOF is disabled.

In this solution the CU-based early termination is disabled additionally if the CU size is greater than 64 in one dimension. The subblocks that are disabled are located at the top or left of the VPDU boundary.

## Disabling BDOF sub-blocks along the VPDU edges – alternative 2

According to the fourth proposed solution, if BDOF is applied at a CU which has one dimension that is greater than 64 samples, the BDOF subblocks along the VPDU edges are disabled. This is illustrated in the example below, where a CU is equal in size to 128x128.

Figure 1: Illustration of the region where BDOF is disabled.

The subblocks at both side of the VPDU boundaries are disabled in this alternative in order to prevent the need to calculate the gradients (or store) at the VPDU boundary. Similarly to the third alternative solution, CU-based early termination is disabled additionally if the CU size is greater than 64 in one dimension.

# Simulation Results

The simulation results for the proposed solutions are presented in the tables below.

## Disabling BDOF for large CUs

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.23% | 0.19% | 0.14% | 98% | 97% |
| Class A2 | 0.30% | 0.23% | 0.20% | 98% | 99% |
| Class B | 0.15% | 0.15% | 0.06% | 99% | 98% |
| Class C | 0.02% | 0.02% | 0.00% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | 0.14% | 0.09% | 99% | 98% |
| Class D | 0.09% | 0.07% | 0.06% | 99% | 100% |
| Class F (optional) | -0.04% | -0.02% | -0.01% | 99% | 98% |

Table 1: Simulation results for disabling BDOF for large CUs

## Implicit splitting of BDOF application region

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | 0.02% | 100% | 100% |
| Class A2 | -0.01% | 0.02% | 0.02% | 100% | 100% |
| Class B | 0.00% | 0.01% | -0.06% | 100% | 100% |
| Class C | 0.00% | -0.01% | 0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.01% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F (optional) | -0.01% | 0.01% | 0.01% | 100% | 100% |

Table 2: Simulation results for implicit splitting of BDOF application region

## Disabling BDOF sub-blocks along the VPDU edges

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.02% | 0.03% | 102% | 103% |
| Class A2 | 0.04% | 0.05% | 0.06% | 102% | 103% |
| Class B | 0.03% | 0.05% | 0.00% | 102% | 103% |
| Class C | 0.00% | 0.01% | -0.01% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.03% | 0.01% | 102% | 103% |
| Class D | 0.03% | 0.02% | 0.02% | 102% | 103% |
| Class F (optional) | 0.00% | -0.01% | -0.01% | 101% | 106% |

Table 3: Simulation results for disabling BDOF sub-blocks along the VPDU edges

## Disabling BDOF sub-blocks along the VPDU edges – second alternative

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.04% | -0.01% | 102% | 103% |
| Class A2 | 0.10% | 0.08% | 0.08% | 102% | 104% |
| Class B | 0.05% | 0.02% | 0.00% | 102% | 103% |
| Class C | 0.00% | -0.02% | 0.00% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.03% | 0.01% | 102% | 103% |
| Class D | 0.04% | 0.05% | 0.01% | 102% | 103% |
| Class F (optional) | -0.01% | -0.03% | -0.02% | 101% | 106% |

Table 4: Simulation results for disabling BDOF sub-blocks along the VPDU edges – alternative 2

# Conclusion

4 alternative methods are presented to enable 64x64 VPDU processing in the presence of BDOF. The results show negligible difference in BD-rate performance. It is recommended to adopt one of the solutions into next VVC draft.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).