| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0286-v2 |
| --- | --- |

| Title: | Non-CE4: Simplifications for triangular prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Timofey Solovyev, Semih Esenlik, Sergey Ikonin, Jianle Chen | Tel: Email: | +49-162-210-6260 solovyev.timofey@huawei.com semih.esenlik@huawei.com sergey.ikonin@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution presents simplifications for the triangular prediction mode. The following modifications are proposed:

- The additional uni-prediction merge candidate list dedicated to triangular prediction mode is removed. Instead, regular merge list is used by converting bi-prediction candidates to uni-prediction candidates.

- Table based mode coding scheme is removed by coding the split direction and merge indices separately.

The simulations results for combined solution reportedly show -0.06% and 0.02% change in luma BD-rate for RA and LDB configurations respectively, without significant change in encoding and decoding times. It is asserted that the proposed modifications eliminate the additional logic necessary for the triangular candidate list construction and simplify the binarization of the triangular prediction mode coding.

# Introduction

During the 12th JVET meeting in Macau, the triangular prediction mode has been adopted into the VVC specification draft [1]. If triangular mode is selected by a CU, it is split diagonally and each resulting triangular part is predicted using a different motion vector. The predictions generated for each triangular part is restricted to be uni-prediction.

The motion candidates used to predict the triangular parts are selected based on a uni-prediction candidate list. The uni-prediction candidate list consists of five candidates. It is derived from seven neighboring blocks including five spatial neighboring blocks (1 to 5) and two temporal co-located blocks (6 to 7). The motion vectors are collected and put into the uni-prediction candidate list according the following order:

- uni-prediction motion vectors

- L0 motion vector of bi-prediction motion vectors

- L1 motion vector of bi-prediction motion vectors

- averaged motion vectors of the L0 and L1 motion vectors of bi-prediction motion vectors.

If the number of candidates is less than five, zero motion vector is added to the list.

When the triangular prediction unit mode is applied to the CU, an index indicating the direction for splitting the CU into two triangular prediction units and the merge indices of the two triangular prediction units are signalled jointly by means of an index. The index ranges from 0 to 39. A look-up table is used for deriving the splitting direction and motion vectors from the index.

# Problem Statement

Two problems exist in the current VVC related to the triangular prediction mode.

- The uni-prediction candidate list is an additional list that requires implementation of a separate logic. Moreover construction of the list requires 28 pruning checks according to the worst case identified by the original authors.

- The joint binarization of the split direction and two merge indices require a table lookup. The table has the dimensions of 40x3. The binarization of the split direction and merge indices result in a total of 21 bins in the worst case.

# Proposed Solutions

The proposed modification consists of 2 parts, which are explained in the subsections below.

## Simplified uni-prediction list construction

The current uni-prediction candidate list construction that is used when the triangular mode is selected is considered to be complex due to the number of pruning checks that are necessary. Instead, 2 alternative solutions are presented:

### Alternative 1

The uni-prediction candidate list is constructed according to the steps of:

- Construct the regular merge candidate list

- If the candidate is bi-prediction candidate, the candidate is converted to uni-prediction by keeping the L0 motion vector.

Described method of conversion bi-prediction merge candidate to the uni-prediction is same as used for 4x4 blocks in merge mode. The number of candidates in the uni-prediction candidate list is set as 5 if the maximum number of merge candidates are greater than or equal to 5. Otherwise the uni-prediction candidate list size is set as equal to number of merge candidates.

### Alternative 2

The uni-prediction candidate list is constructed according to the steps of:

- Construct the regular merge candidate list,

- Starting from the beginning of the merge candidate list;

  - If a candidate in the merge candidate list is uni-prediction candidate, insert into uni-prediction candidate list.

  - If a candidate in the merge candidate list is bi-prediction candidate, insert L0 motion vector into uni-prediction candidate list. Insert the L1 motion vector if it is not identical to the L0 motion vector as additional candidate.

The number of candidates in the uni-prediction candidate list is set as 5 if the maximum number of merge candidates are greater than or equal to 5. Otherwise the uni-prediction candidate list size is set as equal to number of merge candidates.

Due to triangular prediction mode needs at least two different merge candidates in a list, it is not used when number of merge candidates is equal to 1.

According to the second alternative L1 motion vector of a bi-prediction candidate is compared with the L0 motion vector before insertion into the uni-prediction candidate list. The L0-L1 motion vector comparison can be performed in parallel to the other pruning checks of the merge list construction process.

In the worst case 2 additional pruning checks are necessary to construct the uni-prediction candidate list. It is noted that since the maximum size of the list is 5 (instead of 6), the number of worst case pruning checks is less than for regular merge mode.

## Simplified coding of split direction and merge indices

It is prosed to code the split direction flag and merge indices separately. The proposed changes in the syntax are as follows:

| if( mmvd_flag[ x0 ][ y0 ] = = 0 && merge_subblock_flag[ x0 ][ y0 ] = = 0 && mh_intra_flag[ x0 ][ y0 ] == 0 && slice_type = = B && sps_triangle_enabled_flag && ( cbWidth * cbHeight ) >= 64 ) { |  |
| --- | --- |
| split_direction[ x0 ][ y0 ] | ae(v) |
| merge_idx1[ x0 ][ y0 ] | ae(v) |
| merge_idx2[ x0 ][ y0 ] | ae(v) |
| merge_idx2[ x0 ][ y0 ] += merge_idx2[ x0 ][ y0 ] >= merge_idx1[ x0 ][ y0 ] ? 1:0 |  |
| } |  |

According to the proposal the merge_idx1 indicates the motion candidate of the first triangular part, merge_idx2 indicates the motion candidate of the second triangular part and split direction indicates the selection between 45-degree or 135-degree triangular partitioning.

The value of merge_idx2 is incremented by one if its value if greater than or equal to merge_idx1. The reason is to prevent selection of the same motion candidate for both triangular part, since this would be identical to not splitting the CU with triangular mode.

# Simulation Results

The simulation results for the proposed solutions are presented in the tables below. In the simulations the anchor software is VTM 3.0.

## Simplified coding of split direction and merge indices

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.00% | 101% | 101% |
| Class A2 | -0.02% | -0.01% | 0.05% | 101% | 100% |
| Class B | -0.02% | 0.07% | -0.08% | 101% | 101% |
| Class C | 0.00% | 0.06% | -0.07% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.03% | -0.03% | 101% | 101% |
| Class D | -0.05% | 0.03% | 0.07% | 101% | 102% |
| Class F (optional) | -0.01% | -0.03% | -0.02% | 101% | 103% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.18% | -0.08% | 101% | 101% |
| Class C | -0.05% | 0.18% | 0.00% | 101% | 100% |
| Class E | -0.02% | 0.04% | 0.11% | 101% | 101% |
| Overall | -0.03% | -0.01% | -0.01% | 101% | 101% |
| Class D | -0.02% | -0.20% | -0.01% | 101% | 100% |
| Class F (optional) | -0.01% | -0.21% | -0.08% | 101% | 101% |

## Simplified uni-prediction list construction – Alternative 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.04% | 0.13% | 101% | 100% |
| Class A2 | 0.01% | 0.07% | 0.13% | 101% | 100% |
| Class B | 0.07% | 0.05% | 0.02% | 101% | 101% |
| Class C | 0.18% | 0.17% | 0.23% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.07% | 0.12% | 101% | 101% |
| Class D | 0.10% | -0.01% | 0.24% | 101% | 102% |
| Class F (optional) | 0.09% | 0.10% | 0.07% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | 0.08% | 0.14% | 101% | 100% |
| Class C | 0.10% | 0.31% | 0.31% | 100% | 99% |
| Class E | 0.13% | -0.08% | 0.09% | 101% | 101% |
| Overall | 0.09% | 0.11% | 0.18% | 100% | 100% |
| Class D | 0.15% | 0.09% | 0.10% | 100% | 101% |
| Class F (optional) | 0.11% | 0.01% | 0.23% | 100% | 100% |

## Simplified uni-prediction list construction – Alternative 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | -0.04% | 101% | 100% |
| Class A2 | -0.07% | -0.07% | 0.02% | 101% | 100% |
| Class B | -0.03% | -0.03% | -0.06% | 101% | 101% |
| Class C | -0.01% | 0.00% | -0.05% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.02% | -0.04% | 101% | 101% |
| Class D | -0.06% | -0.02% | -0.15% | 100% | 102% |
| Class F (optional) | 0.00% | -0.02% | 0.01% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.18% | 0.31% | 100% | 101% |
| Class C | 0.08% | 0.39% | 0.24% | 100% | 100% |
| Class E | 0.19% | 0.29% | -0.20% | 100% | 101% |
| Overall | 0.10% | 0.13% | 0.16% | 100% | 101% |
| Class D | 0.11% | 0.00% | 0.32% | 100% | 102% |
| Class F (optional) | 0.04% | -0.01% | -0.05% | 100% | 101% |

## 4.1 + 4.2 combined

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.04% | 0.15% | 101% | 100% |
| Class A2 | 0.02% | 0.13% | 0.12% | 101% | 100% |
| Class B | 0.05% | 0.13% | 0.04% | 101% | 101% |
| Class C | 0.19% | 0.27% | 0.35% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.15% | 0.16% | 101% | 101% |
| Class D | 0.13% | 0.15% | 0.20% | 101% | 102% |
| Class F (optional) | 0.06% | 0.09% | 0.05% | 101% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.02% | -0.05% | 101% | 101% |
| Class C | 0.08% | 0.15% | 0.15% | 101% | 101% |
| Class E | 0.09% | 0.54% | -0.13% | 101% | 102% |
| Overall | 0.08% | 0.18% | 0.00% | 101% | 101% |
| Class D | 0.06% | 0.37% | -0.07% | 101% | 101% |
| Class F (optional) | 0.23% | 0.20% | 0.70% | 101% | 102% |

## 4.1 + 4.3 combined

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | 0.02% | 101% | 102% |
| Class A2 | -0.11% | -0.12% | -0.06% | 101% | 102% |
| Class B | -0.06% | 0.00% | -0.14% | 101% | 102% |
| Class C | -0.06% | -0.24% | -0.13% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.10% | -0.09% | 101% | 102% |
| Class D | -0.11% | -0.24% | -0.19% | 101% | 102% |
| Class F (optional) | -0.02% | -0.11% | -0.09% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.29% | -0.09% | 101% | 101% |
| Class C | 0.04% | 0.12% | 0.18% | 101% | 100% |
| Class E | 0.02% | 0.42% | -0.10% | 100% | 101% |
| Overall | 0.02% | 0.03% | 0.00% | 101% | 101% |
| Class D | 0.04% | 0.03% | 0.15% | 100% | 100% |
| Class F (optional) | -0.02% | -0.10% | 0.07% | 101% | 100% |

# Conclusion

The triangular prediction mode in VVC has been simplified with two modifications. According to the first modification the dedicated merge list construction of triangular prediction mode is replaced with regular merge list, where bi-prediction candidates are converted into uni-prediction candidates. According to the second modification, the table based joint binarization of merge indices and split direction syntax elements have been removed. Instead, it is proposed to signal the three syntax elements independently.

Simulation results for the combined solution (test 4.1 + 4.3) show -0.06% and 0.02% change in luma BD-rate for RA and LDB configurations respectively. The presented simplifications are recommended to be included in the next VVC draft specification.

# References

- R.-L. Liao, C. S. Lim, “CE10.3.1.b: Triangular prediction unit mode”, JVET-L0124, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- B. Bross, J. Chen, S. Liu “Versatile Video Coding (Draft 3)”, JVET-L01001_v10, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).