| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0287 |
| --- | --- |

| Title: | CE9: Integer DMVR (Test 9.2.7) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Han Gao, Anand Meher Kotra, Biao Wang, Jianle Chen | Tel: Email: | +49-162-210-6260 semih.esenlik@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution document reports the results of the core experiment CE9.2.7. In the test the initial motion vectors are rounded to integer precision before the application of motion vector refinement in order to reduce the computational complexity of DMVR. The test CE9.2.7 is designed to show the impact of the application of rounding of the initial motion vectors to integer precision in isolation. Therefore the proposed method is implemented on top of the DMVR Base Software (CE9.2.Base) and no other modification is included in the test.

Simulation results show 0.13% luma BD-rate increase and 5% decoding time reduction compared to the DMVR Base Software.

# Introduction

During the 12th JVET meeting in Macau, DMVR Base Software has been defined as anchor for DMVR tests in the upcoming CE cycle, in order to assess the individual complexity reduction aspects. The DMVR Base Software already resolves most of the important complexity issues that are associated with DMVR, such as;

- The memory BW access is same as regular bi-prediction, due to application of reference sample padding.

- The minimum luma block size is restricted to 8x8 to reduce worst case cycle requirements.

- The maximum luma block size is restricted to total 1024 samples to reduce necessary cache size.

- Bilinear interpolation filter is applied instead of 8-tap filter before refinement search to reduce to reduce computational complexity.

- The usage of the refined MVs are restricted to top and top-left CTU in spatial prediction in order eliminate pipeline delays.

The CE tests that are defined in CE9 [1] target further reduction in complexity and increased coding gain while obeying the design considerations that are used in defining the DMVR Base Software.

# Problem Statement

In the DMVR Base Software, the bilinear interpolation process for MV refinement introduces additional computational complexity as 2x(W+4)x(H+4) samples need to be generated using bilinear interpolation filter. It is noted that the filter coefficients of the bilinear interpolation filter have 4 bit accuracy. For a block with size 8x8 (which is the minimum block size for which DMVR is enabled), 624 additions and 1048 multiplications need to be performed to generate bilinear interpolation results.

# Proposed Solution

In CE test 9.2.7, the bilinear interpolation filter of the DMVR Base Software is eliminated and instead integer reference samples are used for MRSAD comparison process of DMVR. The integer reference samples are obtained by rounding the initial motion vector to the nearest integer sample coordinate before DMVR. This means that no interpolation filtering is necessary until the final refined motion vectors are obtained. Only at this step (after the final refined MV is obtained), 8-tap interpolation filtering is applied to obtain the final motion compensation results. It is noted that the final motion vector can still have fractional sample resolution, since the MV rounding operation is reverted before final motion compensated prediction.

The process is elaborated in the following figure:

Figure 1: Application of rounding before MRSAD search and inversion of the rounding operation in order to obtain refined MV.

It is noted that the proposed solution eliminates the 624 additions and 1048 multiplications that are necessary in DMVR Base Software for bilinear interpolation process.

# Simulation Results

The simulation results for the proposed solution are presented in the tables below.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.60% | -0.54% | -0.82% | 107% | 113% |
| Class A2 | -0.85% | -1.02% | -1.00% | 110% | 118% |
| Class B | -0.43% | -0.61% | -0.64% | 110% | 113% |
| Class C | -0.55% | -0.64% | -0.60% | 110% | 117% |
| Class E |  |  |  |  |  |
| Overall | -0.58% | -0.69% | -0.74% | 109% | 115% |
| Class D | -0.34% | -0.62% | -0.42% | 111% | 118% |
| Class F (optional) | -0.26% | -0.39% | -0.32% | 107% | 116% |

Table 1: Simulation results for Integer based DMVR vs. VTM3.0 anchor

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0_CE9.2_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.06% | 0.01% | 98% | 98% |
| Class A2 | 0.12% | 0.11% | 0.14% | 98% | 97% |
| Class B | 0.14% | 0.05% | 0.09% | 98% | 96% |
| Class C | 0.23% | 0.26% | 0.30% | 98% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.13% | 0.12% | 0.14% | 98% | 96% |
| Class D | 0.40% | 0.30% | 0.51% | 97% | 94% |
| Class F (optional) | 0.04% | 0.08% | 0.08% | 100% | 97% |

Table 2: Simulation results for Integer based DMVR vs. DMVR Base Software

It is noted that the simulation results presented in Table 2 are obtained by the crosschecker.

# Conclusion

This contribution document reports the results of the CE test CE9.2.7 (integer DMVR). The test is designed to show the independent impact of replacing the bilinear interpolation with no interpolation (by rounding the initial motion vectors to integer precision) before MV refinement search. The presented results show 0.13% luma BD-rate increase and 4% decoding time reduction compared to the DMVR Base Software.

# References

- Description of Core Experiment 9 (CE9): Decoder-side motion derivation, Document JVET-L1026, 	X. Xiu, S. Esenlik

- Versatile Video Coding (Draft 3), Document JVET-L01001_v10, B. Bross, J. Chen, S. Liu

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).