| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-0290 |
| --- | --- |

| Title: | CE10: Simplification on Combined Inter-Intra Prediction (test 10.1.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weiwei Xu Biao Wang Haitao Yang Jianle Chen | Tel: Email: | +8613758193753 xuweiwei3@huawei.com biao.wang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This proposal simplifies the combined Inter-Intra prediction adopted in Macao meeting from three aspects. It aims to reduce the 4 intra modes used in the combined Inter-Intra prediction to only Planar mode, to simplify the Planar prediction process by skipping PDPC, and to use simpler weighting factor for the whole CU in Inter-Intra prediction sample averaging. Four Tests with different combinations of above simplifications are performed, named as 10.1.3.a, 10.1.3.b, 10.1.3.c and 10.1.3.d.

It is reported that, compared to VTM3.0, the luma BD-rate of proposed CE10.1.3.a are -0.01% in RA, and 0.10% in LDB with encoding time slightly decreased and decoding time remains the same; the luma BD-rate of proposed CE10.1.3.b are 0.01% in RA, and 0.01% in LDB with encoding and decoding time remains the same; the luma BD-rate of proposed CE10.1.3.c are 0.05% in RA, and 0.10% in LDB with encoding and decoding time remains the same; the luma BD-rate of proposed CE10.1.3.d are 0.02% in RA, and 0.01% in LDB with encoding time slightly decreased, respectively.

# Introduction

Four combinations of the proposed simplification methods are tested.

## CE10.1.3.a: combined inter-intra prediction with only planar mode for intra

When the combined inter-intra prediction is applied, the final prediction combines one intra prediction and one merge indexed prediction. In a merge CU, one flag is signaled for merge mode to select the usage of intra planar mode when the flag is true. For chroma component, DM is always applied without extra signaling.

## CE10.1.3.b: combined inter-intra prediction with PDPC skipped for intra planar prediction

When combined inter-intra prediction is applied, the final prediction combines one intra prediction and one merge indexed prediction. In a merge CU, one flag is signaled for merge mode to select an intra mode from an intra candidate list when the flag is true. For luma component, the intra candidate list is derived from 4 intra prediction modes including DC, planar, horizontal, and vertical modes, and the size of the intra candidate list can be 3 or 4 depending on the block shape. When the CU width is larger than the double of CU height, horizontal mode is exclusive of the intra mode list and when the CU height is larger than the double of CU width, vertical mode is removed from the intra mode list. One intra prediction mode selected by the intra mode index and one merge indexed prediction selected by the merge index are combined using weighted average. For chroma component, DM is always applied without extra signaling.

When PLANAR mode is used for intra prediction, the PDPC is skipped during intra prediction sample generation.

## CE10.1.3.c: combined inter-intra prediction with unified weights for the whole CU for intra and inter prediction combination

When combined inter-intra prediction is applied, the final prediction combines one intra prediction and one merge indexed prediction. In a merge CU, one flag is signaled for merge mode to select an intra mode from an intra candidate list when the flag is true. For luma component, the intra candidate list is derived from 4 intra prediction modes including DC, planar, horizontal, and vertical modes. For chroma component, DM is always applied without extra signaling. All intra modes use unified weights for the whole CU of (4:4).

## CE10.1.3.d: combined inter-intra prediction with PDPC skipped for intra planar prediction and modified weights

When combined inter-intra prediction is applied, the final prediction combines one intra prediction and one merge indexed prediction. In a merge CU, one flag is signaled for merge mode to select the usage of intra planar mode when the flag is true. For chroma component, DM is always applied without extra signaling. All intra modes use unified weights for the whole CU of (4:4), while low delay configuration is used (2:6) with weighting factor of intra prediction as 2. The PDPC step is skipped during intra prediction sample generation.

# Simulation results

The contribution has been implemented on top of VTM3.0. The test condition descripted in JVET-L1010 [1] is used for verifying the contribution. The summary results are provided in the following tables.

## CE10.1.3.a

On average, the BD-rate differences are -0.01% in RA, and 0.10% in LDB, respectively.

Table 1 Coding performance over VTM3.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.03% | 0.06% | 99% | 99% |
| Class A2 | -0.02% | 0.14% | 0.22% | 99% | 100% |
| Class B | -0.01% | 0.21% | 0.06% | 99% | 100% |
| Class C | 0.02% | 0.09% | 0.03% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.13% | 0.09% | 99% | 100% |
| Class D | -0.02% | -0.12% | 0.03% | 99% | 100% |
| Class F | 0.01% | -0.01% | 0.02% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.10% | 0.14% | 0.47% | 99% | 99% |
| Class C | 0.12% | 0.58% | 0.43% | 99% | 100% |
| Class E | 0.07% | 0.40% | 0.09% | 99% | 101% |
| Overall | 0.10% | 0.35% | 0.36% | 99% | 100% |
| Class D | 0.07% | 0.50% | 0.47% | 99% | 100% |
| Class F | -0.01% | -0.01% | 0.16% | 99% | 99% |

## CE10.1.3.b

On average, the BD-rate differences are 0.01% in RA, and 0.01% in LDB, respectively.

Table 2 Coding performance over VTM3.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.08% | -0.01% | 100% | 99% |
| Class A2 | 0.00% | -0.05% | 0.04% | 100% | 100% |
| Class B | 0.00% | 0.05% | -0.11% | 100% | 100% |
| Class C | 0.02% | 0.04% | 0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | -0.02% | 100% | 100% |
| Class D | 0.02% | 0.05% | 0.00% | 100% | 100% |
| Class F | 0.02% | -0.01% | -0.05% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.22% | -0.05% | 100% | 100% |
| Class C | 0.03% | 0.37% | 0.06% | 100% | 100% |
| Class E | -0.06% | -0.15% | -0.28% | 100% | 101% |
| Overall | 0.01% | -0.01% | -0.07% | 100% | 101% |
| Class D | 0.03% | 0.20% | 0.29% | 100% | 100% |
| Class F | 0.03% | -0.02% | 0.00% | 100% | 99% |

## CE10.1.3.c

On average, the BD-rate differences are 0.05% in RA, and 0.10% in LDB, respectively.

Table 3 Coding performance over VTM3.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.10% | 0.06% | 100% | 100% |
| Class A2 | 0.04% | 0.02% | 0.13% | 100% | 100% |
| Class B | 0.04% | 0.23% | 0.07% | 100% | 100% |
| Class C | 0.09% | 0.07% | 0.15% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.12% | 0.10% | 100% | 100% |
| Class D | 0.02% | -0.03% | -0.06% | 100% | 101% |
| Class F | 0.05% | 0.04% | 0.07% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.13% | 0.10% | 0.07% | 99% | 99% |
| Class C | 0.11% | 0.36% | 0.28% | 100% | 100% |
| Class E | 0.03% | 0.08% | -0.28% | 100% | 100% |
| Overall | 0.10% | 0.18% | 0.05% | 100% | 100% |
| Class D | 0.08% | 0.49% | 0.22% | 100% | 100% |
| Class F | 0.20% | 0.36% | 0.32% | 99% | 98% |

## CE10.1.3.d

On average, the BD-rate differences are 0.02% in RA, and 0.01% in LDB, respectively.

Table 4 Coding performance over VTM3.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.04% | -0.01% | 99% | 100% |
| Class A2 | 0.02% | 0.08% | 0.10% | 99% | 100% |
| Class B | 0.00% | 0.08% | -0.06% | 99% | 100% |
| Class C | 0.06% | 0.05% | 0.12% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.07% | 0.03% | 99% | 100% |
| Class D | 0.02% | -0.04% | 0.12% | 99% | 100% |
| Class F | -0.04% | -0.03% | 0.00% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.25% | 0.01% | 99% | 99% |
| Class C | 0.09% | 0.46% | 0.33% | 99% | 101% |
| Class E | -0.14% | 0.28% | 0.08% | 98% | 102% |
| Overall | 0.01% | 0.12% | 0.13% | 99% | 100% |
| Class D | 0.15% | 0.14% | -0.06% | 99% | 100% |
| Class F | -0.10% | 0.16% | -0.31% | 99% | 99% |

# Conclusions

This contribution reports the results of simplifications for combined inter-intra prediction. Compared to VTM3.0 anchor, the average BD-rate differences are-0.01%~0.05 in RA, and 0.01%~0.10 in LDB.

We recommend to accept the proposed methods in VVC VTM considering the negligible coding difference.

# Reference

[1]“JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring. JVET-L1010, Nov. 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).