| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0292 |
| --- | --- |

| Title: | CE 6-5.1: Reduced Secondary Transform (RST) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Mehdi Salehifar Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com mehdi.salehifar@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution, reports test results of CE 6-5.1 on reduced secondary transform (RST). According to the request of the previous 12th JVET meeting, all tests were performed without normative change of MTS signaling, which are described as the following:

- Test 1:  (A),

- Test 2:  (A) + (B),

- Test 3:  (A) + (B) + (C),

- Test 4:  (A) + (B) + (D),  where

| Feature | Description |
| --- | --- |
| (A) | 4 transform sets (instead of 35). 2 transforms per set. |
| (B) | Restricted RST to allow maximum 8 multiplications/pixel |
| (C) | RST is disabled for 4x4 TU. |
| (D) | 16x48 matrices are employed instead of 16x64 ones. |

The experimental results of all CE 6-5.1 tests are summarized as follows:

As Test 1 results, with 128% (AI), 108% (RA), and 104% (LD) encoding time compared to VTM anchor, BD-rate degradation is -1.59% (AI), -0.88% (RA), and -0.24% (LD).

As Test 2 results, with 128% (AI), 108% (RA), and 104% (LD) encoding time compared to VTM anchor, BD-rate difference is -1.40% (AI), -0.76% (RA), and -0.21% (LD).

As Test 3 results, with 125% (AI), 107% (RA), and 103% (LD) encoding time compared to VTM anchor, BD-rate degradation is -1.25% (AI), -0.69% (RA), and -0.18% (LD).

As Test 4 results, with 129% (AI), 107% (RA), and 104% (LD) encoding time compared to VTM anchor, BD-rate difference is -1.34% (AI), -0.71% (RA), and -0.18% (LD).

The memory usage of Test 1, Test 2, and Test 3 accounts to 10KB and that of Test 4 to 8KB.

Proposed Method

1.1) Reduced Secondary Transform (RST):

The RST was introduced in JVET-K0099 [1] and 4 transform set (instead of 35 sets) mapping introduced in JVET-L0133 [2], which is still used in CE6-5.1. In this contribution, 16x64 (or 16x48 for Test 4) and 16x16 matrices are employed for 8x8 and 4x4 blocks, respectively. For notational convenience, the 16x64 (or 16x48) transform is denoted as RST8x8 and the 16x16 one as RST4x4.

1.2) Simplification method:

The same simplification method as proposed in [2] is applied in this contribution, which limits worst case number of multiplications per pixel less than or equal to 8.

1.3) RST matrices of reduced dimension:

In Test 4, 16x48 matrices are applied instead of 16x64 ones with the same transform set configuration, each of which takes 48 input data from three 4x4 blocks in a top-left 8x8 block excluding right-bottom 4x4 block. With the help of the reduced dimension, memory usage for storing all RST matrices is reduced from 10KB to 8KB with reasonable performance drop.

Experimental results

The proposed method has been implemented based on VTM 3.0 reference software according to common test conditions defined in JVET-L1010 [3] and Core Experiment description in JVET-L1026 [4].

2.1) Test 1: (A)

(A) 4 transform sets and 2 transforms per set

2.1.1) Results with inter MTS off (CTC)

2.1.2) Inter MTS on

TBD

2.2) Test 2: (A) + (B)

(A) 4 transform sets and 2 transforms per set

(B) Simplification of max. 8 multiplications/pixel

2.2.1) Results with inter MTS off  (CTC)

2.2.1) Results with inter MTS on

2.3) Test 3: (A) + (B) + (C)

(A) 4 transform sets and 2 transforms per set

(B) Simplification of max. 8 multiplications/pixel

(C) RST disabled for 4x4 TU

2.3.1) Results with inter MTS off  (CTC)

2.3.1) Results with inter MTS on

2.4) Test 4: (A) + (B) + (D)

(A) 4 transform sets and 2 transforms per set

(B) Simplification of max. 8 multiplications/pixel

(D) 16x48 matrices

2.4.1) Results with inter MTS off  (CTC)

Conclusion

In the proposed RST, it is reportedly observed that BD-rate/encoding time/decoding time was -1.59%/128%/98% (AI), -0.88%/108%/100% (RA), and -0.24%/104%/101% (LD) for 4 transform set configuration of two RST candidates per set without normative change of MTS signaling. Also, -1.40%/128%/97% (AI), -0.76%/108%/100% (RA), and -0.21%/104%/98% (LD) were achieved for simplified configuration restricting maximum number of multiplications per pixel on top of the former one. As reported, substantial BD-rate is achieved with reasonable memory cost and encoding time increase. It is strongly recommended that the proposed RST should be adopted in the next VTM.

Reference

- M. Salehifar et al., “CE 6.2.6: Reduced Secondary Transform (RST)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K0099, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- M. Koo et al., “CE 6-2.1: Reduced Secondary Transform (RST)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L0133, 12th meeting, Macao, CN, 3-12 Oct. 2018.

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, the 12th JVET meeting, Oct. 2018, Macao, CN.

- A. Said and X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signalling”, JVET-L1026, the 12th JVET meeting, Oct. 2018, Macao, CN.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).