| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0298-v1 |
| --- | --- |

| Title: | CE11: Longer tap deblocking filter (test 11.1.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra, Biao Wang, Semih Esenlik, Han Gao, Jianle Chen | Tel: Email: | anand.meher.kotra@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution reports the results of CE 11 test 11.1.5 which uses a longer tap deblocking filter to reduce blocking artifacts which are observed at large block boundaries. Furthermore, to reduce line buffer requirements for the “longer tap” filter: for the horizontal edges which overlap with the CTU boundaries, the maximum number of samples used in filter decision and the maximum number of samples used in filter modification from the top block are restricted to be the same as in VTM 3.0 deblocking filter.

Objective results are as follows:

Over VTM 3.0 Anchor with ALF OFF (AI, RA, LDB, LDP): Luma BD-Rate of -0.01%, 0.00%, 0.09%, -0.06% is achieved without any increase in EncT and DecT.

Over VTM 3.0 Anchor with ALF ON (AI, RA, LDB, LDP): Luma BD-Rate of 0.00%, 0.02%, 0.13%, 0.02% is achieved without any increase in EncT and DecT.

# Introduction

The proposed method was first introduced in JVET-K0369 [4], and currently being evaluated as part of CE11. The proposal basically applies a longer tap filter for edges belonging to larger blocks. Further details about the longer tap filter can be found in [4]. Furthermore to avoid over smoothing, CE 11.1.5 also includes the position dependent adaptive Tc clipping range [4] for the longer tap filter. The Tc clipping range is modified using the following equation:  [Pi – (tc + (tc >> i)), Pi + (tc + (tc >> i))] where i is the sample distance from the edge.

# Complexity analysis

Complexity of the proposal is evaluated based on the following criteria described in the CE11 document, JVET-L1031 [1].

- Number of samples from the block boundary that can be modified by deblocking (P+Q):

  - For blocks whose width/height less than 16 samples: same as VTM 3.0. Therefore 3 +3.

  - For blocks whose width/height >= 16 samples: 7+7 for all edges, except : 3+ 7 for horizontal edges overlapping with CTU boundaries

- Number of samples from the block boundary that are needed for deblocking decisions:

  - For blocks whose width/height less than 16 samples: same as VTM 3.0. Therefore 4 samples.

  - For blocks whose width/height >= 16 samples: 7 samples on either side of the edge. p0 till p7, q0 till q7 for all edges, except for horizontal edges overlapping with CTU boundaries, where p3 till p7, q0 till q7 samples are used.

- Number of filtering operations: 84 Multiplication (out of which only 4 multiplications are non-power of 2 and rest of the multiplications can be implemented using bit shifts). 82 additions/subtractions.

  - The suggested implementation which can re-use filter operations is described as follows:

    - C = P3+ P2+ P1+ P0+ Q0+ Q1+ Q2+ 3 * Q3+ Q4+ Q5+ Q6+ 3 * Q7  is initially computed,

    - Then

    - Val = C + P4 + P5 + P6 + 2 * Q0  - 2 * Q3 – 2 * Q7,  Q0’= (Val + 8) >> 4;

    - Val = C + P4 + P5 +2 * Q1  - 2 * Q3 – 2 * Q7,      Q1’= (Val + 8) >> 4;

    - Val = C + P4 + 2 * Q2  - 2 * Q3 - Q7,             Q2’= (Val + 8) >> 4;

    - Val = C                                      Q3’= (Val + 8) >> 4;

    - Val = C - P3  - 2 * Q3 + 2 * Q4 + Q7,             Q4 ’= (Val + 8) >> 4;

    - Val = C - P3 – P2  - 2 * Q3 + 2 * Q5 + Q7,          Q5 ’= (Val + 8) >> 4;

    - Val = C - P3 – P2 – P1 - 2 * Q3 + 2 * Q6 + Q7,       Q6 ’= (Val + 8) >> 4;

The computations for the “P” side, could also be computed using the same approach as described above. Once the value of C is computed, all the filter operations can also be performed in parallel.

- Number of line buffers: 4 lines (same as VTM 3.0)

- Parallelization capabilities: same as VTM 3.0. All edges overlapping on 8 x 8 grid can be deblocked in parallel. Longer tap filter is only applied when both blocks sharing the edge have a size >=16 samples.

- Longer tap filter is applied only for the Luma component

# Experimental results

The proposed method is implemented on top of VTM-3.0 [2] and the simulation is performed under common test conditions as described in JVET-L1010 [5]. The results are presented for both ALF ON and ALF OFF conditions as suggested to be reported by the CE11 description document [1].

# Conclusion

This contribution presents the results for CE test 11.1.5. The proposed technique in CE 11.1.5 uses a longer tap deblocking filter which is applied for boundaries belonging to larger blocks. The proposal also applies position dependent Tc clipping to avoid over smoothing by the longer tap filter. Moreover, the line buffer requirements are kept the same as in the deblocking filter of VTM 3.0. Objective results show that the proposed technique does not results in BD-Rate loss, while substantially removing blocking artifacts belonging to large blocks. It is suggested to adopt this technique to the VVC working draft specification and the VTM reference software.

# References

- Description of Core Experiment 9 (CE11): Deblocking, Document JVET-L1031, A. Norkin, A.M. Kotra

- Versatile Video Coding (Draft 3), Document JVET-L01001_v10, B. Bross, J. Chen, S. Liu

- CE2-related: Longer Tap Deblocking Filter”, JVET-K0369

- CE11-related: Position dependent adaptive Tc clipping range for deblocking filter, JVET-L0226

- “JVET common test conditions and software reference configurations”, JVET-L1010

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).