| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0302 |
| --- | --- |

| Title: |  | CE2: Merge Mode with Regression-based Motion Vector Field (Test 2.3.3) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Status: |  | Input document to JVET |  |  |  |
| Purpose: |  | Proposal |  |  |  |
| Author(s) or Contact(s): |  | Ramin Ghaznavi-Youvalari Alireza Aminlou Jani Lainema |  | Emails: | ramin.ghaznavi-youvalari@nokia.com alireza.aminlou@nokia.com jani.lainema@nokia.com |
| Source: |  | Nokia |  |  |  |

_____________________________

# Abstract

This document reports the results for CE 2.3.3 Tests (a) – (d). The Regression-based Motion Vector Field (RMVF) tool, that was proposed in JVET L0171, models motion behavior of a block using motion vectors of the neighboring blocks by a linear model. RMVF tool is used as new merge mode in the VTM-3.0 software. When the mode is selected, motion vectors area generated using the linear model for 8×8 sub-blocks in order to provide sub-PU level motion granularity. Various tests have been conducted as follows.

Tests 2.3.3.a is the same as in the JVET L0171 contribution. This test provides on average -0.18%, -0.02% BD-Rate impact in RA and LDB configurations. The encoding and decoding runtimes are 105% and 102% for RA, and 106% and 102% for LDB, respectively.

In Test 2.3.3.b, in addition to Test 2.3.3.a setup, the RMVF-coded blocks are used for affine merge motion vector inheritance. This test provides -0.20% and -0.02% BD-Rate improvements for RA and LDB configurations, respectively. In this test, the encoding and decoding runtimes are 104% and 101% for RA, and 103% and 102% for LDB configuration.

Test 2.3.3.c is the same as Test 2.3.3.a, but only portion of the neighboring motion information (every other 4x4 sub-block) is used for RMVF parameter derivation. This test provides on average -0.19% and 0.01% BD-Rate impact for RA and LDB configurations, respectively. The encoding and decoding runtimes were observed to be 105% and 102% for RA, and 106% and 101% for LDB.

Test 2.3.3.d is the combination of Tests 2.3.3.b and 2.3.3.c. This test provides on average -0.19% and 0.00%, impact for RA and LDB configurations, respectively. Additionally, the encoding and decoding runtimes are 105% and 100% for RA, 106% and 101% for LDB.

# Introduction

Motion behavior may vary inside a block; hence it is not efficient to derive only one motion vector for each block. This appears to be important particularly for larger blocks. Thus, calculating motion vectors in a finer granularity in each block can provide a smooth and more realistic motion vector field inside that block. To that end, the RMVF tool is proposed in this contribution in order to model such motion behavior based on the motion vectors of the spatially neighboring blocks.

# Proposed Method

In order to provide a fine granularity of motion vectors inside a block, Regression based Motion Vector Field (RMVF) tool is proposed in JVET L0171. This tool attempts to model the motion vectors of each block on a sub-block level based on the spatially neighboring motion vectors.

Figure 1 illustrates the neighboring 4×4 motion blocks that are used for motion parameter derivation of the proposed method. As seen, one line and row of immediate neighboring motion vectors in 4×4 sub-block basis (and their center locations) from each side of the block are used in the regression process. This approach is used in tests (a) and (b).

In order to reduce the number of neighboring motion information for RMVF parameter derivation, the method of Figure 2 is used in which almost half of the neighboring 4×4 motion blocks are used for motion parameter derivation. This is used in tests (c) and (d).

Figure 1. Neighboring motion vectors that are used in RMVF motion parameter derivation.

Figure 2. Reduced neighboring motion vector candidates for RMVF motion parameter derivation.

Five conventional regions (bottom-left, left, top-left, top, top-right) as shown in the figures are used when collecting the motion information for motion parameter derivation. The above-right and bottom-left reference motion regions are limited to only half of the corresponding width or height of the current block.

In RMVF mode, motion of the block is defined by a 6-parameter motion model. These parameters axx, axy, ayx, ayy, bx and by are calculated by solving a linear regression model in mean square error (MSE) sense. The input to the regression model consists of the center locations (x, y) and motion vectors (mvx and mvy) of the available neighboring 4×4 sub-blocks as defined above.

Motion vector (MVX_subPU, MVY_subPU) for an 8×8 sub-block with center location at (XsubPU, YsubPU) is then calculated as:

MVX_subPUMVY_subPU= axxaxyayxayyXsubPUYsubPU+ bxby

The motion vectors are calculated for 8×8 sub-blocks relative to the center location of each sub-block. Consequently, motion compensation is also applied in 8×8 sub-block accuracy in the RMVF mode.

In order to have efficient modelling for the motion vector field, RMVF tool is applied only in the cases that at least one motion vector from at least three of the candidate regions are available.

Further details:

- The minimum coding unit size for RMVF mode is 8×8

- Decision for uni- or bi-directional prediction is done based on the picture type (P/B picture) and availability of neighboring motion information for that prediction direction.

- Reference index in both prediction directions is set to 0.

- When calculating the motion parameters, the conventional motion vector scaling used in VTM-3.0 is applied to the motion vectors that have different reference pictures than the current block.

- The sub-block motion compensation is done as in VTM-3.0 (i.e., 8×8 sub-block size).

Different schemes have been considered for testing the proposed RMVF method:

- The RMVF method used as a separate merge mode as described in above (i.e. same as JVET L0171) (CE 2.3.3. a)

- CE 2.3.3. a, and the RMVF-coded blocks are used for affine merge candidate inheritance (CE 2.3.3.b).

- The RMVF method used as a separate merge mode (i.e. CE 2.3.3. a), but for RMVF parameter derivation, every other 4×4 sub-block motion information (Figure 2) is used (CE 2.3.3. c).

- Combination of Test (b) and Test (c), (CE 2.3.3. d)

# Test results

The experiments were performed under JVET common test conditions. BD-Rate results of the RMVF tool on top of the VTM-3.0 compared to the anchor with RA and LDB configurations are shown in following tables. The full results are provided in the attached excel files.

Table 1. Summary results of CE2.3.3 tests

| Test | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT. | DecT. | Y | U | V | EncT. | DecT. |
| CE2.3.3. a | -0.18% | -0.17% | -0.26% | 105% | 102% | -0.02% | -0.10% | 0.09% | 106% | 102% |
| CE2.3.3. b | -0.20% | -0.25% | -0.24% | 104% | 101% | -0.02% | 0.12% | 0.21% | 103% | 102% |
| CE2.3.3. c | -0.19% | -0.13% | -0.19% | 105% | 102% | 0.01% | 0.05% | -0.03% | 106% | 101% |
| CE2.3.3. d | -0.19% | -0.22% | -0.23% | 105% | 100% | 0.00% | 0.07% | -0.16% | 106% | 101% |

Table 2. Results for Test CE2.3.3. a

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | -0.32% | -0.42% | 105% | 104% |
| Class A2 | -0.28% | -0.24% | -0.29% | 107% | 102% |
| Class B | -0.16% | -0.11% | -0.24% | 106% | 100% |
| Class C | -0.05% | -0.09% | -0.14% | 105% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.18% | -0.17% | -0.26% | 105% | 102% |
| Class D | -0.04% | -0.22% | -0.10% | 104% | 102% |
| Class F (optional) | 0.03% | -0.02% | -0.07% | 106% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | -0.19% | -0.07% | 106% | 102% |
| Class C | 0.00% | 0.10% | 0.34% | 106% | 102% |
| Class E | 0.06% | -0.22% | 0.03% | 106% | 103% |
| Overall | -0.02% | -0.10% | 0.09% | 106% | 102% |
| Class D | 0.05% | 0.46% | 0.39% | 105% | 99% |
| Class F (optional) | 0.07% | 0.25% | -0.37% | 106% | 107% |

Table 3. Results for Test CE2.3.3. b

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.40% | -0.33% | 104% | 104% |
| Class A2 | -0.29% | -0.39% | -0.25% | 106% | 102% |
| Class B | -0.19% | -0.17% | -0.25% | 104% | 99% |
| Class C | -0.06% | -0.13% | -0.16% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.20% | -0.25% | -0.24% | 104% | 101% |
| Class D | -0.08% | -0.20% | -0.19% | 103% | 100% |
| Class F (optional) | 0.03% | -0.01% | -0.02% | 104% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.12% | 0.06% | 105% | 103% |
| Class C | 0.02% | 0.24% | 0.29% | 101% | 102% |
| Class E | 0.01% | 0.35% | 0.37% | 103% | 103% |
| Overall | -0.02% | 0.12% | 0.21% | 103% | 102% |
| Class D | 0.08% | 0.36% | 0.41% | 103% | 99% |
| Class F (optional) | 0.11% | -0.04% | -0.10% | 102% | 104% |

Table 4. Results for Test CE2.3.3. c

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.26% | -0.36% | 104% | 104% |
| Class A2 | -0.29% | -0.32% | -0.30% | 107% | 103% |
| Class B | -0.16% | -0.07% | -0.16% | 105% | 100% |
| Class C | -0.03% | 0.02% | -0.02% | 105% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.19% | -0.13% | -0.19% | 105% | 102% |
| Class D | -0.05% | 0.01% | -0.02% | 104% | 102% |
| Class F (optional) | 0.00% | -0.05% | -0.04% | 106% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.24% | 0.04% | 105% | 102% |
| Class C | 0.01% | 0.20% | 0.05% | 105% | 100% |
| Class E | 0.11% | 0.32% | -0.25% | 106% | 100% |
| Overall | 0.01% | 0.05% | -0.03% | 106% | 101% |
| Class D | 0.05% | 0.15% | 0.24% | 105% | 98% |
| Class F (optional) | 0.20% | 0.06% | 0.03% | 106% | 104% |

Table 5. Results for Test CE2.3.3. d

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.37% | -0.31% | 104% | 103% |
| Class A2 | -0.30% | -0.41% | -0.29% | 106% | 102% |
| Class B | -0.16% | -0.14% | -0.20% | 106% | 97% |
| Class C | -0.02% | -0.06% | -0.17% | 105% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.19% | -0.22% | -0.23% | 105% | 100% |
| Class D | -0.04% | -0.11% | -0.20% | 104% | 101% |
| Class F (optional) | 0.03% | -0.05% | 0.02% | 105% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | -0.20% | -0.21% | 106% | 101% |
| Class C | 0.02% | 0.31% | 0.16% | 105% | 102% |
| Class E | 0.09% | 0.20% | -0.51% | 106% | 101% |
| Overall | 0.00% | 0.07% | -0.16% | 106% | 101% |
| Class D | 0.08% | 0.44% | 0.03% | 105% | 98% |
| Class F (optional) | 0.09% | 0.15% | 0.44% | 106% | 107% |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).