| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0305 |
| --- | --- |

| Title: | CE7-related: Joint coding of chrominance residuals |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document proposes a chrominance residual coding mode where a single joint residual block is used to describe the residuals of both Cb and Cr blocks in the same transform unit. When joint residual mode is active, the indicated joint residual is added to the Cb prediction block and deducted from the Cr prediction block. Bitstream syntax and decoding process for the joint residual follow those of the Cb residual in VTM-3.  In the encoder side the algorithm uses average of the positive Cb residual and negative Cr residual as the input to the transform and quantization process.

It is reported that the proposed tool provides in AI configuration -0.29 %, -0.79 %, -2.16 % DB-rate impact for Y, U and V components, respectively. In RA configuration the BD-rate impacts are -0.19 %, -1.44 % and -2.00 %, in LD-B the BD-rate impacts are -0.08 %, -2.38 % and -4.83 %, and in LD-P configuration the BD-rate impacts are -0.11 %, -2.46 % and -5.08 % for Y, U and V components, respectively.

It is further reported that the decoder runtimes are practically not affected in AI and RA, and somewhat improved in LD-B and LD-P configurations. The decoding process for the joint residual mode is asserted to be less complex than decoding individual residuals for the chrominance channels. Reported encoder runtimes are 2-3 % higher than the anchor runtimes, but it is noted that the encoder used in the experiments did not try to make “smart decisions” to speed-up the process.

# Introduction

As Cb and Cr residuals appear to correlate inversely with each other, it is proposed to take advantage of this phenomenon and introduce a mode for joint coding of the chrominance residuals. In this mode there is a single residual indicated for the two chrominance blocks of a transform unit. The indicated residual is added to the prediction block in the first channel (typically representing Cb) and deducted from the prediction block in the second channel (typically representing Cr).

# Proposed tool

The joint residual mode is indicated with a flag in the bitstream if the coded block flags (cbf) for both Cb and Cr are true. If the mode is activated, a single residual block is decoded. The bitstream syntax and decoding process of joint residual blocks follow those of the Cb residual in VTM-3. The residuals of the Cr blocks are generated by negating the decoded joint residual. As a single residual is used to represent residuals of two blocks, the chroma QP offset parameter is reduce by 2 when the joint chrominance residual mode is active.

The required software changes for the VTM DecoderLib are given below:

New function:

void CABACReader::joint_cb_cr( TransformUnit& tu )

{

tu.jointCbCr = m_BinDecoder.decodeBin( Ctx::JointCbCrFlag( 0 ) );

}

Added in CABACReader::residual_coding:

// Joint Cb-Cr residual mode is signaled if both Cb and Cr cbfs are true

if ( compID == COMPONENT_Cr && TU::getCbf( tu, COMPONENT_Cb ) )

{

joint_cb_cr( tu );

// No Cr residual in bitstream in joint Cb-Cr residual mode

if ( tu.jointCbCr )

return;

}

Added in DecCu::xIntraRecBlk and DecCu::xDecodeInterTU:

if ( tu.jointCbCr && compID == COMPONENT_Cr )

{

// Cr uses negative of the signalled Cb residual

for (int y = 0; y < piResi.height; y++)

for (int x = 0; x < piResi.width; x++)

crResiPel[y*crStride + x] = -cbResiPel[y*cbStride + x];

}

else

On the encoder side the average of positive Cb residual and negative Cr residual are used as the joint residual:

resJoint = (resCb – resCr) / 2

It is noted that there are nice possibilities for encoder speed-ups by checking how residuals behave with respect to each other and avoiding entering transform and RDOQ stages when joint residual is unlikely to be the best coding alternative.

# Proposed text

## Syntax and semantics

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| if( cIdx == 2 && tu_cbf_cb[ x0 ][ y0 ] ) { |  |
| tu_cb_cr_joint_residual[ x0 ][ y0 ] | ae(v) |
| if( tu_cb_cr_joint_residual [ x0 ][ y0 ] ) { |  |
| return |  |
| } |  |
| … |  |

tu_cb_cr_joint_residual[ x0 ][ y0 ] specifies whether indicated Cb residual is used to derive both Cb and Cr residuals. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. tu_cb_cr_joint_residual[ x0 ][ y0 ] equal to 1 specifies that the indicated Cb residual is used to derive the Cr residual. tu_cb_cr_joint_residual[ x0 ][ y0 ] equal to 0 specifies that Cr residual may be present in the bitstream depending on other syntax elements. When tu_cb_cr_joint_residual[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

## Decoding process

8.5.2	Scaling and transformation process

…

The (nTbW)x(nTbH) array of residual samples resSamples is derived as follows:

  - If cIdx is equal to 2 and tu_cb_cr_joint_residual[ xTbY ][ yTbY ] is equal to 1, the residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

resSamples[ x ][ y ] = -resSamplesCb[ x ][ y ]

where resSamplesCb[ x ][ y ] is the residual sample array generated for a chrominance block with the transform block location ( xTbY, yTbY ) and cIdx equal to 1.

Otherwise:

…

### 8.5.3 Scaling process for transform coefficients

…

The quantization parameter qP is derived as follows:

- If cIdx is equal to 0, the following applies:

qP = Qp′Y

- Otherwise, if cIdx is equal to 1 and tu_cb_cr_joint_residual[ xTbY ][ yTbY ] is equal to 1, the following applies:

qP = Qp′Cb - 2

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Qp′Cb

- Otherwise (cIdx is equal to 2), the following applies:

qP = Qp′Cr

…

# Coding efficiency

The following tables summarize the coding efficiency of the proposed mode. Detailed simulation results are provided in the attached Excel sheets. In the extra tests table 100 frame AI encodings are reported. QP set for the low QP case is {12, 17, 22, 27} and for the high QP case it is {32, 37, 42, 47}.

| CTC | Y | U | V | EncTime | DecTime |
| --- | --- | --- | --- | --- | --- |
| AI | -0.29 % | -0.79 % | -2.16 % | 102 % | 100 % |
| RA | -0.19 % | -1.44 % | -2.00 % | 103 % | 100 % |
| LD-B | -0.08 % | -2.38 % | -4.83 % | 102 % | 94 % |
| LD-P | -0.11 % | -2.46 % | -5.08 % | 104 % | 95 % |

| Extra tests | Y | U | V | Ave-UV |
| --- | --- | --- | --- | --- |
| AI/low QP | -0.29 % | 0.38 % | -0.69 % | -0.16 % |
| AI/high QP | -0.41 % | -1.42 % | -2.63 % | -2.03 % |

|  |  |  | AI Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17 % | -1.00 % | 0.15 % | 100 % | 100 % |
| Class A2 | -0.52 % | -1.71 % | -0.22 % | 104 % | 101 % |
| Class B | -0.20 % | -0.66 % | -2.84 % | 100 % | 99 % |
| Class C | -0.30 % | -1.14 % | -3.67 % | 103 % | 100 % |
| Class E | -0.33 % | 0.57 % | -3.26 % | 101 % | 101 % |
| Overall | -0.29 % | -0.79 % | -2.16 % | 102 % | 100 % |
| Class D | -0.25 % | -1.28 % | -3.21 % | 102 % | 100 % |
| Class F | -0.36 % | -1.84 % | -3.79 % | 103 % | 101 % |

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11 % | -1.74 % | 0.26 % | 104 % | 99 % |
| Class A2 | -0.33 % | -2.28 % | -0.41 % | 104 % | 98 % |
| Class B | -0.15 % | -1.06 % | -3.15 % | 102 % | 101 % |
| Class C | -0.18 % | -1.05 % | -3.47 % | 103 % | 100 % |
| Class E |  |  |  |  |  |
| Overall | -0.19 % | -1.44 % | -2.00 % | 103 % | 100 % |
| Class D | -0.13 % | -1.29 % | -3.03 % | 102 % | 99 % |
| Class F | -0.43 % | -1.79 % | -3.46 % | 103 % | 99 % |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.14 % | -3.01 % | -4.12 % | 102 % | 94 % |
| Class C | -0.04 % | -2.97 % | -4.85 % | 102 % | 94 % |
| Class E | -0.03 % | -0.55 % | -5.98 % | 101 % | 95 % |
| Overall | -0.08 % | -2.38 % | -4.83 % | 102 % | 94 % |
| Class D | -0.05 % | -3.73 % | -4.14 % | 102 % | 96 % |
| Class F | -0.26 % | -3.00 % | -4.05 % | 103 % | 99 % |

|  |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.14 % | -2.79 % | -4.24 % | 104 % | 95 % |
| Class C | -0.07 % | -3.13 % | -5.11 % | 103 % | 94 % |
| Class E | -0.11 % | -1.01 % | -6.43 % | 103 % | 96 % |
| Overall | -0.11 % | -2.46 % | -5.08 % | 104 % | 95 % |
| Class D | -0.06 % | -3.86 % | -4.32 % | 104 % | 95 % |
| Class F | -0.17 % | -3.85 % | -4.70 % | 106 % | 93 % |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).