| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0315 |
| --- | --- |

| Title: | Non-CE4: MMVD scaling simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Ru-ling Liao Chong Soon Lim | Tel: Email: | +65-6550-5423 Jingya.li@sg.panasonic.com Chongsoon.lim@sg.panasoni.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to simplify MV offset calculation process for MMVD. In this contribution, scaling process of generating MMVD MV offset is removed and flipping process is kept. In addition, a flag is added to SPS to indicate whether MMVD tool is used.

Experimental results show -0.01% (RA) and 0.08% (LDB) luma BD-rate change Ignorable change in encoding and decoding time is observed.

# Algorithm description

Scaling process in MMVD that adjusts MV offset according to distance to two reference pictures is complex and unnecessary. This contribution proposes to remove scaling process for generating MMVD MV offset to reduce complexity and save cycle.

Additionally, a flag is added to SPS to indicate whether MMVD tool is used.

# Experimental Results

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.03% | 99% | 99% |
| Class A2 | 0.00% | -0.01% | 0.03% | 100% | 97% |
| Class B | -0.01% | 0.03% | 0.01% | 100% | 97% |
| Class C | -0.01% | -0.01% | 0.03% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.00% | 0.03% | 100% | 98% |
| Class D | -0.02% | -0.03% | 0.01% | 100% | 99% |
| Class F | -0.01% | -0.01% | -0.02% | 100% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.09% | -0.14% | -0.16% | 100% | 103% |
| Class C | 0.09% | 0.21% | 0.10% | 100% | 104% |
| Class E | 0.03% | 0.15% | -0.05% | 99% | 104% |
| Overall | 0.08% | 0.05% | -0.05% | 100% | 104% |
| Class D | 0.15% | 0.30% | 0.19% | 100% | 102% |
| Class F | 0.10% | -0.04% | 0.07% | 100% | 104% |

# Conclusions

In this contribution, a simplification of MV offset calculation process for MMVD is proposed. It is reported that the proposed solution introduces -0.01% (RA) and 0.08% (LDB) luma BD-rate change with negligible impact on encoding and decoding time. We recommend to adopt the proposed solution into the next version of VVC.

# References

- M. W. Park, Y. Piao, M. Park, K. Choi, “CE4 Ultimate motion vector expression,” JVET-L0054, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

Additional information:

Results for removing both scaling process and flipping process for generation MMVD offset is as below:

0.45% (RA) and 0.08% (LDB) luma BD-rate change. Ignorable change in encoding and decoding time is observed.

Table 1. results of test 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.35% | 0.23% | 0.31% | 98% | 99% |
| Class A2 | 0.80% | 0.73% | 0.72% | 99% | 98% |
| Class B | 0.48% | 0.31% | 0.31% | 98% | 97% |
| Class C | 0.23% | 0.16% | 0.28% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.45% | 0.34% | 0.38% | 99% | 98% |
| Class D | 0.62% | 0.29% | 0.29% | 99% | 97% |
| Class F | 0.17% | 0.13% | 0.18% | 98% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.09% | -0.14% | -0.16% | 99% | 100% |
| Class C | 0.09% | 0.21% | 0.10% | 100% | 100% |
| Class E | 0.03% | 0.15% | -0.05% | 99% | 100% |
| Overall | 0.08% | 0.05% | -0.05% | 99% | 100% |
| Class D | 0.15% | 0.30% | 0.19% | 100% | 101% |
| Class F | 0.10% | -0.04% | 0.07% | 100% | 100% |

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD change

8.3.2.7 Derivation process for merge motion vector difference

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

Outputs of this process are the luma merge motion vector differences in 1/16 fractional-sample accuracy mMvdL0 and mMvdL1.

The variable currPic specifies the current picture.

The luma merge motion vector differences mMvdL0 and mMvdL1 are derived as follows:

- If both predFlagL0 and predFlagL1 are equal to 1, the following applies:

currPocDiffL0  =  DiffPicOrderCnt( currPic, RefPicList0[ refIdxL0 ] )	(8-272)

currPocDiffL1  =  DiffPicOrderCnt( currPic, RefPicList1[ refIdxL1 ] )	(8-273)

- If −currPocDiffL0 * currPocDiffL1 is greater than 0, the following applies:

mMvdL0[ 0 ]  =  MmvdOffset[ 0 ]		(8-274)

mMvdL0[ 1 ]  =  MmvdOffset[ 1 ]		(8-275)

mMvdL1[ 0 ]  =  −MmvdOffset[ 0 ]		(8-276)

mMvdL1[ 1 ]  =  −MmvdOffset[ 1 ]		(8-277)

  - If currPocDiffL0 is equal to currPocDiffL1Otherwise ( −currPocDiffL0 * currPocDiffL1 is less than 0 ), the following applies:

mMvdL0[ 0 ]  =  MmvdOffset[ 0 ]		(8-278)

mMvdL0[ 1 ]  =  MmvdOffset[ 1 ]		(8-279)

mMvdL1[ 0 ]  =  MmvdOffset[ 0 ]		(8-280)

mMvdL1[ 1 ]  =  MmvdOffset[ 1 ]		(8-281)

  - Otherwise if Abs( currPocDiffL0 ) is greater than Abs( currPocDiffL1 ), the following applies:

td = Clip3( −128, 127, currPocDiffL0 )		(8-282)

tb = Clip3( −128, 127, currPocDiffL1 )		(8-283)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(8-284)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-285)

mMvdL0[ 0 ]  =  MmvdOffset[ 0 ]		(8-278)

mMvdL0[ 1 ]  =  MmvdOffset[ 1 ]		(8-279)

mMvdL1[ 0 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL0L1[ 0 ] )  *  	(8-286)
	( ( Abs( distScaleFactor * mMvdL0L1[ 0 ] ) + 127 )  >>  8 ) )

mMvdL1[ 1 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL0L1[ 1 ] )  *  	(8-287)
	( ( Abs( distScaleFactor * mMvdL0L1[ 1 ] ) + 127 )  >>  8 ) )

- Otherwise if Abs( currPocDiffL0 ) is less than or equal to Abs( currPocDiffL1 ), the following applies:

td = Clip3( −128, 127, currPocDiffL1 )		(8-288)

tb = Clip3( −128, 127, currPocDiffL0 )		(8-289)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(8-290)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-291)

mMvdL1[ 0 ]  =  MmvdOffset[ 0 ]		(8-280)

mMvdL1[ 1 ]  =  MmvdOffset[ 1 ]		(8-281)

mMvdL0[ 0 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL1L0[ 0 ] )  *  	(8-292)
	( ( Abs( distScaleFactor * mMvdL1L0[ 0 ] ) + 127 )  >>  8 ) )

mMvdL0[ 1 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL1L0[ 1 ] )  *  	(8-293)
	( ( Abs( distScaleFactor * mMvdL1L0[ 1 ] ) + 127 )  >>  8 ) )

- Otherwise ( predFlagL0 or predFlagL1 are equal to 1 ), the following applies for X being 0 and 1:

mMvdLX[ 0 ] = ( predFlagLX = = 1 )  ?  MmvdOffset[ 0 ]  :  0	(8-294)

mMvdLX[ 1 ] = ( predFlagLX = = 1 )  ?  MmvdOffset[ 1 ]  :  0	(8-295)