| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0326_v2 |
| --- | --- |

| Title: | CE8-related: Remove the redundancy of CPR-related syntax coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shurui Ye Fangdong Chen Li Wang | Email: | yeshurui@hikvision.com chenfangdong@hikvision.com wangli7@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

This document proposes to remove the redundancy of CPR-related syntax coding. CPR-P frame is called for one frame where the current picture is the only reference picture. In fact, it is not allowed to use MMVD (Merge with MVD), Affine, merge triangle mode or Combined inter merge / intra prediction (CIIP) mode in the CPR-P frame. Therefore, this contribution proposes to remove those unused tools related syntax coding for CPR blocks in the CPR-P frame. By doing so, a small gain for luma BD rate of 0.01% was observed on AI configuration.

Introduction

Current picture referencing (CPR) was adopted into VTM in last JVET meeting. In the VTM3.0, the current (partially) decoded picture is considered as a reference picture. This current picture is put in the last position of reference picture list 0. Therefore, for a frame using the current picture as the only reference picture, it is a P frame. The bitstream syntax follows the same syntax structure for inter coding while the decoding process is unified with inter coding. The only outstanding difference is that the block vector (which is the motion vector pointing to the current picture) always uses integer-pel resolution.

Proposed method

It is proposed to remove the redundancy of CPR-related syntax coding. For CPR-P frame, some inter tools like MMVD, Affine, merge triangle mode and CIIP are disabled, but the related syntax are signaled. The proposed method is to remove MMVD, Affine, merge triangle mode and CIIP mode syntax coding for CPR blocks in CPR-P frame. All involved syntax elements are showed in Table 1.

And for non-CPR-P frame, the redundancy of CPR-related syntax could not be removed. Because a block cannot be verified whether using CPR mode before parsing those syntax elements. In [1], we propose to add a CPR flag to indicate if current block uses CPR mode. By doing that, the redundancy can be further removed.

Table 1. Redundancy of CPR-related syntax

|  | Related Syntax Elements |
| --- | --- |
| 1 | mmvd_flag(mmvd_skip, mmvd_merge) |
| 2 | inter_affine_flag |
| 3 | merge_subblock_flag |
| 4 | ciip_flag |
| 5 | merge_triangle_flag |

Proposed specification

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) { |  |
| if( treeType ! = DUAL_TREE_CHROMA ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| pred_mode_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( pcm_enabled_flag && cbWidth >= MinIpcmCbSizeY && cbWidth <= MaxIpcmCbSizeY && cbHeight >= MinIpcmCbSizeY && cbHeight <= MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( y0 % CtbSizeY ) > 0 ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if (intra_luma_ref_idx[ x0 ][ y0 ] = = 0) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else if( treeType ! = DUAL_TREE_CHROMA ) { /* MODE_INTER */ |  |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ¦ ¦ cu_skip_flag[ x0 ][ y0 ]) { |  |
| merge_data( x0, y0, cbWidth, cbHeight ) |  |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if(!CPR-P frame && sps_affine_enabled_flag && cbWidth >= 16 && cbHeight >= 16 ) { |  |
| inter_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_type_flag && inter_affine_flag[ x0 ][ y0 ] ) |  |
| cu_affine_type_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| …. |  |

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if (!CPR-P frame) { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if(MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if(sps_mh_intra_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| mh_intra_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| mh_intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| mh_intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if(sps_triangle_enabled_flag && slice_type = = B && cbWidth * cbHeight >= 16 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) |  |
| merge_triangle_idx[ x0 ][ y0 ] | ae(v) |
| else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } else { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |

Simulation results

The proposed method is integrated on VTM3.0. All tests were conducted under the common test condition [2]. Results are shown in the Table 1.

Table 1. Remove the redundancy of CPR-related syntax coding

|  | Over VTM3.0 with CPR enabled (AI configuration) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.00% | 99% | 98% |
| Class A2 | 0.00% | -0.02% | 0.03% | 99% | 96% |
| Class B | 0.00% | -0.04% | -0.02% | 100% | 101% |
| Class C | -0.01% | -0.02% | 0.01% | 101% | 102% |
| Class E | -0.02% | 0.04% | -0.08% | 100% | 99% |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Class D | -0.03% | 0.05% | -0.14% | 101% | 102% |
| Class F | -0.02% | -0.01% | 0.07% | 100% | 101% |
| Class SCC | -0.08% | -0.06% | -0.08% | 100% | 99% |
|  | Over VTM3.0 with CPR enabled (RA configuration) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.06% | 100% | 99% |
| Class A2 | -0.01% | 0.05% | -0.09% | 99% | 98% |
| Class B | 0.00% | -0.06% | -0.13% | 100% | 101% |
| Class C | 0.01% | -0.02% | -0.15% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.09% | 100% | 100% |
| Class D | 0.07% | 0.13% | 0.15% | 100% | 99% |
| Class F | -0.03% | 0.00% | -0.01% | 100% | 101% |
| Class SCC | -0.07% | -0.10% | -0.07% | 100% | 103% |
|  | Over VTM3.0 with CPR enabled (LDB configuration) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.28% | 0.42% | 100% | 98% |
| Class C | -0.01% | -0.01% | 0.10% | 100% | 99% |
| Class E | -0.16% | 0.66% | 0.39% | 99% | 100% |
| Overall | -0.05% | 0.28% | 0.30% | 100% | 99% |
| Class D | -0.01% | 0.29% | 0.52% | 100% | 103% |
| Class F | -0.13% | -0.13% | 0.33% | 99% | 96% |
| Class SCC | 0.02% | 0.03% | 0.01% | 99% | 98% |

Conclusion

This contribution proposes to remove some unnecessary syntax coding for CPR-related syntax coding, including MMVD, Affine, merge triangle mode and CIIP mode syntax coding. This results in a simplified CPR syntax coding with a little gain (0.01%) in AI configuration. Furthermore, the proposed method improves class SCC luma BD-rate by 0.08% in AI configurations.

- Reference

- S. Ye, F. Chen, L. Wang, “CE8-related: A new CPR syntax scheme”, JVET-M0327, Marrakech, MA, 9-18 Jan. 2019.

- F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

Hikvision may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).