| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0332 |
| --- | --- |

| Title: | CE8: Block vector prediction for CPR (test 8.1.1a and test 8.1.1b) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | junghak.nam@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports the CE test results for CE8.1.1a and CE8.1.1b. In CE8.1.1a, alternative candidates using default positions are tested. In CE8.1.1b, alternative candidates using middle positions are tested. From experiment results, following BD rate changes are observed over VTM-3.0 with CPR:

For CE8.1.1a:

- AI: -0.07% (CTC), -0.41% (ClassF), -1.45% (SCC).

- RA: -0.03% (CTC), -0.27% (ClassF), -0.96% (SCC).

For CE8.1.1b:

- AI: -0.06% (CTC), -0.40% (Class F), -1.45% (SCC)

- RA: -0.04% (CTC), -0.24% (Class F), -0.86% (SCC).

Introduction

In JVET-L0159 [1], alternative candidates for block vector prediction in CPR mode were proposed. Zero candidate within MVP and merge lists for CPR mode can be inadequate because block vector of CPR mode must be larger than width and height of the current coding block. In order to replace zero candidate, alternative candidates using default positions and middle positions are tested.

Proposed method

When CPR method is used, two alternative candidates are added for both MVP and merge mode. For MVP mode, when reference picture of current coding block indicated by reference index is same as current picture and the number of candidates in the constructed list is smaller than the maximum number of candidates, alternative candidates are inserted into MVP candidate list. Before alternative candidates are added, each candidate in the MVP candidate list can be removed if the candidate is not within current CTU. For Merge mode, when the current picture exists in the reference picture list and the number of candidates in the constructed list is smaller than the maximum number of candidates, alternative candidates are added into merge candidate list. If a neighboring candidate is coded by CPR mode and doesn’t exist within current CTU, the candidate is not inserted into merge candidate list.

If only one candidate for MVP and merge candidate list is allowed, the first one of default candidates is added. The maximum number of candidates for MVP and merge mode are unchanged and there is no syntax changes introduced by the proposed method also.

CE 8.1.1a: Alternative candidates using default positions

Two alternative candidates using default positions, which are (-2*width, 0) and (0, -2*height), are added before zero candidates. When alternative candidates are out of current CTU, proposed candidates are clipped to boundary position.

CE 8.1.1b: Alternative candidates using middle positions

Two alternative candidates using middle positions are added before zero candidates. The first horizontal candidate is middle position between x-axis location of the top-left sample of current block and left boundary of current CTU. The second vertical candidate is middle position between y-axis location of the top-left sample of current block and above boundary of current CTU. In this test, proposed candidates are not clipped to boundary position.

Experimental results

The proposed methods have been implemented on top of the CE8 software based on VTM-3.0 and tested under common test conditions in [2] and CE8 test conditions [3].

Table 1. Performance of test 8.1.1a with CPR off in AI

|  |  |  | AI Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.08% | 0.03% | 137% | 102% |
| Class A2 | -0.52% | -0.57% | -0.56% | 137% | 103% |
| Class B | -0.36% | -0.42% | -0.40% | 146% | 104% |
| Class C | -0.16% | -0.24% | -0.37% | 149% | 105% |
| Class E | -0.78% | -0.91% | -0.73% | 149% | 103% |
| Overall | -0.34% | -0.43% | -0.41% | 144% | 104% |
| Class D | -0.18% | -0.16% | -0.03% | 145% | 104% |
| Class F | -12.42% | -12.24% | -12.27% | 161% | 104% |
| Class SCC | -37.81% | -36.76% | -37.03% | 139% | 102% |

Table 2. Performance of test 8.1.1a with CPR off in RA

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.02% | 0.29% | 101% | 101% |
| Class A2 | -0.04% | 0.05% | 0.07% | 98% | 101% |
| Class B | 0.00% | -0.14% | -0.31% | 100% | 101% |
| Class C | 0.15% | -0.06% | 0.09% | 104% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | -0.06% | 0.00% | 101% | 101% |
| Class D | 0.07% | -0.30% | 0.10% | 104% | 102% |
| Class F | -10.10% | -9.99% | -10.12% | 108% | 102% |
| Class SCC | -22.77% | -22.27% | -22.78% | 104% | 100% |

Table 3. Performance of test 8.1.1a with CPR off in LDB

|  |  |  | LDB Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.06% | -0.10% | 108% | 102% |
| Class C | 0.12% | 0.21% | 0.38% | 111% | 103% |
| Class E | 0.10% | 0.19% | 0.33% | 107% | 102% |
| Overall | 0.09% | 0.14% | 0.17% | 109% | 102% |
| Class D | 0.13% | 0.25% | 0.22% | 113% | 105% |
| Class F | -5.27% | -5.45% | -5.58% | 109% | 102% |
| Class SCC | -13.75% | -13.88% | -14.23% | 110% | 102% |

Table 4. Performance of test 8.1.1a with CPR on in AI

|  |  |  | AI Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.02% | -0.03% | 108% | 102% |
| Class A2 | -0.05% | -0.07% | -0.02% | 107% | 103% |
| Class B | -0.07% | -0.03% | -0.05% | 107% | 104% |
| Class C | -0.07% | 0.00% | -0.12% | 107% | 105% |
| Class E | -0.15% | -0.10% | -0.14% | 107% | 103% |
| Overall | -0.07% | -0.03% | -0.07% | 107% | 103% |
| Class D | -0.08% | 0.07% | -0.24% | 107% | 104% |
| Class F | -0.41% | -0.26% | -0.23% | 104% | 105% |
| Class SCC | -1.45% | -1.06% | -1.10% | 127% | 106% |

Table 5. Performance of test 8.1.1a with CPR on in RA

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.02% | 0.07% | 102% | 101% |
| Class A2 | -0.04% | -0.02% | -0.17% | 102% | 101% |
| Class B | -0.02% | -0.14% | -0.10% | 101% | 101% |
| Class C | -0.04% | 0.00% | 0.01% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.05% | -0.05% | 102% | 102% |
| Class D | -0.08% | -0.02% | -0.06% | 102% | 103% |
| Class F | -0.27% | -0.11% | -0.22% | 102% | 102% |
| Class SCC | -0.96% | -0.90% | -0.88% | 101% | 104% |

Table 6. Performance of test 8.1.1a with CPR on in LDB

|  |  |  | LDB Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.38% | 0.04% | 101% | 101% |
| Class C | -0.01% | -0.18% | 0.05% | 102% | 104% |
| Class E | -0.18% | -0.85% | 0.51% | 101% | 101% |
| Overall | 0.00% | -0.11% | 0.16% | 102% | 102% |
| Class D | -0.01% | 0.41% | -0.47% | 102% | 104% |
| Class F | -0.15% | -0.10% | -0.13% | 101% | 103% |
| Class SCC | -0.76% | -0.56% | -0.71% | 102% | 105% |

Table 7. Performance of test 8.1.1a with CPR and PLT on in AI

|  |  |  | AI Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR+PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | -0.06% | 105% | 100% |
| Class A2 | -0.04% | -0.03% | -0.04% | 104% | 101% |
| Class B | -0.07% | -0.06% | -0.11% | 105% | 100% |
| Class C | -0.05% | -0.03% | -0.10% | 105% | 100% |
| Class E | -0.16% | -0.20% | -0.17% | 105% | 99% |
| Overall | -0.07% | -0.06% | -0.10% | 105% | 100% |
| Class D | -0.06% | -0.08% | 0.03% | 105% | 100% |
| Class F | -0.14% | -0.25% | -0.26% | 103% | 100% |
| Class SCC | -0.59% | -0.59% | -0.58% | 125% | 101% |

Table 8. Performance of test 8.1.1a with CPR and PLT on in RA

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR+PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.15% | -0.02% | 100% | 100% |
| Class A2 | -0.04% | 0.04% | -0.04% | 100% | 100% |
| Class B | -0.03% | -0.02% | -0.15% | 101% | 100% |
| Class C | -0.05% | 0.08% | -0.32% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.01% | -0.15% | 100% | 100% |
| Class D | -0.12% | -0.04% | 0.56% | 101% | 100% |
| Class F | -0.14% | -0.02% | -0.09% | 101% | 100% |
| Class SCC | -0.58% | -0.55% | -0.58% | 101% | 100% |

Table 9. Performance of test 8.1.1a with CPR and PLT on in LDB

|  |  |  | LDB Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR+PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.28% | 0.24% | 100% | 99% |
| Class C | -0.03% | -0.25% | 0.12% | 100% | 100% |
| Class E | -0.15% | -0.18% | -0.07% | 100% | 99% |
| Overall | 0.00% | -0.01% | 0.12% | 100% | 99% |
| Class D | -0.03% | -0.75% | 0.13% | 100% | 100% |
| Class F | -0.14% | -0.38% | -0.28% | 100% | 99% |
| Class SCC | -0.43% | -0.49% | -0.48% | 100% | 99% |

Table 10. Performance of test 8.1.1b with CPR off in AI

|  |  |  | AI Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.02% | 0.06% | 137% | 102% |
| Class A2 | -0.52% | -0.57% | -0.56% | 138% | 103% |
| Class B | -0.36% | -0.39% | -0.40% | 146% | 104% |
| Class C | -0.16% | -0.26% | -0.27% | 149% | 108% |
| Class E | -0.74% | -0.90% | -0.69% | 149% | 105% |
| Overall | -0.34% | -0.41% | -0.37% | 144% | 104% |
| Class D | -0.21% | -0.07% | 0.12% | 146% | 106% |
| Class F | -12.41% | -12.37% | -12.34% | 162% | 106% |
| Class SCC | -37.80% | -36.76% | -37.03% | 138% | 101% |

Table 11. Performance of test 8.1.1b with CPR off in RA

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.08% | 0.20% | 100% | 101% |
| Class A2 | -0.03% | 0.14% | 0.21% | 98% | 101% |
| Class B | -0.01% | 0.00% | -0.15% | 100% | 101% |
| Class C | 0.13% | -0.15% | 0.06% | 104% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.00% | 0.05% | 101% | 102% |
| Class D | 0.18% | -0.31% | 0.10% | 104% | 102% |
| Class F | -10.07% | -9.94% | -10.16% | 108% | 102% |
| Class SCC | -22.69% | -22.18% | -22.73% | 104% | 100% |

Table 12. Performance of test 8.1.1b with CPR off in LDB

|  |  |  | LDB Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.07% | -0.20% | 107% | 102% |
| Class C | 0.10% | 0.47% | 0.34% | 111% | 103% |
| Class E | 0.29% | 0.77% | -1.04% | 106% | 101% |
| Overall | 0.14% | 0.32% | -0.23% | 108% | 102% |
| Class D | 0.18% | -0.35% | 0.71% | 113% | 103% |
| Class F | -5.41% | -5.62% | -5.99% | 109% | 102% |
| Class SCC | -13.56% | -13.81% | -14.03% | 110% | 101% |

Table 13. Performance of test 8.1.1b with CPR on in AI

|  |  |  | AI Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.12% | 0.01% | 108% | 102% |
| Class A2 | -0.05% | -0.07% | -0.02% | 107% | 103% |
| Class B | -0.06% | -0.01% | -0.05% | 107% | 103% |
| Class C | -0.07% | -0.02% | -0.03% | 107% | 108% |
| Class E | -0.11% | -0.09% | -0.09% | 107% | 105% |
| Overall | -0.06% | -0.01% | -0.04% | 107% | 104% |
| Class D | -0.11% | 0.16% | -0.10% | 107% | 105% |
| Class F | -0.40% | -0.40% | -0.30% | 105% | 107% |
| Class SCC | -1.45% | -1.05% | -1.10% | 127% | 108% |

Table 14. Performance of test 8.1.1b with CPR on in RA

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.12% | -0.02% | 101% | 101% |
| Class A2 | -0.03% | 0.07% | -0.03% | 101% | 102% |
| Class B | -0.02% | 0.00% | 0.06% | 101% | 101% |
| Class C | -0.06% | -0.09% | -0.03% | 102% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.01% | 0.00% | 101% | 102% |
| Class D | 0.03% | -0.03% | -0.06% | 102% | 103% |
| Class F | -0.24% | -0.06% | -0.27% | 101% | 103% |
| Class SCC | -0.86% | -0.77% | -0.81% | 101% | 104% |

Table 15. Performance of test 8.1.1b with CPR on in LDB

|  |  |  | LDB Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.25% | -0.07% | 101% | 101% |
| Class C | -0.03% | 0.08% | 0.01% | 101% | 104% |
| Class E | 0.02% | -0.30% | -0.86% | 101% | 101% |
| Overall | 0.00% | 0.06% | -0.24% | 101% | 102% |
| Class D | 0.04% | -0.20% | 0.02% | 101% | 102% |
| Class F | -0.32% | -0.29% | -0.58% | 101% | 102% |
| Class SCC | -0.54% | -0.47% | -0.48% | 101% | 104% |

Table 16. Performance of test 8.1.1b with CPR and PLT on in AI

|  |  |  | AI Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR+PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | -0.05% | 105% | 101% |
| Class A2 | -0.04% | -0.01% | -0.04% | 104% | 101% |
| Class B | -0.06% | -0.02% | -0.06% | 105% | 100% |
| Class C | -0.06% | 0.00% | -0.02% | 105% | 100% |
| Class E | -0.11% | -0.14% | 0.00% | 105% | 100% |
| Overall | -0.06% | -0.04% | -0.04% | 105% | 100% |
| Class D | -0.07% | -0.18% | -0.10% | 105% | 100% |
| Class F | -0.15% | -0.16% | -0.16% | 103% | 100% |
| Class SCC | -0.59% | -0.58% | -0.59% | 125% | 101% |

Table 17. Performance of test 8.1.1b with CPR and PLT on in RA

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR+PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | -0.05% | 100% | 100% |
| Class A2 | -0.01% | 0.09% | -0.06% | 100% | 100% |
| Class B | -0.01% | 0.08% | -0.23% | 100% | 100% |
| Class C | -0.05% | -0.07% | -0.23% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.03% | -0.16% | 100% | 100% |
| Class D | -0.07% | -0.01% | 0.34% | 100% | 100% |
| Class F | -0.15% | -0.03% | 0.02% | 100% | 100% |
| Class SCC | -0.55% | -0.53% | -0.59% | 100% | 100% |

Table 18. Performance of test 8.1.1b with CPR and PLT on in LDB

|  |  |  | LDB Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR+PLT on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.15% | -0.17% | 100% | 99% |
| Class C | -0.05% | -0.02% | 0.11% | 100% | 101% |
| Class E | -0.11% | -0.39% | -0.07% | 100% | 99% |
| Overall | 0.00% | -0.04% | -0.05% | 100% | 100% |
| Class D | -0.01% | 0.02% | -0.06% | 100% | 100% |
| Class F | -0.42% | -0.33% | -0.30% | 100% | 99% |
| Class SCC | -0.43% | -0.48% | -0.37% | 100% | 99% |

Conclusion

In this contribution, two additional candidates for block vector prediction on CPR mode are tested in CE8.1.1 as a solution for invalid zero candidate insertion. It is recommended to adopt the proposed candidates for CPR mode.

Reference

- J. Nam, et al., “Non-CE8: Block vector predictor for IBC,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L0159, 12th meeting, Macau, CN, 08–12 Oct. 2018.

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET- L1010, 12th meeting, Macau, CN, 08–12 Oct. 2018.

- X. Xu, et al., “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” , Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET- L1028, 12th meeting, Macau, CN, 08–12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).