Draft text of CE8.1.1a

Decoding process for coding units coded in inter prediction mode

General decoding process for coding units coded in inter prediction mode

Inputs to this process are:

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components.

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Output of this process is a modified reconstructed picture before in-loop filtering.

The derivation process for quantization parameters as specified in clause 8.4.1 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight, and the variable treeType as inputs.

The decoding process for coding units coded in inter prediction mode consists of the following ordered steps:

- The motion vector components and reference indices of the current coding unit are derived as follows:

- If merge_triangle_flag[ xCb ][ yCb ], inter_affine_flag[ xCb ][ yCb ] and merge_subblock_flag[ xCb ][ yCb ] are all equal to 0, the following applies:

- If treeType is equal to SINGLE_TREE, the derivation process for motion vector components and reference indices as specified in clause 8.3.2.1 is invoked with the treeType, the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the luma motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], the chroma motion vectors mvCL0[ 0 ][ 0 ] and mvCL1[ 0 ][ 0 ], the reference indices refIdxL0 and refIdxL1 and the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], and the bi-prediction weight index gbiIdx as outputs.

- Otherwise, if treeType is equal to DUAL_TREE_LUMA, the derivation process for motion vector components and reference indices as specified in clause 8.3.2.1 is invoked with the treeType, the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the luma motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], the reference indices refIdxL0 and refIdxL1, and the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] as outputs.

- Otherwise, if treeType is equal to DUAL_TREE_CHROMA, the derivation process for motion vector components and reference indices as specified in clause 8.3.2.1 is invoked with the treeType, luma coding block location ( xCb, yCb ), the chroma coding block width cbWidth in luma samples, and the chroma coding block height cbHeight in luma samples as inputs, the chroma motion vector array mvCL0[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..(cbWidth >> 2) − 1, and ySbIdx = 0..( cbHeight >> 2 ) − 1 as outputs.

- The number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY are both set equal to 1.

- Otherwise, if merge_triangle_flag[ xCb ][ yCb ] is equal to 1, inter_affine_flag[ xCb ][ yCb ] and merge_subblock_flag[ xCb ][ yCb ] are both equal to 0, the derivation process for triangle motion vector components and reference indices as specified in clause 8.3.3.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the luma motion vectors mvLA and mvLB, the chroma motion vectors mvCLA and mvCLB, the reference indices refIdxLA and refIdxLB and the prediction list flags predFlagLA and predFlagLB as outputs.

- Otherwise (inter_affine_flag[ xCb ][ yCb ] or merge_subblock_flag[ xCb ][ yCb ] is equal to 1), the following applies:

- The derivation process for subblock motion vector components and reference indices as specified in clause 8.3.4.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight as inputs, and the reference indices refIdxL0 and refIdxL1, the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the prediction list utilization flags predFlagLX[ xSbIdx ][ ySbIdx ], the luma motion vector array mvLX[ xSbIdx ][ ySbIdx ], and the chroma motion vector array mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..(cbWidth >> 2) − 1, and ySbIdx = 0..( cbHeight >> 2 ) − 1, and with X being 0 or 1, and the bi-prediction weight index gbiIdx as outputs.

- When mh_intra_flag[ xCb ][ yCb ] is equal to 1, the derivation process for intra prediction mode in combined merge and intra prediction as specified in 8.3.5 is invoked.

- The prediction samples of the current coding unit are derived as follows:

- If merge_triangle_flag[ xCb ][ yCb ] is equal to 0, and if treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA, the prediction samples of the current coding unit are derived by invoking the decoding process for inter blocks as specified in clause 8.3.6.1 with the treeType, the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight, the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the luma motion vectors mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ] the reference indices refIdxL0 and refIdxL1, the prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ], and the generalization bi-prediction weight index gbiIdx as inputs, and the inter prediction samples (predSamples) that are an (cbWidth)x(cbHeight) array predSamplesL of prediction luma samplesas outputs.

- Otherwise if merge_triangle_flag[ xCb ][ yCb ] is equal to 0, and if treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA,

      - If treeType is equal to DUAL_TREE_CHROMA, the reference indices refIdxL0 is set to 0; and refIdxL1 is set to −1; the prediction list utilization flags predFlagL0 is set to 1 and predFlagL1 is set to 0.

      - The decoding process as specified in clause 8.3.6.1 is invoked with the treeType, the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight, the chroma motion vectors mvCL0[ xSbIdx ][ ySbIdx ] and mvCL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the reference indices refIdxL0 and refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1 as inputs, and the inter prediction samples (predSamples) that are two (cbWidth / 2)x(cbHeight / 2) arrays predSamplesCb and predSamplesCr of prediction chroma samples, one for each of the chroma components Cb and Cr, as outputs.

- Otherwise (merge_triangle_flag[ xCb ][ yCb ] is equal to 1 [Ed. (SL): Seems to be a typo.]), the decoding process for triangular inter blocks as specified in clause 8.3.7.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight, the luma motion vectors mvLA and mvLB, the chroma motion vectors mvCLA and mvCLB, the reference indices refIdxLA and refIdxLB, and the prediction list flags predFlagLA and predFlagLB as inputs, and the inter prediction samples (predSamples) that are an (cbWidth)x(cbHeight) array predSamplesL of prediction luma samples and two (cbWidth / 2)x(cbHeight / 2) arrays predSamplesCb and predSamplesCr of prediction chroma samples, one for each of the chroma components Cb and Cr, as outputs.

- The variables NumSbX[ xCb ][ yCb ] and NumSbY[ xCb ][ yCb ] are set equal to numSbX and numSbY, respectively.

- The residual samples of the current coding unit are derived as follows:

- If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA, the decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.3.8 is invoked with the location ( xTb0, yTb0 ) set equal to the luma location ( xCb, yCb ), the width nTbW set equal to the luma coding block width cbWidth, the height nTbH set equal to the luma coding block height cbHeight and the variable cIdxset equal to 0 as inputs, and the array resSamplesL as output.

- If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA, the decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.3.8 is invoked with the location ( xTb0, yTb0 ) set equal to the chroma location ( xCb / 2, yCb / 2 ), the width nTbW set equal to the chroma coding block width cbWidth / 2, the height nTbH set equal to the chroma coding block height cbHeight / 2 and the variable cIdxset equal to 1 as inputs, and the array resSamplesCb as output.

- The decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.3.8 is invoked with the location ( xTb0, yTb0 ) set equal to the chroma location ( xCb / 2, yCb / 2 ), the width nTbW set equal to the chroma coding block width cbWidth / 2, the height nTbH set equal to the chroma coding block height cbHeight / 2 and the variable cIdxset equal to 2 as inputs, and the array resSamplesCr as output.

- The reconstructed samples of the current coding unit are derived as follows:

- If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA, the picture reconstruction process for a colour component as specified in clause 8.4.5 is invoked with the block location ( xB, yB ) set equal to ( xCb, yCb ), the block width bWidth set equal to cbWidth, the block height bHeight set equal to cbHeight, the variable cIdx set equal to 0, the (cbWidth)x(cbHeight) array predSamples set equal to predSamplesL and the (cbWidth)x(cbHeight) array resSamples set equal to resSamplesL as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA, the picture reconstruction process for a colour component as specified in clause 8.4.5 is invoked with the block location ( xB, yB ) set equal to ( xCb / 2, yCb / 2 ), the block width bWidth set equal to cbWidth / 2, the block height bHeight set equal to cbHeight / 2, the variable cIdx set equal to 1, the (cbWidth / 2)x(cbHeight / 2) array predSamples set equal to predSamplesCb and the (cbWidth / 2)x(cbHeight / 2) array resSamples set equal to resSamplesCb as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- The picture reconstruction process for a colour component as specified in clause 8.4.5 is invoked with the block location ( xB, yB ) set equal to ( xCb / 2, yCb / 2 ), the block width bWidth set equal to cbWidth / 2, the block height bHeight set equal to cbHeight / 2, the variable cIdx set equal to 2, the (cbWidth / 2)x(cbHeight / 2) array predSamples set equal to predSamplesCb and the (cbWidth / 2)x(cbHeight / 2) array resSamples set equal to resSamplesCb as inputs, and the output is a modified reconstructed picture before in-loop filtering.

Derivation process for motion vector components and reference indices

General

Inputs to this process are:

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components.

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

If treeType is equal to SINGLE_TREE,

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ],

- the chroma motion vectors in 1/32 fractional-sample accuracy mvCL0[ 0 ][ 0 ] and mvCL1[ 0 ][ 0 ],the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0 and predFlagL1[ 0 ][ 0 ],

- the bi-prediction weight index gbiIdx.

Otherwise, if treeType is equal to DUAL_TREE_LUMA,

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ],

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0 and predFlagL1[ 0 ][ 0 ],

- the bi-prediction weight index gbiIdx.

Otherwise (treeType is equal to DUAL_TREE_CHROMA),

- the chroma motion vector arrays in 1/32 fractional-sample accuracy mvCL0[ xSbIdx ][ ySbIdx ] and mvCL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0..numSbY – 1,

- the bi-prediction weight index gbiIdx.

Let the variable LX be RefPicListX, with X being 0 or 1, of the current picture.

For the derivation of the variables mvL0 and mvL1, refIdxL0 and refIdxL1, as well as predFlagL0 and predFlagL1, the following applies:

- If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA,

- If merge_flag[ xCb ][ yCb ] is equal to 1, the derivation process for luma motion vectors for merge mode as specified in clause 8.3.2.2 is invoked with the luma location ( xCb, yCb ), the variables cbWidth and cbHeight inputs, and the output being the luma motion vectors mvL0[ 0 ][ 0 ], mvL1[ 0 ][ 0 ], the reference indices refIdxL0, refIdxL1, the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], and the bi-prediction weight index gbiIdx.

- Otherwise, the following applies:

  - For X being replaced by either 0 or 1 in the variables predFlagLX[ 0 ][0 ], mvLX[ 0 ][0 ] and refIdxLX, in PRED_LX, and in the syntax elements ref_idx_lX and MvdLX, the following ordered steps apply:

- The variables refIdxLX and predFlagLX[ 0 ][0 ] are derived as follows:

- If inter_pred_idc[ xCb ][ yCb ] is equal to PRED_LX or PRED_BI,

refIdxLX = ref_idx_lX[ xCb ][ yCb ]	(8-186)

predFlagLX[ 0 ][0 ] = 1		(8-187)

- Otherwise, the variables refIdxLX and predFlagLX[ 0 ][0 ] are specified by:

refIdxLX = −1		(8-188)

predFlagLX[ 0 ][0 ] = 0		(8-189)

- The variable mvdLX is derived as follows:

mvdLX[ 0 ] = MvdLX[ xCb ][ yCb ][ 0 ]	(8-190)

mvdLX[ 1 ] = MvdLX[ xCb ][ yCb ][ 1 ]	(8-191)

- When predFlagLX[ 0 ][0 ] is equal to 1, the derivation process for luma motion vector prediction in clause 8.3.2.8 is invoked with the luma coding block location ( xCb, yCb ), the coding block width cbWidth, the coding block height cbHeight and the variable refIdxLX as inputs, and the output being mvpLX.

- When predFlagLX[ 0 ][0 ] is equal to 1, the luma motion vector mvLX[ 0 ][0 ] is derived as follows:

uLX[ 0 ] = ( mvpLX[ 0 ] + mvdLX[ 0 ] + 218 ) % 218	(8-192)

mvLX[ 0 ][0 ][ 0 ] = ( uLX[ 0 ]  >=  217 ) ? ( uLX[ 0 ] − 218 ) : uLX[ 0 ]	(8-193)

uLX[ 1 ] = ( mvpLX[ 1 ] + mvdLX[ 1 ] + 218 ) % 218	(8-194)

mvLX[ 0 ][0 ][ 1 ] = ( uLX[ 1 ]  >=  217 ) ? ( uLX[ 1 ] − 218 ) : uLX[ 1 ]	(8-195)

NOTE 1– The resulting values of mvLX[ 0 ] and mvLX[ 1 ] as specified above will always be in the range of −217 to 217 − 1, inclusive.

  - The bi-prediction weight index gbiIdx is set equal to gbi_idx[ xCb ][ yCb ].

- When predFlagLX[ 0 ][0 ], with X being 0 or 1, is equal to 1, the derivation process for chroma motion vectors in clause 8.3.2.13 is invoked with mvLX[ 0 ][ 0 ] and  refIdxLX as input, and the output being mvCLX[ 0 ][ 0 ].

- Otherwise (if treeType is equal to DUAL_TREE_CHROMA),

- The variables numSbX and numSbY are derived as follows:

numSbX = ( cbWidth >> 2 )		(8-xxx)

numSbY = ( cbHeight >> 2 )		(8-xxx)

- For xSbIdx = 0..numSbX – 1, ySbIdx = 0..numSbY – 1, the following applies:

  - If CuPredMode[ xCb + xSbIdx*4 ][ yCb + ySbIdx*4 ] for luma coding block is equal to MODE_INTRA, mvLX[ xSbIdx ][ ySbIdx ][0] and mvLX[ xSbIdx ][ ySbIdx ][1] with xSbIdx = 0..numSbX – 1, ySbIdx = 0 .. numSbY – 1, are set to be 0.

  - The derivation process for chroma motion vectors in clause 8.3.2.13 is invoked with mvLX[ xSbIdx ][ ySbIdx ] from collocated luma samples and refIdxLX as input, and the output being mvCLX[ xSbIdx ][ ySbIdx ], with xSbIdx = 0..numSbX – 1, ySbIdx = 0 .. numSbY – 1.

When all of the following conditions are true, refIdxL1 is set equal to −1, predFlagL1 is set equal to 0, and gbiIdx is set equal to 0:

- predFlagL0 is equal to 1.

- predFlagL1 is equal to 1.

- cbWidth is equal to 4.

- cbHeight is equal to 4.

The updating process for the history-based motion vector predictor list as specified in clause 8.3.2.15 is invoked with luma motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], reference indices refIdxL0 and refIdxL1, and prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ].

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA, it is a requirement of bitstream conformance that when the reference picture is the current picture, the luma motion vector mvLX shall obey the following constraints:

- When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xCb + ( mvLX[ 0 ]  >>  4 ), yCb + ( mvLX[ 1 ]  >>  4 )  ) as inputs, and the output the output shall be equal to TRUE.

- When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xCb + ( mvLX[ 0 ]  >>  4 ) + cbWidth – 1, yCb + ( mvLX[ 1]  >>  4 ) + cbHeight – 1 ) as inputs, and the output the output shall be equal to TRUE.

- One or both the following conditions shall be true:

- The value of ( mvLX[ 0 ]  >>  4 ) + cbWidth is less than or equal to 0.

- The value of ( mvLX[ 1 ]  >>  4 ) + cbHeight is less than or equal to 0.

- The following conditions shall be true:

( yCb + ( mvLX[ 1 ]  >>  4 ) ) >>  CtbLog2SizeY = yCb >>  CtbLog2SizeY	(8-xxx)

( yCb + ( mvLX[ 1 ]  >>  4 )  + cbHeight – 1) >>  CtbLog2SizeY = yCb >>  CtbLog2SizeY						   (8-xxx)

( xCb + ( mvLX[ 0 ]  >>  4 ) ) >>  CtbLog2SizeY = xCb >>  CtbLog2SizeY	(8-xxx)

( xCb + ( mvLX[ 0 ]  >>  4 )  + cbWidth – 1) >>  CtbLog2SizeY = xCb >>  CtbLog2SizeY						 (8-xxx)

If treeType is equal to DUAL_TREE_CHROMA, for xSbIdx = 0..numSbX – 1, ySbIdx = 0..numSbY – 1, it is a requirement of bitstream conformance that the chroma motion vector mvCLX[ xSbIdx ][ ySbIdx ] shall obey the following constraints:

- One or both the following conditions shall be true:

- The value of ( mvCLX[ xSbIdx ][ ySbIdx ] [ 0 ]  >>  5 ) + xSbIdx * 2 + 2 is less than or equal to 0.

- The value of ( mvCLX[ xSbIdx ][ ySbIdx ] [ 1 ]  >>  5 ) + ySbIdx * 2 + 2 is less than or equal to 0.

Derivation process for luma motion vectors for merge mode

This process is only invoked when merge_flag[ xCb ][ yPb ] is equal to 1, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ],

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ],

- the bi-prediction weight index gbiIdx.

The bi-prediction weight index gbiIdx is set equal to 0.

The motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], the reference indices refIdxL0 and refIdxL1 and the prediction utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are derived by the following ordered steps:

- The derivation process for merging candidates from neighbouring coding units as specified in clause 8.3.2.3 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, and the luma coding block height cbHeight as inputs, and the output being the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1 and refIdxLXB2, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and predFlagLXB2, and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1 and mvLXB2, with X being 0 or 1, and the bi-prediction weight indices gbiIdxA0, gbiIdxA1, gbiIdxB0, gbiIdxB1, gbiIdxB2.

- The reference indices, refIdxLXCol, with X being 0 or 1, and the bi-prediction weight index gbiIdxCol for the temporal merging candidate Col are set equal to 0.

- The derivation process for temporal luma motion vector prediction as specified in in clause 8.3.2.11 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the variable refIdxL0Col as inputs, and the output being the availability flag availableFlagL0Col and the temporal motion vector mvL0Col. The variables availableFlagCol, predFlagL0Col and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col		(8-196)

predFlagL0Col = availableFlagL0Col		(8-197)

predFlagL1Col = 0		(8-198)

- When slice_type is equal to B, the derivation process for temporal luma motion vector prediction as specified in clause 8.3.2.11 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the variable refIdxL1Col as inputs, and the output being the availability flag availableFlagL1Col and the temporal motion vector mvL1Col. The variables availableFlagCol and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col  | |  availableFlagL1Col	(8-199)

predFlagL1Col = availableFlagL1Col		(8-200)

- The merging candidate list, mergeCandList, is constructed as follows:

i = 0
if( availableFlagA1 )
	mergeCandList[ i++ ] = A1
if( availableFlagB1 )
	mergeCandList[ i++ ] = B1
if( availableFlagB0 )
	mergeCandList[ i++ ] = B0		(8-201)
if( availableFlagA0 )
	mergeCandList[ i++ ] = A0
if( availableFlagB2 )
	mergeCandList[ i++ ] = B2
if( availableFlagCol )
	mergeCandList[ i++ ] = Col

- The variable numCurrMergeCand and numOrigMergeCand are set equal to the number of merging candidates in the mergeCandList.

- When numCurrMergeCand is less than (MaxNumMergeCand – 1) and NumHmvpCand is greater than 0, the following applies:

- The derivation process of history-based merging candidates as specified in 8.3.2.6 is invoked with mergeCandList, and numCurrMergeCand as inputs, and modified mergeCandList and numCurrMergeCand as outputs.

- numOrigMergeCand is set equal to numCurrMergeCand.

- The derivation process for pairwise average merging candidates specified in clause 8.3.2.4 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList, numCurrMergeCand and numOrigMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0avgCandk and refIdxL1avgCandk, the prediction list utilization flags predFlagL0avgCandk and predFlagL1avgCandk and the motion vectors mvL0avgCandk and mvL1avgCandk of every new candidate avgCandk being added into mergeCandList. The bi-prediction weight index gbiIdx of every new candidate avgCandk being added into mergeCandList is set equal to 0. The number of candidates being added, numAvgMergeCand, is set equal to ( numCurrMergeCand – numOrigMergeCand ). When numAvgMergeCand is greater than 0, k ranges from 0 to numAvgMergeCand – 1, inclusive.

- When numCurrMergeCand is less than (MaxNumMergeCand – 1), the following applies:

- The derivation process for default candidates on current picture referencing specified in clause 8.3.2.17 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight, mergeCandList, numCurrMergeCand, and the maximum number of candidates MaxNumMergeCand as inputs, and modified mergeCandList and numCurrMergeCand as outputs.

- The derivation process for zero motion vector merging candidates specified in clause 8.3.2.5 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList and numCurrMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm and the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added into mergeCandList. The bi-prediction weight index gbiIdx of every new candidate zeroCandm being added into mergeCandList is set equal to 0. The number of candidates being added, numZeroMergeCand, is set equal to ( numCurrMergeCand – numOrigMergeCand – numAvgMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand – 1, inclusive.

- If mmvd_flag[ xCb ][ yCb ] is equal to 1, the variable mmvdCnt is set to 0. The following applies until mmvdCnt is greater than merge_idx[ xCb ][ yCb ] or mmvdCnt is equal to MaxNumMergeCand:

- If candidate mergeCandList[ mmvdCnt ] uses the current decoded picture as its reference picture, merge_idx[ xCb ][ yCb ]++;

- mmvdCnt++;

[Ed. (SL): to disallow CPR and MMVD combination.]

- The following assignments are made with N being the candidate at position merge_idx[ xCb ][ yCb ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xCb ][ yCb ] ] ) and X being replaced by 0 or 1:

refIdxLX = refIdxLXN		(8-202)

predFlagLX[ 0 ][ 0 ] = predFlagLXN		(8-203)

mvLX[ 0 ][ 0 ][ 0 ] = mvLXN[ 0 ]		(8-204)

mvLX[ 0 ][ 0 ][ 1 ] = mvLXN[ 1 ]		(8-205)

gbiIdx = gbiIdxN		(8-206)

- When mmvd_flag[ xCb ][ yCb ] is equal to 1, the following applies:

- The derivation process for merge motion vector difference as specified in 8.3.2.7 is invoked with the luma location ( xCb, yCb ), the luma motion vectors mvL0[ 0 ][ 0 ], mvL1[ 0 ][ 0 ], the reference indices refIdxL0, refIdxL1 and the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] as inputs, and the motion vector differences mMvdL0 and mMvdL1 as outputs.

- The motion vector difference mMvdLX is added to the merge motion vectors mvLX for X being 0 and 1 as follows:

mvLX[ 0 ][ 0 ][ 0 ] += mMvdLX[ 0 ]		(8-207)

mvLX[ 0 ][ 0 ][ 1 ] += mMvdLX[ 1 ]		(8-208)

Derivation process for spatial merging candidates

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are as follows, with X being 0 or 1:

- the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2 of the neighbouring coding units,

- the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1 and refIdxLXB2 of the neighbouring coding units,

- the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and predFlagLXB2 of the neighbouring coding units,

- the motion vectors in 1/16 fractional-sample accuracy mvLXA0, mvLXA1, mvLXB0, mvLXB1 and mvLXB2 of the neighbouring coding units,

- the bi-prediction weight indices gbiIdxA0, gbiIdxA1, gbiIdxB0, gbiIdxB1, and gbiIdxB2.

For the derivation of availableFlagA1, refIdxLXA1, predFlagLXA1 and mvLXA1 the following applies:

- The luma location ( xNbA1, yNbA1 ) inside the neighbouring luma coding block is set equal to ( xCb – 1,  yCb + cbHeight – 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA1, yNbA1 ) as inputs, and the output is assigned to the block availability flag availableA1.

- If availableA1 is equal to TRUE and RefIdxL0[ xNbA1 ][ yNbA1 ] indicates that the current picture is used as the reference picture, availability checking process for block vector on current picture referencing as specified in clause 8.3.2.16 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the neighbouring motion vector mvL0A1 as inputs, and the output is assigned to the CPR availability flag availableFlagCPRA1.

- The variables availableFlagA1, refIdxLXA1, predFlagLXA1 and mvLXA1 are derived as follows:

- If availableA1 or availableFlagCPRA1 is equal to FALSE, availableFlagA1 is set equal to 0, both components of mvLXA1 are set equal to 0, refIdxLXA1 is set equal to −1 and predFlagLXA1 is set equal to 0, with X being 0 or 1, and gbiIdxA1 is set equal to 0.

- Otherwise, availableFlagA1 is set equal to 1 and the following assignments are made:

mvLXA1 = MvLX[ xNbA1 ][ yNbA1 ]		(8-209)

refIdxLXA1 = RefIdxLX[ xNbA1 ][ yNbA1 ]                                                     (8-210)

predFlagLXA1 = PredFlagLX[ xNbA1 ][ yNbA1 ]	(8-211)

gbiIdxA1 = GbiIdx[ xNbA1 ][ yNbA1 ]			(8-212)

For the derivation of availableFlagB1, refIdxLXB1, predFlagLXB1 and mvLXB1 the following applies:

- The luma location ( xNbB1, yNbB1 ) inside the neighbouring luma coding block is set equal to ( xCb + cbWidth – 1, yCb – 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB1, yNbB1 ) as inputs, and the output is assigned to the block availability flag availableB1.

- If availableB1 is equal to TRUE and RefIdxL0[ xNbB1 ][ yNbB1 ] indicates that the current picture is used as the reference picture, availability checking process for block vector on current picture referencing as specified in clause 8.3.2.16 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the neighbouring motion vector mvL0B1 as inputs, and the output is assigned to the CPR availability flag availableFlagCPRB1.

- The variables availableFlagB1, refIdxLXB1, predFlagLXB1 and mvLXB1 are derived as follows:

- If one or more of the following conditions are true, availableFlagB1 is set equal to 0, both components of mvLXB1 are set equal to 0, refIdxLXB1 is set equal to −1 and predFlagLXB1 is set equal to 0, with X being 0 or 1, and gbiIdxB1 is set equal to 0:

  - availableB1 is equal to FALSE.

  - availableFlagCPRB1 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB1, yNbB1 ) have the same motion vectors and the same reference indices.

- Otherwise, availableFlagB1 is set equal to 1 and the following assignments are made:

mvLXB1 = MvLX[ xNbB1 ][ yNbB1 ]		(8-213)

refIdxLXB1 = RefIdxLX[ xNbB1 ][ yNbB1 ]		(8-214)

predFlagLXB1 = PredFlagLX[ xNbB1 ][ yNbB1 ]		(8-215)

gbiIdxB1 = GbiIdx[ xNbB1 ][ yNbB1 ]			(8-216)

For the derivation of availableFlagB0, refIdxLXB0, predFlagLXB0 and mvLXB0 the following applies:

- The luma location ( xNbB0, yNbB0 ) inside the neighbouring luma coding block is set equal to ( xCb + cbWidth, yCb – 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB0, yNbB0 ) as inputs, and the output is assigned to the block availability flag availableB0.

- If availableB0 is equal to TRUE and RefIdxL0[ xNbB0 ][ yNbB0 ] indicates that the current picture is used as the reference picture, availability checking process for block vector on current picture referencing as specified in clause 8.3.2.16 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the neighbouring motion vector mvL0B0 as inputs, and the output is assigned to the CPR availability flag availableFlagCPRB0.

- The variables availableFlagB0, refIdxLXB0, predFlagLXB0 and mvLXB0 are derived as follows:

- If one or more of the following conditions are true, availableFlagB0 is set equal to 0, both components of mvLXB0 are set equal to 0, refIdxLXB0 is set equal to −1 and predFlagLXB0 is set equal to 0, with X being 0 or 1, and gbiIdxB0 is set equal to 0:

  - availableB0 is equal to FALSE.

  - availableFlagCPRB0 is equal to FALSE.

  - availableB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB0, yNbB0 ) have the same motion vectors and the same reference indices.

- Otherwise, availableFlagB0 is set equal to 1 and the following assignments are made:

mvLXB0 = MvLX[ xNbB0 ][ yNbB0 ]		(8-217)

refIdxLXB0 = RefIdxLX[ xNbB0 ][ yNbB0 ]		(8-218)

predFlagLXB0 = PredFlagLX[ xNbB0 ][ yNbB0 ]		(8-219)

gbiIdxB0 = GbiIdx[ xNbB0 ][ yNbB0 ]		(8-220)

For the derivation of availableFlagA0, refIdxLXA0, predFlagLXA0 and mvLXA0 the following applies:

- The luma location ( xNbA0, yNbA0 ) inside the neighbouring luma coding block is set equal to ( xCb – 1,  yCb + cbWidth ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA0, yNbA0 ) as inputs, and the output is assigned to the block availability flag availableA0.

- If availableA0 is equal to TRUE and RefIdxL0[ xNbA0 ][ yNbA0 ] indicates that the current picture is used as the reference picture, availability checking process for block vector on current picture referencing as specified in clause 8.3.2.16 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the neighbouring motion vector mvL0A0 as inputs, and the output is assigned to the CPR availability flag availableFlagCPRA0.

- The variables availableFlagA0, refIdxLXA0, predFlagLXA0 and mvLXA0 are derived as follows:

- If one or more of the following conditions are true, availableFlagA0 is set equal to 0, both components of mvLXA0 are set equal to 0, refIdxLXA0 is set equal to −1 and predFlagLXA0 is set equal to 0, with X being 0 or 1, and gbiIdxA0 is set equal to 0:

  - availableA0 is equal to FALSE.

  - availableFlagCPRA0 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbA0, yNbA0 ) have the same motion vectors and the same reference indices.

- Otherwise, availableFlagA0 is set equal to 1 and the following assignments are made:

mvLXA0 = MvLX[ xNbA0 ][ yNbA0 ]		(8-221)

refIdxLXA0 = RefIdxLX[ xNbA0 ][ yNbA0 ]		(8-222)

predFlagLXA0 = PredFlagLX[ xNbA0 ][ yNbA0 ]	(8-223)

gbiIdxA0 = GbiIdx[ xNbA0 ][ yNbA0 ]		(8-224)

For the derivation of availableFlagB2, refIdxLXB2, predFlagLXB2 and mvLXB2 the following applies:

- The luma location ( xNbB2, yNbB2 ) inside the neighbouring luma coding block is set equal to ( xCb – 1, yCb – 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB2, yNbB2 ) as inputs, and the output is assigned to the block availability flag availableB2.

- If availableB2 is equal to TRUE and RefIdxL0[ xNbB2 ][ yNbB2 ] indicates that the current picture is used as the reference picture, availability checking process for block vector on current picture referencing as specified in clause 8.3.2.16 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the neighbouring motion vector mvL0B2 as inputs, and the output is assigned to the CPR availability flag availableFlagCPRB2.

- The variables availableFlagB2, refIdxLXB2, predFlagLXB2 and mvLXB2 are derived as follows:

- If one or more of the following conditions are true, availableFlagB2 is set equal to 0, both components of mvLXB2 are set equal to 0, refIdxLXB2 is set equal to −1 and predFlagLXB2 is set equal to 0, with X being 0 or 1, and gbiIdxB2 is set equal to 0:

  - availableB2 is equal to FALSE.

  - availableFlagCPRB2 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB2, yNbB2 ) have the same motion vectors and the same reference indices.

  - availableB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB2, yNbB2 ) have the same motion vectors and the same reference indices.

  - availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4 and merge_triangle_flag[ xCb ][ yCb ] is equal to 0.

- Otherwise, availableFlagB2 is set equal to 1 and the following assignments are made:

mvLXB2 = MvLX[ xNbB2 ][ yNbB2 ]		(8-225)

refIdxLXB2 = RefIdxLX[ xNbB2 ][ yNbB2 ]		(8-226)

predFlagLXB2 = PredFlagLX[ xNbB2 ][ yNbB2 ]		(8-227)

gbiIdxB2 = GbiIdx[ xNbB2 ][ yNbB2 ]		(8-228)

Derivation process for pairwise average merging candidates

Inputs to this process are:

- a merging candidate list mergeCandList,

- the reference indices refIdxL0N and refIdxL1N of every candidate N in mergeCandList,

- the prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N in mergeCandList,

- the motion vectors in 1/16 fractional-sample accuracy mvL0N and mvL1N of every candidate N in mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the number of elements numOrigMergeCand within the mergeCandList before pairwise average merging candidate derivation process.

Outputs of this process are:

- the merging candidate list mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the reference indices refIdxL0avgCandk and refIdxL1avgCandk of every new candidate avgCandk added into mergeCandList during the invocation of this process,

- the prediction list utilization flags predFlagL0avgCandk and predFlagL1avgCandk of every new candidate avgCandk added into mergeCandList during the invocation of this process,

- the motion vectors in 1/16 fractional-sample accuracy mvL0avgCandk and mvL1avgCandk of every new candidate avgCandk added into mergeCandList during the invocation of this process.

The variables numCutoff, avgIdx, avgStop, numRefLists are derived as follows:

numCutoff = Min( numCurrMergeCand, 4 ) 	(8-229)

avgIdx = 0	(8-230)

avgStop = (avgIdx >= ( numCutoff * ( numCutoff – 1 ) / 2 )  ||  	(8-231)
	numCurrMergeCand  = =  MaxNumMergeCand )

numRefLists  =  ( slice_type = = B )  ?  2 : 1	(8-232)

The following ordered steps are repeated until avgStop is equal to TRUE:

- The variables p0CandIdx and p1CandIdx are derived using avgIdx as specified in Table 8-7.

- The following assignments are made, with p0Cand being the candidate at position p0CandIdx and p1Cand being the candidate at position p1CandIdx in the merging candidate list mergeCandList:

p0Cand = mergeCandList[ p0CandIdx ]	(8-233)

p1Cand = mergeCandList[ p1CandIdx ]	(8-234)

- If neither or both of pNCand  (N is 0 or 1) use the current picture as the reference picture, [Ed. (SL): this condition allows pairwise average if both candidates are CPR, as implemented in current VTM. Another option could be disallowing CPR for pairwise average at all. Then the text should be changed to “If neither of pNCand (N is 0 or 1) uses current picture as the reference picture, “ Yet another option could be to allow CPR-inter and conventional inter pairwise average, then this condition can be totally eliminated, i.e. no text change here.] the candidate avgCandk with k equal to ( numCurrMergeCand – numOrigMergeCand ) is added at the end of mergeCandList, i.e., mergeCandList[ numCurrMergeCand ] is set equal to avgCandk, and the reference indices, the prediction list utilization flags and the motion vectors of avgCandk are derived as follows and numCurrMergeCand is incremented by 1:

- For each reference picture list LX with X ranging from 0 to ( numRefLists – 1 ), the following applies:

  - If predFlagLXp0Cand is equal to 1 and predFlagLXp1Cand is equal to 1, the variables refIdxLXavgCandk, predFlagLXavgCandk, mvLXavgCandk[ 0 ], and mvLXavgCandk[ 1 ] are derived as follows:

refIdxLXavgCandk = refIdxLXp0Cand	(8-235)

predFlagLXavgCandk = 1	(8-236)

mvLXavgCandk[ 0 ] = ( mvLXp0Cand[ 0 ] + mvLXp1Cand[ 0 ] ) / 2	(8-237)

mvLXavgCandk[ 1 ] = ( mvLXp0Cand[ 1 ] + mvLXp1Cand[ 1 ] ) / 2	(8-238)

If both refIdxLXp0Cand and refIdxLXp1Cand indicate that the current picture is used as the reference picture, the follow applies:

mvLXavgCandk[ 0 ] = ( mvLXavgCandk [ 0 ] / 16 ) << 4	(8-xxx)

mvLXavgCandk[ 1 ] = ( mvLXavgCandk [ 1 ] / 16 ) << 4	(8-xxx)

  - Otherwise, if predFlagLXp0Cand is equal to 1 and predFlagLXp1Cand is equal to 0, the variables refIdxLXavgCandk, predFlagLXavgCandk, mvLXavgCandk[ 0 ], mvLXavgCandk[ 1 ] are derived as follows:

refIdxLXavgCandk = refIdxLXp0Cand	(8-239)

predFlagLXavgCandk = 1	(8-240)

mvLXavgCandk[ 0 ] = mvLXp0Cand[ 0 ] 	(8-241)

mvLXavgCandk[ 1 ] = mvLXp0Cand[ 1 ] 	(8-242)

  - Otherwise, if predFlagLXp0Cand is equal to 0 and predFlagLXp1Cand is equal to 1, the variables refIdxLXavgCandk, predFlagLXavgCandk, mvLXavgCandk[ 0 ], mvLXavgCandk[ 1 ] are derived as follows:

refIdxLXavgCandk = refIdxLXp1Cand	(8-243)

predFlagLXavgCandk = 1	(8-244)

mvLXavgCandk[ 0 ] = mvLXp1Cand[ 0 ] 	(8-245)

mvLXavgCandk[ 1 ] = mvLXp1Cand[ 1 ] 	(8-246)

  - Otherwise, if predFlagLXp0Cand is equal to 0 and predFlagLXp1Cand is equal to 0, the variables refIdxLXavgCandk, predFlagLXavgCandk, mvLXavgCandk[ 0 ], mvLXavgCandk[ 1 ] are derived as follows:

refIdxLXavgCandk = −1	(8-247)

predFlagLXavgCandk = 0	(8-248)

mvLXavgCandk[ 0 ] = 0	(8-249)

mvLXavgCandk[ 1 ] = 0	(8-250)

- When numRefLists is equal to 1, the following applies:

refIdxL1avgCandk = −1	(8-251)

predFlagL1avgCandk = 0	(8-252)

- The variable avgIdx is incremented by 1.

- When avgIdx is equal to ( numCutoff * ( numCutoff – 1 ) / 2 ) or numCurrMergeCand is equal to MaxNumMergeCand, avgStop is set equal to TRUE.

Table 8-7 – Specification of p0CandIdx and p1CandIdx

| avgIdx | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| p0CandIdx | 0 | 0 | 1 | 0 | 1 | 2 |
| p1CandIdx | 1 | 2 | 2 | 3 | 3 | 3 |

Derivation process for zero motion vector merging candidates

Inputs to this process are:

- a merging candidate list mergeCandList,

- the reference indices refIdxL0N and refIdxL1N of every candidate N in mergeCandList,

- the prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N in mergeCandList,

- the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList.

Outputs of this process are:

- the merging candidate list mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm of every new candidate zeroCandm added into mergeCandList during the invocation of this process,

- the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm of every new candidate zeroCandm added into mergeCandList during the invocation of this process,

- the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm added into mergeCandList during the invocation of this process.

The variable numRefIdx is derived as follows:

- If slice_type is equal to P, numRefIdx is set equal to num_ref_idx_l0_active_minus1 + 1.

- Otherwise (slice_type is equal to B), numRefIdx is set equal to Min( num_ref_idx_l0_active_minus1 + 1, num_ref_idx_l1_active_minus1 + 1 ).

When numCurrMergeCand is less than MaxNumMergeCand, the variable numInputMergeCand is set equal to numCurrMergeCand, the variable zeroIdx is set equal to 0 and the following ordered steps are repeated until numCurrMergeCand is equal to MaxNumMergeCand:

- For the derivation of the reference indices, the prediction list utilization flags and the motion vectors of the zero motion vector merging candidate, the following applies:

  - If slice_type is equal to P, the candidate zeroCandm with m equal to ( numCurrMergeCand – numInputMergeCand ) is added at the end of mergeCandList, i.e., mergeCandList[ numCurrMergeCand ] is set equal to zeroCandm, and the reference indices, the prediction list utilization flags and the motion vectors of zeroCandm are derived as follows and numCurrMergeCand is incremented by 1:

refIdxL0zeroCandm = ( zeroIdx < numRefIdx ) ? zeroIdx : 0	(8-253)

refIdxL1zeroCandm = −1	(8-254)

predFlagL0zeroCandm = 1	(8-255)

predFlagL1zeroCandm = 0	(8-256)

mvL0zeroCandm[ 0 ] = 0	(8-257)

mvL0zeroCandm[ 1 ] = 0	(8-258)

mvL1zeroCandm[ 0 ] = 0	(8-259)

mvL1zeroCandm[ 1 ] = 0	(8-260)

numCurrMergeCand = numCurrMergeCand + 1	(8-261)

  - Otherwise (slice_type is equal to B), the candidate zeroCandm with m equal to ( numCurrMergeCand – numInputMergeCand ) is added at the end of mergeCandList, i.e., mergeCandList[ numCurrMergeCand ] is set equal to zeroCandm, and the reference indices, the prediction list utilization flags and the motion vectors of zeroCandm are derived as follows and numCurrMergeCand is incremented by 1:

refIdxL0zeroCandm = ( zeroIdx < numRefIdx ) ? zeroIdx : 0	(8-262)

refIdxL1zeroCandm = ( zeroIdx < numRefIdx ) ? zeroIdx : 0	(8-263)

predFlagL0zeroCandm = 1	(8-264)

predFlagL1zeroCandm = 1	(8-265)

mvL0zeroCandm[ 0 ] = 0	(8-266)

mvL0zeroCandm[ 1 ] = 0	(8-267)

mvL1zeroCandm[ 0 ] = 0	(8-268)

mvL1zeroCandm[ 1 ] = 0	(8-269)

numCurrMergeCand = numCurrMergeCand + 1	(8-270)

- The variable zeroIdx is incremented by 1.

Derivation process for history-based merging candidates

Inputs to this process are:

- a merge candidate list mergeCandList,

- the number of available merging candidates in the list numCurrMergeCand.

Outputs to this process are:

- the modified merging candidate list mergeCandList,

- the modified number of merging candidates in the list numCurrMergeCand.

Each element of the array isPruned[ i ] with i = 0..numCurrMergeCand – 1 is set equal to FALSE and the variable numOrigMergeCand is set equal to numCurrMergeCand.

For each candidate in HmvpCandList[ hMvpIdx ] with index hMvpIdx = 1..NumHmvpCand, the following ordered steps are repeated until numCurrMergeCand is equal to ( MaxNumMergeCand – 1):

- The variable sameMotion is derived as follows:

  - If all of the following conditions are true for any i = 0..numOrigMergeCand – 1, sameMotion and isPruned[ i ] are both set equal to TRUE:

  - The candidate HmvpCandList[NumHmvpCand – hMvpIdx] is equal to the merging candidate mergeCandList[ i ].

  - isPruned[ i ] is equal to FALSE.

  - Otherwise, sameMotion is set equal to FALSE.

- When sameMotion is equal to FALSE, the candidate HmvpCandList[NumHmvpCand – hMvpIdx] is added to the merging candidate list as follows:

mergeCandList[ numCurrMergeCand++ ] = HmvpCandList[ NumHmvpCand – hMvpIdx ] 	(8-271)

Derivation process for merge motion vector difference

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

Outputs of this process are the luma merge motion vector differences in 1/16 fractional-sample accuracy mMvdL0 and mMvdL1.

The variable currPic specifies the current picture.

The luma merge motion vector differences mMvdL0 and mMvdL1 are derived as follows:

- If both predFlagL0 and predFlagL1 are equal to 1, the following applies:

currPocDiffL0  =  DiffPicOrderCnt( currPic, RefPicList0[ refIdxL0 ] )	(8-272)

currPocDiffL1  =  DiffPicOrderCnt( currPic, RefPicList1[ refIdxL1 ] )	(8-273)

- If −currPocDiffL0 * currPocDiffL1 is greater than 0, the following applies:

mMvdL0[ 0 ]  =  MmvdOffset[ 0 ]		(8-274)

mMvdL0[ 1 ]  =  MmvdOffset[ 1 ]		(8-275)

mMvdL1[ 0 ]  =  −MmvdOffset[ 0 ]		(8-276)

mMvdL1[ 1 ]  =  −MmvdOffset[ 1 ]		(8-277)

- Otherwise ( −currPocDiffL0 * currPocDiffL1 is less than 0 ), the following applies:

mMvdL0[ 0 ]  =  MmvdOffset[ 0 ]		(8-278)

mMvdL0[ 1 ]  =  MmvdOffset[ 1 ]		(8-279)

mMvdL1[ 0 ]  =  MmvdOffset[ 0 ]		(8-280)

mMvdL1[ 1 ]  =  MmvdOffset[ 1 ]		(8-281)

- If Abs( currPocDiffL0 ) is greater than Abs( currPocDiffL1 ), the following applies:

td = Clip3( −128, 127, currPocDiffL0 )		(8-282)

tb = Clip3( −128, 127, currPocDiffL1 )		(8-283)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(8-284)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-285)

mMvdL1[ 0 ] = Clip3( −215, 215 – 1, Sign( distScaleFactor * mMvdL1[ 0 ] )  *  	(8-286)
	( ( Abs( distScaleFactor * mMvdL1[ 0 ] ) + 127 )  >>  8 ) )

mMvdL1[ 1 ] = Clip3( −215, 215 – 1, Sign( distScaleFactor * mMvdL1[ 1 ] )  *  	(8-287)
	( ( Abs( distScaleFactor * mMvdL1[ 1 ] ) + 127 )  >>  8 ) )

- Otherwise if Abs( currPocDiffL0 ) is less than Abs( currPocDiffL0 ), the following applies:

td = Clip3( −128, 127, currPocDiffL1 )		(8-288)

tb = Clip3( −128, 127, currPocDiffL0 )		(8-289)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(8-290)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-291)

mMvdL0[ 0 ] = Clip3( −215, 215 – 1, Sign( distScaleFactor * mMvdL0[ 0 ] )  *  	(8-292)
	( ( Abs( distScaleFactor * mMvdL0[ 0 ] ) + 127 )  >>  8 ) )

mMvdL0[ 1 ] = Clip3( −215, 215 – 1, Sign( distScaleFactor * mMvdL0[ 1 ] )  *  	(8-293)
	( ( Abs( distScaleFactor * mMvdL0[ 1 ] ) + 127 )  >>  8 ) )

- Otherwise ( predFlagL0 or predFlagL1 are equal to 1 ), the following applies for X being 0 and 1:

mMvdLX[ 0 ] = ( predFlagLX = = 1 )  ?  MmvdOffset[ 0 ]  :  0	(8-294)

mMvdLX[ 1 ] = ( predFlagLX = = 1 )  ?  MmvdOffset[ 1 ]  :  0	(8-295)

Derivation process for luma motion vector prediction

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- the reference index of the current coding unit partition refIdxLX, with X being 0 or 1.

Output of this process is the prediction mvpLX in 1/16 fractional-sample accuracy of the motion vector mvLX, with X being 0 or 1.

The motion vector predictor mvpLX with X being 0 or 1  is derived in the following ordered steps:

- The derivation process for motion vector predictor candidate list as specified in clause 8.3.2.9 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and refIdxLX, with X being 0 or 1 as inputs, and the motion vector predictor candidate list, mvpListLX with X being 0 or 1, as output.

- The motion vector predictor mvpLX with X being 0 or 1 is derived as follows:

mvpLX = mvpListLX[ mvp_lX_flag[ xCb ][ yCb ] ] 	(8-296)

Derivation process for motion vector predictor candidate list

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- the reference index of the current coding unit partition refIdxLX, with X being 0 or 1.

Output of this process is motion vector predictor candidate list mvpListLX in 1/16 fractional-sample accuracy with X being 0 or 1.

The motion vector predictor candidate list mvpListLX with X being 0 or 1 is derived in the following ordered steps:

- The derivation process for spatial motion vector predictor candidates from neighbouring coding unit partitions as specified in clause 8.3.2.10 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and refIdxLX, with X being 0 or 1 as inputs, and the availability flags availableFlagLXN and the motion vectors mvLXN, with N being replaced by A or B, as output.

- The rounding process for motion vectors as specified in clause 8.3.2.14 is invoked the with mvX set equal to mvLXN, with N being replaced by A or B, rightShift set equal to MvShift + 2, and leftShift set equal to MvShift + 2 as inputs and the rounded mvLXN, with N being replaced by A or B, as output.

- If both availableFlagLXA and availableFlagLXB are equal to 1 and mvLXA is not equal to mvLXB, availableFlagLXCol is set equal to 0.

- Otherwise, the following applies:

- The derivation process for temporal luma motion vector prediction as specified in clause 8.3.2.11 is with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and refIdxLX, with X being 0 or 1 as inputs, and with the output being the availability flag availableFlagLXCol and the temporal motion vector predictor mvLXCol.

- The rounding process for motion vectors as specified in clause 8.3.2.14 is invoked the with mvX set equal to mvLXCol, rightShift set equal to MvShift + 2, and leftShift set equal to MvShift + 2 as inputs and the rounded mvLXCol as output.

- The motion vector predictor candidate list, mvpListLX, is constructed as follows:

numCurrMvpCand = 0
if( availableFlagLXA ) {
	mvpListLX[ numCurrMvpCand++ ] = mvLXA
	if( availableFlagLXB  &&  ( mvLXA  !=  mvLXB ) )
		mvpListLX[ numCurrMvpCand++ ] = mvLXB	(8-297)
} else if( availableFlagLXB )
	mvpListLX[ numCurrMvpCand++ ] = mvLXB
if( numCurrMvpCand < 2  &&  availableFlagLXCol )
	mvpListLX[ numCurrMvpCand++ ] = mvLXCol

- When numCurrMvpCand is less than 2 and NumHmvpCand is greater than 0, the following applies for i= 1..Min( 4, NumHmvpCand ) until numCurrMvpCand is equal to 2:

- For each reference picture list LY with Y = 0..1, the following applies until numCurrMvpCand is equal to 2:

- When all of the following conditions are true, mvpListLX[ numCurrMvpCand++ ] is set to equal to the LY motion vector of the candidate HmvpCandList[ NumHmvpCand – i ]:

- The LY reference index of the history-based motion vector predictor candidate HmvpCandList[i] is equal to refIdxLX

- The motion vector predictor candidate list is empty (numCurrMvpCan is equal to 0) or the LY motion vector of the candidate HmvpCandList[ NumHmvpCand – i ] is not equal to mvpListLX[j] for j = 0.. numCurrMvpCand – 1.

- If numCurrMvpCand is greater than 0 and DiffPicOrderCnt( currPic, refPicList0[ refIdxLX ] ) is equal to 0, the following applies:

- The variable numOrgMvpCand is set to numCurrMvpCand.

- for i= 0..Min( 2, numCurrMvpCand )

  - Availability checking process for block vector on current picture referencing as specified in clause 8.3.2.16 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the neighbouring motion vector mvpListLX[ i ] as inputs, and the output is assigned to the CPR availability flag availableFlagCandi.

  - If availableFlagCandi is equal to FALSE, numOrgMvpCand is decremented by 1.

- numCurrMvpCand is set to numOrgMvpCand.

- When numCurrMergeCand is less than 2, the following applies:

- The derivation process for default candidates on current picture referencing specified in clause 8.3.2.17 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight, mvpListLX, numCurrMvpCand, the maximum number of candidates 2 as inputs, and modified mvpListLX and numCurrMvpCand as outputs.

- When numCurrMvpCand is less than 2, the following applies for until numCurrMvpCand is equal to 2:

mvpListLX[ numCurrMvpCand ][ 0 ] = 0		(8-298)

mvpListLX[ numCurrMvpCand ][ 1 ] = 0		(8-299)

numCurrMvpCand++		(8-300)

Derivation process for spatial motion vector predictor candidates

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- the reference index of the current coding unit partition refIdxLX, with X being 0 or 1.

Outputs of this process are (with N being replaced by A or B):

- the motion vectors mvLXN in 1/16 fractional-sample accuracy of the neighbouring coding units,

- the availability flags availableFlagLXN of the neighbouring coding units.

Figure 8-2 provides an overview of spatial motion vector neighbours.

Figure 8-2 – Spatial motion vector neighbours (informative)

The variable currCb specifies the current luma coding block at luma location ( xCb, yCb ) and the variable currPic specifies the current picture.

The variable isScaledFlagLX, with X being 0 or 1, is set equal to 0.

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- The sample location ( xNbA0, yNbA0 ) is set equal to ( xCb – 1, yCb + cbHeight ) and the sample location ( xNbA1, yNbA1 ) is set equal to ( xNbA0, yNbA0 – 1 ).

- The availability flag availableFlagLXA is set equal to 0 and both components of mvLXA are set equal to 0.

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA0, yNbA0 ) as inputs, and the output is assigned to the block availability flag availableA0.

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA1, yNbA1 ) as inputs, and the output is assigned to the block availability flag availableA1.

- When availableA0 or availableA1 is equal to TRUE, the variable isScaledFlagLX is set equal to 1.

- The following applies for ( xNbAk, yNbAk ) from ( xNbA0, yNbA0 ) to ( xNbA1, yNbA1 ):

- When availableAk is equal to TRUE and availableFlagLXA is equal to 0, the following applies:

- If PredFlagLX[ xNbAk ][ yNbAk ] is equal to 1 and DiffPicOrderCnt( RefPicListX[ RefIdxLX[ xNbAk ][ yNbAk ] ], RefPicListX[ refIdxLX ] ) is equal to 0, availableFlagLXA is set equal to 1 and the following applies:

mvLXA = MvLX[ xNbAk ][ yNbAk ]	(8-301)

- Otherwise, when PredFlagLY[ xNbAk ][ yNbAk ] (with Y = !X) is equal to 1 and DiffPicOrderCnt( RefPicListY[ RefIdxLY[ xNbAk ][ yNbAk ] ], RefPicListX[ refIdxLX ] ) is equal to 0, availableFlagLXA is set equal to 1 and the following applies:

mvLXA = MvLY[ xNbAk ][ yNbAk ]	(8-302)

- When availableFlagLXA is equal to 0, the following applies for ( xNbAk, yNbAk ) from ( xNbA0, yNbA0 ) to ( xNbA1, yNbA1 ) or until availableFlagLXA is equal to 1:

- When availableAk is equal to TRUE and availableFlagLXA is equal to 0, the following applies:

- If PredFlagLX[ xNbAk ][ yNbAk ] is equal to 1 , availableFlagLXA is set equal to 1 and the following assignments are made:

mvLXA = MvLX[ xNbAk ][ yNbAk ]	(8-303)

refIdxA = RefIdxLX[ xNbAk ][ yNbAk ]	(8-304)

refPicListA = RefPicListX	(8-305)

- Otherwise, if PredFlagLY[ xNbAk ][ yNbAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1 and the following assignments are made:

mvLXA = MvLY[ xNbAk ][ yNbAk ]	(8-306)

refIdxA = RefIdxLY[ xNbAk ][ yNbAk ]	(8-307)

refPicListA = RefPicListY	(8-308)

- When availableFlagLXA is equal to 1, DiffPicOrderCnt( refPicListA[ refIdxA ], RefPicListX[ refIdxLX ] ) is not equal to 0, mvLXA is derived as follows:

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td	(8-309)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-310)

mvLXA = Clip3( −32768, 32767, Sign( distScaleFactor * mvLXA ) * 
				( ( Abs( distScaleFactor * mvLXA ) + 127 )  >>  8 ) )	(8-311)

where td and tb are derived as follows:

td = Clip3( −128, 127, DiffPicOrderCnt( currPic, refPicListA[ refIdxA ] ) )	(8-312)

tb = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] ) )	(8-313)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- The sample locations ( xNbB0, yNbB0 ), ( xNbB1, yNbB1 ) and ( xNbB2, yNbB2 ) are set equal to ( xCb + cbWidth, yCb – 1 ), ( xCb + cbWidth – 1, yCb – 1 ) and ( xCb – 1, yCb – 1 ), respectively.

- The availability flag availableFlagLXB is set equal to 0 and the both components of mvLXB are set equal to 0.

- The following applies for ( xNbBk, yNbBk ) from ( xNbB0, yNbB0 ) to ( xNbB2, yNbB2 ):

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbBk, yNbBk ) as inputs, and the output is assigned to the block availability flag availableBk.

- When availableBk is equal to TRUE and availableFlagLXB is equal to 0, the following applies:

- If PredFlagLX[ xNbBk ][ yNbBk ] is equal to 1, and DiffPicOrderCnt( RefPicListX[ RefIdxLX[ xNbBk ][ yNbBk ] ], RefPicListX[ refIdxLX ] ) is equal to 0, availableFlagLXB is set equal to 1 and the following assignments are made:

mvLXB = MvLX[ xNbBk ][ yNbBk ]	(8-314)

refIdxB = RefIdxLX[ xNbBk ][ yNbBk ]	(8-315)

- Otherwise, when PredFlagLY[ xNbBk ][ yNbBk ] (with Y = !X) is equal to 1 and DiffPicOrderCnt( RefPicListY[ RefIdxLY[ xNbBk ][ yNbBk ] ], RefPicListX[ refIdxLX ] ) is equal to 0, availableFlagLXB is set equal to 1 and the following assignments are made:

mvLXB = MvLY[ xNbBk ][ yNbBk ]	(8-316)

refIdxB = RefIdxLY[ xNbBk ][ yNbBk ]	(8-317)

- When isScaledFlagLX is equal to 0 and availableFlagLXB is equal to 1, availableFlagLXA is set equal to 1 and the following applies:

mvLXA = mvLXB	(8-318)

- When isScaledFlagLX is equal to 0, availableFlagLXB is set equal to 0 and the following applies for ( xNbBk, yNbBk ) from ( xNbB0, yNbB0 ) to ( xNbB2, yNbB2 ) or until availableFlagLXB is equal to 1:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbBk, yNbBk ) as inputs, and the output is assigned to the block availability flag availableBk.

- When availableBk is equal to TRUE and availableFlagLXB is equal to 0, the following applies:

- If PredFlagLX[ xNbBk ][ yNbBk ] is equal to 1, availableFlagLXB is set equal to 1 and the following assignments are made:

mvLXB = MvLX[ xNbBk ][ yNbBk ]	(8-319)

refIdxB = RefIdxLX[ xNbBk ][ yNbBk ]	(8-320)

refPicListB = RefPicListX	(8-321)

- Otherwise, when PredFlagLY[ xNbBk ][ yNbBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1 and the following assignments are made:

mvLXB = MvLY[ xNbBk ][ yNbBk ]	(8-322)

refIdxB = RefIdxLY[ xNbBk ][ yNbBk ]	(8-323)

refPicListB = RefPicListY	(8-324)

- When availableFlagLXB is equal to 1, DiffPicOrderCnt( refPicListB[ refIdxB ], RefPicListX[ refIdxLX ] ) is not equal to 0, mvLXB is derived as follows:

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td	(8-325)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-326)

mvLXB = Clip3( −32768, 32767, Sign( distScaleFactor * mvLXB ) *
				( ( Abs( distScaleFactor * mvLXB ) + 127 )  >>  8 ) )	(8-327)

where td and tb are derived as follows:

td = Clip3( −128, 127, DiffPicOrderCnt( currPic, refPicListB[ refIdxB ] ) )	(8-328)

tb = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] ) )	(8-329)

Derivation process for temporal luma motion vector prediction

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a reference index refIdxLX, with X being 0 or 1.

Outputs of this process are:

- the motion vector prediction mvLXCol in 1/16 fractional-sample accuracy,

- the availability flag availableFlagLXCol.

The variable currCb specifies the current luma coding block at luma location ( xCb, yCb ).

The variables mvLXCol and availableFlagLXCol are derived as follows:

- If slice_temporal_mvp_enabled_flag is equal to 0, or if the reference picture is the current picture, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise (slice_temporal_mvp_enabled_flag is equal to 1 and the reference picture is not the current picture), the following ordered steps apply:

- The bottom right collocated motion vector is derived as follows:

xColBr = xCb + cbWidth	(8-330)

yColBr = yCb + cbHeight	(8-331)

- If yCb  >>  CtbLog2SizeY is equal to yColBr  >>  CtbLog2SizeY, yColBr is less than pic_height_in_luma_samples and xColBr is less than pic_width_in_luma_samples, the following applies:

- The variable colCb specifies the luma coding block covering the modified location given by ( ( xColBr  >>  3 )  <<  3, ( yColBr  >>  3 )  <<  3 ) inside the collocated picture specified by ColPic.

- The luma location ( xColCb, yColCb ) is set equal to the top-left sample of the collocated luma coding block specified by colCb relative to the top-left luma sample of the collocated picture specified by ColPic.

- The derivation process for collocated motion vectors as specified in clause 8.3.2.12 is invoked with currCb, colCb, ( xColCb, yColCb ), refIdxLX and sbFlag set equal to 0 as inputs, and the output is assigned to mvLXCol and availableFlagLXCol.

- Otherwise, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- When availableFlagLXCol is equal to 0, the central collocated motion vector is derived as follows:

xColCtr = xCb + ( cbWidth  >>  1 )	(8-332)

yColCtr = yCb + ( cbHeight  >>  1 )	(8-333)

- The variable colCb specifies the luma coding block covering the modified location given by ( ( xColCtr  >>  3 )  <<  3, ( yColCtr  >>  3 )  <<  3 ) inside the collocated picture specified by ColPic.

- The luma location ( xColCb, yColCb ) is set equal to the top-left sample of the collocated luma coding block specified by colCb relative to the top-left luma sample of the collocated picture specified by ColPic.

- The derivation process for collocated motion vectors as specified in clause 8.3.2.12 is invoked with currCb, colCb, ( xColCb, yColCb ), refIdxLX and sbFlag set equal to 0 as inputs, and the output is assigned to mvLXCol and availableFlagLXCol.

Derivation process for collocated motion vectors

Inputs to this process are:

- a variable currCb specifying the current coding block,

- a variable colCb specifying the collocated coding block inside the collocated picture specified by ColPic,

- a luma location ( xColCb, yColCb ) specifying the top-left sample of the collocated luma coding block specified by colCb relative to the top-left luma sample of the collocated picture specified by ColPic,

- a reference index refIdxLX, with X being 0 or 1,

- a flag indicating a subblock temporal merging candidate sbFlag.

Outputs of this process are:

- the motion vector prediction mvLXCol in 1/16 fractional-sample accuracy,

- the availability flag availableFlagLXCol.

The variable currPic specifies the current picture.

The arrays predFlagL0Col[ x ][ y ], mvL0Col[ x ][ y ] and refIdxL0Col[ x ][ y ] are set equal to PredFlagL0[ x ][ y ], MvL0[ x ][ y ] and RefIdxL0[ x ][ y ], respectively, of the collocated picture specified by ColPic, and the arrays predFlagL1Col[ x ][ y ], mvL1Col[ x ][ y ] and refIdxL1Col[ x ][ y ] are set equal to PredFlagL1[ x ][ y ], MvL1[ x ][ y ] and RefIdxL1[ x ][ y ], respectively, of the collocated picture specified by ColPic.

[Ed. (BB): Define ColPic NoBackwardPredFlag.]

The variables mvLXCol and availableFlagLXCol are derived as follows:

- If colCb is coded in an intra prediction mode, or its reference picture is ColPic, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise, the motion vector mvCol, the reference index refIdxCol and the reference list identifier listCol are derived as follows:

  - If sbFlag is equal to 0, availableFlagLXCol is set to 1 and the following applies:

  - If predFlagL0Col[ xColCb ][ yColCb ] is equal to 0, mvCol, refIdxCol and listCol are set equal to mvL1Col[ xColCb ][ yColCb ], refIdxL1Col[ xColCb ][ yColCb ] and L1, respectively.

  - Otherwise, if predFlagL0Col[ xColCb ][ yColCb ] is equal to 1 and predFlagL1Col[ xColCb ][ yColCb ] is equal to 0, mvCol, refIdxCol and listCol are set equal to mvL0Col[ xColCb ][ yColCb ], refIdxL0Col[ xColCb ][ yColCb ] and L0, respectively.

  - Otherwise (predFlagL0Col[ xColCb ][ yColCb ] is equal to 1 and predFlagL1Col[ xColCb ][ yColCb ] is equal to 1), the following assignments are made:

      - If NoBackwardPredFlag is equal to 1, mvCol, refIdxCol and listCol are set equal to mvLXCol[ xColCb ][ yColCb ], refIdxLXCol[ xColCb ][ yColCb ] and LX, respectively.

      - Otherwise, mvCol, refIdxCol and listCol are set equal to mvLNCol[ xColCb ][ yColCb ], refIdxLNCol[ xColCb ][ yColCb ] and LN, respectively, with N being the value of collocated_from_l0_flag.

  - Otherwise (sbFlag is equal to 1), the following applies:

  - If PredFlagLXCol[ xColCb ][ yColCb ] is equal to 1, mvCol, refIdxCol, and listCol are set equal to mvLXCol[ xColCb ][ yColCb ], refIdxLXCol[ xColCb ][ yColCb ], and LX, respectively, availableFlagLXCol is set to 1.

  - Otherwise (PredFlagLXCol[ xColCb ][ yColCb ] is equal to 0), the following applies:

      - If DiffPicOrderCnt( aPic, currPic ) is less than or equal to 0 for every picture aPic in every reference picture list of the current slice and PredFlagLYCol[ xColCb ][ yColCb ] is equal to 1, mvCol, refIdxCol, and listCol are set to mvLYCol[ xColCb ][ yColCb ], refIdxLYCol[ xColCb ][ yColCb ] and LY, respectively, with Y being equal to !X where X being the value of X this process is invoked for. availableFlagLXCol is set to 1.

      - Both the components of mvLXCol are set to 0 and availableFlagLXCol is set equal to 0.

  - When availableFlagLXCol is equal to TRUE, mvLXCol and availableFlagLXCol are derived as follows:

  - If LongTermRefPic( currPic, currCb, refIdxLX, LX ) is not equal to LongTermRefPic( ColPic, colCb, refIdxCol, listCol ), both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

  - Otherwise, the variable availableFlagLXCol is set equal to 1, refPicListCol[ refIdxCol ] is set to be the picture with reference index refIdxCol in the reference picture list listCol of the slice containing coding block colCb in the collocated picture specified by ColPic, and the following applies:

colPocDiff = DiffPicOrderCnt( ColPic, refPicListCol[ refIdxCol ] )	(8-334)

currPocDiff = DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] )	(8-335)

      - If RefPicListX[ refIdxLX ] is a long-term reference picture, or colPocDiff is equal to currPocDiff, mvLXCol is derived as follows:

mvLXCol = mvCol	(8-336)

      - Otherwise, mvLXCol is derived as a scaled version of the motion vector mvCol as follows:

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td	(8-337)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-338)

mvLXCol =  Clip3( −32768, 32767, Sign( distScaleFactor * mvCol ) * 
				( ( Abs( distScaleFactor * mvCol ) + 127 )  >>  8 ) )	(8-339)

where td and tb are derived as follows:

td = Clip3( −128, 127, colPocDiff )	(8-340)

tb = Clip3( −128, 127, currPocDiff )	(8-341)

Derivation process for chroma motion vectors

Input to this process are a luma motion vector in 1/16 fractional-sample accuracy mvLX and the reference index refIdxLX.

Output of this process is a chroma motion vector in 1/32 fractional-sample accuracy mvCLX.

A chroma motion vector is derived from the corresponding luma motion vector.

For the derivation of the chroma motion vector mvCLX, the following applies:

- If the reference picture with index refIdxLX from reference picture list LX of the slice is not the current picture,

mvCLX[ 0 ] = mvLX[ 0 ] * 2 / SubWidthC	(8-342)

mvCLX[ 1 ] = mvLX[ 1 ] * 2 / SubHeightC	(8-343)

- Otherwise

mvCLX[ 0 ] = ( ( mvLX[ 0 ] >> ( 3 + SubWidthC ) ) * 32	(8-xxx)

mvCLX[ 1 ] = ( ( mvLX[ 1 ] >> ( 3 + SubHeightC ) ) * 32	(8-xxx)

Rounding process for motion vectors

Inputs to this process are

- the motion vector mvX,

- the right shift parameter rightShift for rounding,

- the left shift parameter leftShift for resolution increase.

Output of this process is the rounded motion vector mvX.

For the rounding of mvX, the following applies:

offset = 1 << ( rightShift – 1 )		(8-344)

mvX[ 0 ] = ( mvX[ 0 ] >= 0 ? ( mvX[ 0 ] + offset) >> rightShift : 	
				− ( (− mvX[ 0 ] + offset ) >>rightShift ) ) << leftShift	(8-345)

mvX[ 1 ] = ( mvX[ 1 ] >= 0 ? ( mvX[ 1 ] + offset) >> rightShift : 	
				− ( (− mvX[ 1 ] + offset ) >> rightShift ) ) << leftShift	(8-346)

Updating process for the history-based motion vector predictor candidate list

Inputs to this process are:

- luma motion vectors in 1/16 fractional-sample accuracy mvL0 and mvL1,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

The MVP candidate hMvpCand consists of the luma motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction list utilization flags predFlagL0 and predFlagL1, and the bi-prediction weight index gbiIdx set equal to 0.

The candidate list HmvpCandList is modified using the candidate hMvpCand by the following ordered steps:

- The variable identicalCandExist is set equal to FALSE and the variable removeIdx is set equal to 0.

- When NumHmvpCand is greater than 0, for each index hMvpIdx with hMvpIdx = 0..NumHmvpCand – 1, the following steps apply until identicalCandExist is equal to TRUE:

  - When hMvpCand is equal to HmvpCandList[ hMvpIdx ], identicalCandExist is set equal to TRUE and removeIdx is set equal to hMvpIdx.

- The candidate list HmvpCandList is updated as follows:

  - If identicalCandExist is equal to TRUE or NumHmvpCand is equal to 6, the following applies:

  - For each index i with i = ( removeIdx + 1 )..( NumHmvpCand – 1 ), HmvpCandList[ i – 1] is set equal to HmvpCandList[ i ].

  - HmvpCandList[ NumHmvpCand – 1 ] is set equal to mvCand.

  - Otherwise (identicalCandExist is equal to FALSE and NumHmvpCand is less than 6), the following applies:

  - HmvpCandList[ NumHmvpCand++ ] is set equal to mvCand.

[Ed. (BB): Given that the maximum number of merging candidates is restricted to 6, the 1st HMVP candidate (HmvpCandList[ 0 ]) is never used and thus the maximum number of HMVP candidates could be reduced to 5 without any impact.]

Availability checking process for block vector on current picture referencing

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- luma block vectors in full-sample accuracy xBv and yBv,

Output to this process is:

- the availability flag availableFlagCPR

- If one or more of the following conditions are true, availableFlagCPR is set equal to 0:

  - xCb  + xBv is less than 0.

  - xCb  + xBv + cbWidth − 1 is greater than or equal to pic_width_in_luma_samples.

  - yCb  + yBv is less than 0.

  - yCb  + yBv + cbHeight − 1 is greater than or equal to pic_height_in_luma_samples.

  - xBv + cbWidth and yBv + cbHeight are greater than 0.

  - (yCb  + yBv) >> CtbLog2SizeY is less than yCb >> CtbLog2SizeY.

  - (yCb  + yBv + cbHeight − 1) >> CtbLog2SizeY is greater than yCb >> CtbLog2SizeY.

- Otherwise, if (xCb  + xBv + cbWidth – 1) >> CtbLog2SizeY is less than or equal to xCb >> CtbLog2SizeY and (xCb  + xBv) >> CtbLog2SizeY is greater than or equal to xCb >> CtbLog2SizeY, the following applies:

    - When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xCb + xBv, yCb + yBv ) as inputs, and the output is assigned to the availability flag availableFlagTL.

    - When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xCb + xBv + cbWidth – 1, yCb + yBv + cbHeight − 1) as inputs, and the output is assigned to the availability flag availableFlagBR.

    - When availableFlagTL or availableFlagBR is equal to FALSE, the availability flag availableFlagCPR is set equal to 0.

- Otherwise, the availability flag availableFlagCPR is set equal to 1.

Derivation process for default candidates on current picture referencing

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a candidate list CandList,

- the number of elements numCurrCand within CandList,

- the maximum number of candidates MaxNumCand.

Output to this process is:

- a candidate list CandList,

- the number of elements numCurrCand within CandList.

The variable numRefIdx is set equal to num_ref_idx_l0_active_minus1 + 1.

- If DiffPicOrderCnt( currPic, refPicList0[ numRefIdx – 1 ] ) is equal to 0 and numCurrCand is less than MaxNumCand and xCb % CtbSizeY is greater than or equal to cbWidth, the following applies:

refIdxL0defaultCandm = numRefIdx − 1	(8-xxx)

refIdxL1defaultCandm = −1	(8-xxx)

predFlagL0defaultCandm = 1	(8-xxx)

predFlagL1defaultCandm = 0	(8-xxx)

mvL0defaultCandm[ 1 ] = 0	(8-xxx)

mvL1defaultCandm[ 0 ] = 0	(8-xxx)

mvL1defaultCandm[ 1 ] = 0	(8-xxx)

numCurrCand = numCurrCand + 1	(8-xxx)

  - If xCb % CtbSizeY is less than (cbWidth << 1),

mvL0defaultCandm[ 0 ] = ((xCb % CtbSizeY) << 4)	(8-xxx)

  - Otherwise,

mvL0defaultCandm[ 0 ] = (cbWidth << 5)	(8-xxx)

- If DiffPicOrderCnt( currPic, refPicList0[ numRefIdx – 1 ] ) is equal to 0 and numCurrCand is less than MaxNumCand and yCb % CtbSizeY is greater than or equal to cbHeight, the following applies:

refIdxL0defaultCandm = numRefIdx − 1	(8-xxx)

refIdxL1defaultCandm = −1	(8-xxx)

predFlagL0defaultCandm = 1	(8-xxx)

predFlagL1defaultCandm = 0	(8-xxx)

mvL0defaultCandm[ 0 ] = 0	(8-xxx)

mvL1defaultCandm[ 0 ] = 0	(8-xxx)

mvL1defaultCandm[ 1 ] = 0	(8-xxx)

numCurrCand = numCurrCand + 1	(8-xxx)

  - If yCb % CtbSizeY is less than (cbHeight << 1),

mvL0defaultCandm[ 1 ] = ((yCb % CtbSizeY) << 4)	(8-xxx)

  - Otherwise,

mvL0defaultCandm[ 1 ] = (cbHeight << 5)	(8-xxx)