| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0341 |
| --- | --- |

| Title: | Non-CE8 : MMVD harmonization with CPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Seung-Hwan Kim Jayhyun Lim | Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution presents a simplified MMVD considering CPR mode. When the current decoded picture is the only reference picture for the current slice, mmvd_merge_flag is not signalled and also 4 indices for mmvd_distance_idx are allowed. From experimental results, following BD-rate changes are observed over VTM-3.0 with CPR :

- AI: 0.XX% (CTC), -0.13% (Class F), -0.3% (Class SCC)

- RA: 0.XX% (CTC), 0.XX% (Class F), -0.13% (Class SCC)

Problem Statement

When CPR is enabled in the SPS, I-Slice is treated as P-Slice to allow inter prediction coding tools; merge and AMVP. Current CPR mode signalling disallowed inter prediction coding tools even though CPR doesn’t work with inter coding tools at the same time. For example, syntaxes regarding with MMVD, Affine, Multi-hypothesis and Tri-angular prediction are signaled to indicate those coding tool do not work.

Proposed method

This contribution proposes simplified MMVD for CPR. When the current decoded picture is the only reference picture, proposed simplification  is allowed. Proposed MMVD simplification is that one base candidate is used without mmvd_merge_flag signaling to indicate which base candidate is used and only number of 4 mmvd difference is used and mmvd_distance_idx is signaled to indicate which mmvd distance is used among 4 distance index.

…

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if (!CurrPicIsOnlyRef) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if(MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if(sps_mh_intra_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| mh_intra_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| mh_intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| mh_intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if(sps_triangle_enabled_flag && slice_type = = B && cbWidth * cbHeight >= 16 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) |  |
| merge_triangle_idx[ x0 ][ y0 ] | ae(v) |
| else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |

Experimental result

The proposed methods have been implement on top of VTM-3.0 and experiment is tested under common test conditions in [1].

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 with CPR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | -0.05% | 101% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.02% | -0.08% | -0.03% | 102% | 100% |
| Class C | -0.01% | -0.01% | -0.07% | 103% | 101% |
| Class E | -0.06% | -0.01% | -0.07% | 102% | 100% |
| Overall |  |  |  | #NUM! | #NUM! |
| Class D | -0.02% | 0.10% | -0.21% | 103% | 99% |
| Class F | -0.13% | -0.01% | 0.13% | 104% | 100% |
| Class SCC | -0.30% | -0.22% | -0.26% | 105% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 with CPR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class SCC | Class SCC | -0.13% | -0.18% | -0.20% | 100% |

Reference

- S.Wang, X. Zheng, X. Zhang, S.Wang, S.Ma, “CE4-related: Simplification of ATMVP candidate derivation,” JVET-L0198, Macao, CN, October 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).