| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0350 |
| --- | --- |

| Title: | CE4-related: Quadtree-based Merge Estimation Region for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tianliang Fu1, Xiaozhen Zheng2, Shanshe Wang1, Siwei Ma1 Peking University1, DJI2 | Email: | futl@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | SZ DJI Technology Co., Ltd., Peking University |  |  |

_____________________________

- Abstract

In this contribution, a method for parallel merge estimation of neighboring blocks is proposed, which can handle quadtree, binary tree and ternary tree partitions. In principal, the proposed method sets the leaf node of quad-tree as the size of merge estimation region (MER), which means candidate blocks in the same leaf node of quad-tree is not included in the merge candidate list construction. Simulation results for N equal to 8, 16, 32 and 64 are presented in the contribution document.

Introduction

Parallel merge estimation was introduced in HEVC that indicates the region in which merge candidate lists can be independently derived by checking whether a candidate block is located in that merge estimation region (MER). A candidate block that locates at the same MER is not included in the merge candidate list construction. In the current VTM software, an implementation similar to that of HEVC is already existed, but it is currently not included in the specification text. This input contribution proposes an extended method of HEVC MER design, which provides parallel properties of quadtree, ternary tree and binary tree partitions as well as offers both encoder and decoder friendly implementation.

Problem Statement

In HEVC, if two coding blocks are in the same MER, one block is set unavailable for prediction to the other block during the merge list construction process. In VTM, there are some problems if applying the HEVC parallel merge estimation rule directly to VVC considering binary and ternary partitions. The problems are mainly related to the fact that a coding unit can be divided into non-square coding units which cause a coding unit might be partially outside a MER. The detailed problems have been described in [1].

Follow the concept of HEVC MER, this contribution proposes an extended HEVC MER method that use quadtree’s leaf and the parameter log2_parallel_merge_level_minus2 that has been defined in HEVC to control and determine MER size.

Proposed Implementation

Considering that all the problems can be solved if one block can be covered by the MER completely or the MER is covered by one block completely, a quadtree-based MER method is proposed in this contribution. The MER size is determined by following two rules:

- If coding block size of quadtree leaf node is larger than or equal to 1 <<  (log2_parallel_merge_level_minus2 + 2), MER size is set equal to the coding block size of quadtree leaf node;

- If coding block size of quadtree leaf node is smaller than 1 <<  (log2_parallel_merge_level_minus2 + 2), MER size is set equal to 1 <<  (log2_parallel_merge_level_minus2 + 2).

Follow the above MER size determination rules, a CTU’s MER size can be decided immediately when it’s quadtree left node is got. Meanwhile, the proposed method has the limitation on the minimum MER size which means an encoder or a decoder can choose parallel granularity as they prefer to. The proposed quadtree MER method is illustrated at figure 1.

Figure 1. Illustration of quadtree-based MER method ()

Since MER is used to remove the dependencies between neighbouring blocks at merge candidate list construction, the proposed quadtree-based MER method can be applied to both inter merge candidate and affine merge candidate list construction.

In addition, the proposed method has minor changes on current VTM implementation since information of split-tree depth, QT-depth, BT-depth, TT-depth of each CU has been recoreded in VTM3.0. The MER size of one CU is decided by its QT-depth and MIN_MER_SZIE. The major modification of VTM3.0 is described as follows.

int min_MER_size = 1 << (tempCS->pps->getLog2ParallelMergeLevelMinus2() + 2);

if (( 1 << (MAX_CU_DEPTH - partitioner.currQtDepth)) >= min_MER_size) {

cu.Curr_MER_Size = (1 << (MAX_CU_DEPTH - partitioner.currQtDepth));

}

else {

cu.Curr_MER_Size = min_MER_size;

}

Simulation Results

The simulations are conducted following the common test conditions [2] on top of reference software VTM-3.0. The MER size are set to 8x8, 16x16, 32x32 and 64x64. The experimental results are summarized at table 1~4.

Table 1: MER size = 8x8

Table 2: MER size = 16x16

Table 3: MER size = 32x32

Table 4: MER size = 64x64

Conclusion

This contribution proposes a hardware friendly parallel merge estimation method that follow and extend HEVC MER concept. It can guarantee the minimum MER size without any other constraint. Meanwhile, a decoder can derive current CTU’s MER size when quadtree’s left node is determined. Compared to VTM3.0 anchor, the coding lost is acceptable. Therefore, we recommend to integrate the proposed method into the next version of VVC working draft and VTM software.

References

[1] Semih Esenlik, Han Gao, Biao Wang, Anand Meher Kotra, Jianle Chen, “Non-CE4: Parallel Merge Estimation for VVC”, JVET-L0216, 12th Meeting: Macao, CN, 3–12 Oct. 2018

Patent rights declaration(s)

SZ DJI Technology Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).