| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0356-v1 |
| --- | --- |

| Title: | CE3-related: simplified calculation for CCLM parameters derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Xiang Ma, Vasily Rufitskiy, Haitao Yang, Jianle Chen | Email: | alexey.filippov@huawei.com maxiang6@huawei.com vasily.rufitskiy@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co. |  |  |

_____________________________

# Abstract

This document proposes a further simplification of Cross-Component Linear Model (CCLM) originally proposed in [1]. This contribution consists of two parts: reduction of bit-depth of parameter α and size reduction of LUT that is used for integer division implementation. Low-bit LUT is removed completely, high-bit LUT precision is lowered up to 8 bits as well as several methods to reduce the number of its entries are considered. This proposal reportedly shows that a non-linear sampling of LUT (Test 1) provides the overall results of 0.01% (Y), -0.08% (U), -0.04% (V) for AI configuration and 0.01% (Y), -0.10% (U), -0.09% (V) for RA configuration. Regular LUT sampling for 32 entries is reported to provide the following results  (Test 3):  0.08% (Y), 0.34% (U), 0.41% (V) for AI configuration and 0.06% (Y), 0.46% (U), 0.57% (V) for RA configuration. Regular LUT sampling for 64 entries is reported to provide the following results (Test 2):  0.03% (Y), 0.09% (U), 0.11% (V) for AI configuration and 0.03% (Y), 0.16% (U), 0.24% (V) for RA configuration.

# Introduction

In the JVET-L meeting, simplification of Cross-Component Linear Model (CCLM) was adopted into the VTM software and the VVC spec draft as proposed in [1] where both the linear model coefficients α and β are derived by a linear equation directly. In particular, the linear model parameters α and β are obtained according to the following equations:β= ∑Cn-α·∑LnN

α=CB-CALB-LA

β=LA-αCA,

where B=argmaxLn and A=argmin(Ln) are positions of maximum and minimum values in luma samples.

It was also proposed to implement division operation using multiplication by a number that is stored in a look-up table (LUT), which is specified in Table 1. This substitution is possible by using the following method:

v0v1≈v0∙LUTv1≫S,

where S is a shift parameter that specifies the precision of division.

Table 1 provides matching of the LUT indices range (given in the first line of the table) with the lists of values stored in LUT, each list corresponding to its indices range. It could be noticed, that LUTv1 values could be calculated as follows:

LUTv1=1≪Sv1 .

Table 1. LUT table to implement division operation using multiplication by a number [1]

| Indices ranges of the LUT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 32 | 64 | 96 | 128 | 160 | 192 | 224 | 256 | 288 | 320 | 352 | 384 | 416 | 448 | 480 |
| .. | .. | .. | .. | .. | .. | .. | .. | .. | .. | .. | .. | .. | .. | .. | .. |
| 31 | 63 | 95 | 127 | 159 | 191 | 223 | 255 | 287 | 319 | 351 | 383 | 415 | 447 | 479 | 511 |
| Values stored in the LUT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 65536 | 1985 | 1008 | 675 | 508 | 407 | 339 | 291 | 255 | 226 | 204 | 185 | 170 | 157 | 145 | 136 |
| 32768 | 1927 | 992 | 668 | 504 | 404 | 337 | 289 | 254 | 225 | 203 | 185 | 169 | 156 | 145 | 135 |
| 21845 | 1872 | 978 | 661 | 500 | 402 | 336 | 288 | 253 | 225 | 202 | 184 | 169 | 156 | 145 | 135 |
| 16384 | 1820 | 963 | 655 | 496 | 399 | 334 | 287 | 252 | 224 | 202 | 184 | 168 | 156 | 144 | 135 |
| 13107 | 1771 | 949 | 648 | 492 | 397 | 332 | 286 | 251 | 223 | 201 | 183 | 168 | 155 | 144 | 135 |
| 10922 | 1724 | 936 | 642 | 489 | 394 | 330 | 284 | 250 | 222 | 201 | 183 | 168 | 155 | 144 | 134 |
| 9362 | 1680 | 923 | 636 | 485 | 392 | 329 | 283 | 249 | 222 | 200 | 182 | 167 | 154 | 144 | 134 |
| 8192 | 1638 | 910 | 630 | 481 | 390 | 327 | 282 | 248 | 221 | 199 | 182 | 167 | 154 | 143 | 134 |
| 7281 | 1598 | 897 | 624 | 478 | 387 | 326 | 281 | 247 | 220 | 199 | 181 | 166 | 154 | 143 | 134 |
| 6553 | 1560 | 885 | 618 | 474 | 385 | 324 | 280 | 246 | 219 | 198 | 181 | 166 | 153 | 143 | 133 |
| 5957 | 1524 | 873 | 612 | 471 | 383 | 322 | 278 | 245 | 219 | 197 | 180 | 165 | 153 | 142 | 133 |
| 5461 | 1489 | 862 | 606 | 468 | 381 | 321 | 277 | 244 | 218 | 197 | 180 | 165 | 153 | 142 | 133 |
| 5041 | 1456 | 851 | 601 | 464 | 378 | 319 | 276 | 243 | 217 | 196 | 179 | 165 | 152 | 142 | 132 |
| 4681 | 1424 | 840 | 595 | 461 | 376 | 318 | 275 | 242 | 217 | 196 | 179 | 164 | 152 | 141 | 132 |
| 4369 | 1394 | 829 | 590 | 458 | 374 | 316 | 274 | 241 | 216 | 195 | 178 | 164 | 152 | 141 | 132 |
| 4096 | 1365 | 819 | 585 | 455 | 372 | 315 | 273 | 240 | 215 | 195 | 178 | 163 | 151 | 141 | 132 |
| 3855 | 1337 | 809 | 579 | 451 | 370 | 313 | 271 | 240 | 214 | 194 | 177 | 163 | 151 | 140 | 131 |
| 3640 | 1310 | 799 | 574 | 448 | 368 | 312 | 270 | 239 | 214 | 193 | 177 | 163 | 151 | 140 | 131 |
| 3449 | 1285 | 789 | 569 | 445 | 366 | 310 | 269 | 238 | 213 | 193 | 176 | 162 | 150 | 140 | 131 |
| 3276 | 1260 | 780 | 564 | 442 | 364 | 309 | 268 | 237 | 212 | 192 | 176 | 162 | 150 | 140 | 131 |
| 3120 | 1236 | 771 | 560 | 439 | 362 | 307 | 267 | 236 | 212 | 192 | 175 | 161 | 149 | 139 | 130 |
| 2978 | 1213 | 762 | 555 | 436 | 360 | 306 | 266 | 235 | 211 | 191 | 175 | 161 | 149 | 139 | 130 |
| 2849 | 1191 | 753 | 550 | 434 | 358 | 304 | 265 | 234 | 210 | 191 | 174 | 161 | 149 | 139 | 130 |
| 2730 | 1170 | 744 | 546 | 431 | 356 | 303 | 264 | 234 | 210 | 190 | 174 | 160 | 148 | 138 | 130 |
| 2621 | 1149 | 736 | 541 | 428 | 354 | 302 | 263 | 233 | 209 | 189 | 173 | 160 | 148 | 138 | 129 |
| 2520 | 1129 | 728 | 537 | 425 | 352 | 300 | 262 | 232 | 208 | 189 | 173 | 159 | 148 | 138 | 129 |
| 2427 | 1110 | 720 | 532 | 422 | 350 | 299 | 261 | 231 | 208 | 188 | 172 | 159 | 147 | 137 | 129 |
| 2340 | 1092 | 712 | 528 | 420 | 348 | 297 | 260 | 230 | 207 | 188 | 172 | 159 | 147 | 137 | 129 |
| 2259 | 1074 | 704 | 524 | 417 | 346 | 296 | 259 | 229 | 206 | 187 | 172 | 158 | 147 | 137 | 128 |
| 2184 | 1057 | 697 | 520 | 414 | 344 | 295 | 258 | 229 | 206 | 187 | 171 | 158 | 146 | 137 | 128 |
| 2114 | 1040 | 689 | 516 | 412 | 343 | 293 | 257 | 228 | 205 | 186 | 171 | 157 | 146 | 136 | 128 |
| 2048 | 1024 | 682 | 512 | 409 | 341 | 292 | 256 | 227 | 204 | 186 | 170 | 157 | 146 | 136 | 128 |

In this contribution, we propose a further improvements of CCLM.

# Proposal

This contribution proposes several modifications as compared with VTM3.0:

- After parameter α is derived, its bit-depth is decreased to 6 bits.  Parameter β is calculated using 6-bit α.

- LUT for lower-bit division (g_aiLMDivTableLow) is removed

- Values stored in LUT for higher-bit division (aiLMDivTableHigh) is set to be 8 bit-depth (not higher 255)

- Number of entries in LUT for higher-bit division (g_aiLMDivTableHigh) is reduced as shown in Table 2.

Table 2. Summary of 3 proposed test

| Test 1 | 48 entries, sampling of the difference LB-LA to obtain LUT index is non-linear |
| --- | --- |
| Test 2 | 64 entries, sampling of the difference LB-LA to obtain LUT index is regular |
| Test 3 | 32 entries, sampling of the difference LB-LA to obtain LUT index is regular |

After a regular subsampling (Tests 2-3) with a power-of-two subsampling ratio of N, a fetch from the LUT is defined as:

m= LUT(LB-LA)≫log2N

instead of

m= LUTLB-LA

A LUT with non-linear sampling is shown in Table 3. The value of fetched from the LUT is obtained as follows:

m= LUTidx≪ms,

idx= Log2LB-LA-1 - 3≪3+col

where ms linearly depends on the Log2LB-LA-1  value and the value of “col” could be obtained from  LB-LA using masking operation as shown in Figure 1.

Figure 1 illustrates the derivation of index within LUT with non-linear (log2-scale) sampling for a given binary representation of LB-LA value.

Figure 1: derivation of index within LUT with non-linear sampling for a given binary representation of LB-LA value.

Table 3. A LUT with non-linear sampling (48 entries), all the values are not greater than 255

| idx | col |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0..7 | 128 | 128 | 171 | 128 | 205 | 171 | 146 | 128 |
| 8..15 | 228 | 205 | 186 | 171 | 158 | 146 | 137 | 128 |
| 16..23 | 241 | 216 | 195 | 178 | 164 | 152 | 141 | 132 |
| 24..31 | 248 | 221 | 200 | 182 | 167 | 155 | 144 | 134 |
| 32..39 | 252 | 224 | 202 | 184 | 169 | 156 | 145 | 135 |
| 40..47 | 254 | 226 | 204 | 185 | 170 | 157 | 146 | 136 |

# Experiment Results

The results reportedly obtained for Tests 1-3 using the common test conditions [2] are presented in Table 4-Table 6 for AI and RA configurations.

Table 4. Experimental results for Test 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.25% | 0.10% | 100% | 100% |
| Class A2 | 0.01% | -0.06% | 0.04% | 99% | 100% |
| Class B | 0.00% | -0.08% | -0.17% | 100% | 100% |
| Class C | -0.01% | -0.05% | -0.17% | 100% | 99% |
| Class E | 0.02% | 0.04% | 0.10% | 99% | 99% |
| Overall | 0.01% | -0.08% | -0.04% | 100% | 100% |
| Class D | -0.01% | -0.03% | 0.11% | 99% | 99% |
| Class F | 0.04% | 0.04% | 0.18% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.28% | 0.01% | 99% | 99% |
| Class A2 | 0.01% | -0.11% | 0.09% | 99% | 100% |
| Class B | 0.00% | -0.02% | -0.21% | 100% | 99% |
| Class C | 0.00% | -0.06% | -0.12% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.10% | -0.09% | 99% | 99% |
| Class D | -0.02% | 0.04% | 0.04% | 99% | 99% |
| Class F | 0.01% | 0.08% | 0.10% | 100% | 99% |

Table 5. Experimental results for Test 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.28% | 0.43% | 99% | 100% |
| Class A2 | 0.01% | 0.05% | 0.06% | 100% | 99% |
| Class B | 0.02% | 0.01% | 0.07% | 100% | 99% |
| Class C | 0.00% | 0.11% | -0.08% | 100% | 99% |
| Class E | 0.02% | 0.04% | 0.16% | 100% | 100% |
| Overall | 0.03% | 0.09% | 0.11% | 100% | 99% |
| Class D | 0.01% | -0.02% | 0.07% | 99% | 99% |
| Class F | 0.01% | 0.24% | 0.16% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | 0.34% | 0.73% | 99% | 99% |
| Class A2 | -0.01% | 0.06% | 0.11% | 100% | 99% |
| Class B | 0.00% | 0.17% | 0.12% | 100% | 100% |
| Class C | 0.01% | 0.08% | 0.13% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.16% | 0.24% | 100% | 100% |
| Class D | -0.04% | -0.39% | -0.05% | 100% | 100% |
| Class F | 0.00% | 0.11% | 0.14% | 100% | 100% |

Table 6. Experimental results for Test 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.33% | 0.94% | 1.49% | 99% | 99% |
| Class A2 | 0.06% | 0.30% | 0.21% | 100% | 99% |
| Class B | 0.02% | 0.17% | 0.23% | 100% | 99% |
| Class C | 0.01% | 0.18% | 0.04% | 99% | 100% |
| Class E | 0.02% | 0.25% | 0.29% | 100% | 99% |
| Overall | 0.08% | 0.34% | 0.41% | 100% | 99% |
| Class D | 0.02% | -0.01% | 0.10% | 99% | 99% |
| Class F | 0.04% | 0.34% | 0.29% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.32% | 1.09% | 1.85% | 99% | 100% |
| Class A2 | 0.00% | 0.53% | 0.30% | 100% | 99% |
| Class B | -0.01% | 0.33% | 0.15% | 100% | 99% |
| Class C | 0.02% | 0.08% | 0.32% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.46% | 0.57% | 100% | 99% |
| Class D | -0.02% | -0.16% | 0.03% | 99% | 100% |
| Class F | 0.01% | 0.35% | 0.40% | 100% | 100% |

# Conclusion

We propose to adopt Test 1 (LUT with reduced size using non-linear sampling) into the VTM software and the VVC spec draft.

# References

- G. Laroche, J. Taquet, C. Gisquet, P. Onno (Canon), “CE3: Cross-component linear model simplification (Test 5.1)”, JVET-L0191, 12th JVET Meeting in Macao, China, October 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, 12th JVET Meeting, Macao, China, October 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).