| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0357-v4 |
| --- | --- |

| Title: | CE10-related: Reduction of the worst-case memory bandwidth and operation number of OBMC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kei Kawamura Kyohei Unno Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-70-4560-2752 yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes the reduction of the worst-case memory bandwidth and operation number of overlapped block motion compensation (OBMC) prediction which is related to Core Experiment 10.2 (CE10.2). The worst-case memory bandwidth and operation cost of OBMC are studied in CE10.2 and applying uni-prediction constraint on OBMC were recommended to reduce them in the Macao meeting. The memory bandwidth and operation number of bi-directional MC prediction was placed as the target criteria of the reduction but that of CE10.2.1 is still over than the thresholds. In this contribution, two method are proposed to reduce memory bandwidth and operation furthermore; 1) modifying the OBMC filter according to CU size, 2) modifying uni-prediction constraint on neighbor block according to CU size. The two simulations are conducted under different OBMC applicable conditions which include the above modifications without exceedance of the worst-case memory bandwidth and operation number than bi-directional MC. In condition A, the OBMC filter is modified from 8-tap to 4-tap in all CU size and the uni-prediction constraint on neighbor block is removed from all CU size. In condition B, the OBMC filter tap and the constraint are changed depending on CU size. The condition A results show the BD-rate saving for luma by -0.37% in RA and -0.35% in LD, and the condition B results  show the BD-rate saving for luma by -0.42% in RA and -0.43% in LD. It is confirmed that the proposed methods have the effect for not only reducing of worst-case memory bandwidth and operation number but also for improving the coding gains up to -0.15% in RA comparing CE10.2.1.

Introductions

Problem Statement

Overlapped block motion compensation has been studied from JEM [1] to improve coding gain. In the previous Macao meeting, it was taken up as an issue that the worst-case (WC) memory bandwidth and operation number of OBMC should not be worse than bi-directional MC prediction [2]. For the target, applying uni-prediction constraints on OBMC is tested in CE10.2.1 [3] as one of the method for reducing the WC memory bandwidth and operation but it was found that they are still over than those of bi-directional MC from the analysis described in section 1.2.

Analysis of the worst-case memory bandwidth and operation number

Memory bandwidth

We calculated three cases for WC memory bandwidth of MC and OBMC as follows:

- Uni-prediction block MC without OBMC.

- Bi-prediction block MC without OBMC.

- Uni-prediction block MC with OBMC which corresponds to CE10.2.1.

Figure 1 shows the calculation methods of MC and OBMC and Table 1 shows the applicable condition of CE10.2.1. It is noted that we set CTU size as 128x128 and calculated the memory bandwidth of chroma (Cb and Cr) with 1/4 luma CU size and 4-tap filter.

As shown in Figure 1, the WC block sizes for A, B and C are 4x4, 4x8/8x4 and 8x8. The WC memory bandwidth are calculated as the total pixels per CTU.

Figure 1 Calculation method of memory bandwidth

Table 1 OBMC applicable condition of CE10.2.1

|  | OBMC |
| --- | --- |
| Current block prediction | Uni-pred.: Applied Bi-pred.: Not applied |
| Current block size | CU < 64: Not applied. Otherwise: Applied |
| Current block MV precision | Decimal |
| Neighbor block prediction | All size: Uni-pred. (but when neighbour block is bi-directional when it is always converted into uni-prediction.) |
| Neighbor block size | Don’t care |
| Neighbor block MV precision | Decimal |
| OBMC filter tap | Luma: 8 tap Chroma: 4 tap |
| OBMC line | Luma: - CU width or height <= 4: 2 line - Otherwise: 4 line Chroma - CU width or height <= 2: 1 line - Otherwise: 2 line |

Figure 2 shows the OBMC applicable condition of CE10.2.1 with CU size. “Short” and “Long” in Figure 2 are defined by comparison between the width and height of CU. “Short” is smaller than or equal to “Long”, so the shaded area is unnecessary consideration area. It is noted that “No applied” denotes the OBMC non-applied area and the otherwise denotes OBMC applied area. It is noted that bi-directional neighbor blocks are always converted to uni-directional in CE10.2.1.

Figure 2 OBMC applicable condition of CE10.2.1 with CU size

Table 2 shows the summary of WC memory bandwidth of Case A, B and C. As show in Table 2, the memory bandwidth of CE10.2.1 is over that of bi-directional MC without OBMC.

Table 2 Summary of the worst-case memory bandwidth (pixel) per CTU

| CaseA: Uni-pred. MC w/o OBMC | Case B: Bi-pred. MC w/o OBMC | Case C: CE10.2.1 (Uni-pred. MC w/ OBMC) |
| --- | --- | --- |
| WC@CU = 4x4 Luma: 123,904 Chroma: 51,200 Total: 175,104 | WC@CU = 4x8/8x4 Luma: 168,960 Chroma: 71,680 Total: 240,640 | WC@CU = 8x8 Luma: 181,504 Chroma: 76,288 Total: 257,792 |

In Appendix, the different analysis of WC memory bandwidth is shown. It is derived from calculation method of JVET-M0178. The analysis shows the memory bandwidth of CE10.2.1 is smaller than that of bi-directional MC without OBMC.

Operation number

To calculate the operation number of MC and OBMC for Case A, B and C, we divided the MC and OBMC processing into interpolation, averaging when using bi-directional prediction and OBMC blending [4] as described in Table 3.

Table 3 Operation number of MC and OBMC processing

|  |  | Multiplication | Addition | Shift |
| --- | --- | --- | --- | --- |
| Interpolation | Luma | 8 | 8 | 1 |
|  | Chroma | 4 | 4 | 1 |
| Averaging | Luma | 0 | 2 | 1 |
|  | Chroma | 0 | 2 | 1 |
| OBMC blending | Top-left sub-CU | 1 | 4 | 3 |
|  | Otherwise | 0 | 3 | 2 |

For simplicity, we picked up only the multiplication number on behalf of the operation number because the multiplier is heavier than the adder and shifter.

For example, the multiplication numbers for luma of interpolation, averaging and OBMC blending per CU can be calculated as follows:

- MC interpolation: 8*(CUW*(CUH+8-1) + CUW*CUH)

- Averaging: 0* CUW*CUH

- OBMC interpolation (width): 8*(subCUW*(OBMCW+8-1) + subCUW*OBMCW)*CUW/subCUW

- OBMC interpolation (height): 8*(OBMCH*(subCUH+8-1) + subCUH*OBMCH)*CUH/subCUH

- OBMC blending (Top left corner): 1*subCUW* subCUH

- OBMC blending (Otherwise): 0*subCUW* subCUH

CUW, CUH, subCUW, subCUH denote width of CU, height of CU, width of subCU and height of subCU, respectively. OBMCW and OBMCH corresponds to the number of OBMC applying lines which is 4 by default but it is noted that they become from 4 to 2 for luma and from 2 to 1 for chroma if CU size are smaller than OBMC applying lines.

Table 4 shows the summary of WC memory bandwidth of case A, B and C. As show in Table 4, the multiplication number of CE10.2.1 is over that of bi-directional MC.

Table 4 The Multiplication number of times per CTU

| CaseA: Uni-pred. MC w/o OBMC | Case B: Bi-pred. MC w/o OBMC | Case C: CE10.2.1 (Uni-pred. MC w/ OBMC) |
| --- | --- | --- |
| WC@CU Height = 4 Luma: 491,520 Chroma: 114,688 Total: 606,208 | WC@CU Height = 4 Luma: 983,040 Chroma: 229,376 Total: 1,212,416 | @CU = 16x4 Luma: 976,896 Chroma: 340,992 Total: 1,317,888 |

Proposed method

To reduce the worst-case memory bandwidth and operation number, two modification from CE10.2.1 are proposed as follows:

    - To modify OBMC filter according to CU size

    - To modify uni-prediction constraint on neighbour blocks according to CU size

To evaluate the effects of these proposal, we re-designed the OBMC applicable condition as described in section 2.1 and section 2.2.

Condition A

Figure 3 shows one of the proposed OBMC applicable condition (Condition A). In this condition, two points are different from CE10.2.1 condition; 1) modifying OBMC filter tap from 8-tap to 4-tap statically, 2) removing uni-prediction on neighbor block, that is, applying OBMC to both uni-direcitonal and bi-directional neighbor block. It is noted that 4-tap OBMC filter for luma is used with the existing 4-tap MC filter for chroma in our proposal and 2-tap OBMC filter for chroma is used with bi-linear filter.

Figure 3 proposed OBMC application condition A

Condition B

Figure 4 shows the other one of proposed OBMC applicable condition (Condition B). In this condition, two points are different from CE10.2.1 condition; 1) modifying OBMC filter tap according to CU sizes, 2) modifying the uni-prediction constraint on neighbor block according to CU sizes. It is noted that OBMC is not applied at the 4x4, 4x8/8x4 CU area but OBMC is applied at those area in this condition by changing filter tap from 8-tap to 2-tap. It is noted that when 2-tap filter is used for luma OBMC, 2-tap filter also is used for chroma OBMC.

s

Figure 4 proposed OBMC application condition B

Analysis of the worst-case memory bandwidth and operation number

Figure 5 shows the calculation method of memory bandwidth for MC and OBMC in Condition A/B. Table 5 shows the summary of the WC memory bandwidth and operation number in Condition A/B. In both condition, WC memory bandwidth and operation number are smaller than those of MC for bi-prediction as shown in Table 2 and Table 4.

Figure 5 Calculation method of memory bandwidth for MC and OBMC in Condition A/B

Table 5 Summary of the worst-case memory bandwidth and operation number in condition A/B

|  | Condition A | Condition B |
| --- | --- | --- |
| WC memory bandwidth w/ OBMC | WC@CU = 4x16/16x4 (4-tap, neighbor block is Bi-pred.) Luma: 161536 Chroma: 61952 Total: 223,488 | WC@CU = 4x4 (2-tap, neighbor block is Uni-pred.) Luma: 154,624 Chroma: 75,776 Total: 230,400 |
| WC multiplication number w/ OBMC | WC@CU = 16x4 (4-tap, neighbor block is Bi-pred.) Luma: 813,056 Chroma: 300,032 Total: 1,113,088 | WC@CU = 16x4 (4-tap, neighbor block is Bi-pred.) Luma: 813,056 Chroma: 300,032 Total: 1,113,088 |

In Appendix, the different analysis of WC memory bandwidth is shown. It is derived from calculation method of JVET-M0178. The analysis shows the memory bandwidths of Condition A and B are smaller than that of bi-directional MC without OBMC.

Experimental Results

Coding conditions

The proposed OBMC applicable condition have been implemented on top of CE10.2.1 based on VTM-3.0 [5]. The common test conditions are used for evaluation [6].

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Simulation results

Condition A

Table 6 and Table 7 show the simulation results of condition A in RA and LDB respectively.

Table 6 Simulation results of condition A in RA compared to VTM-3.0

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.41% | -0.60% | 102.6% | 101.4% |
| Class A2 | -0.40% | -0.70% | -0.66% | 103.2% | 102.5% |
| Class B | -0.30% | -0.61% | -0.74% | 103.1% | 102.8% |
| Class C | -0.55% | -0.79% | -0.87% | 103.6% | 104.4% |
| Class E |  |  |  |  |  |
| Overall | -0.37% | -0.64% | -0.73% | 103.2% | 102.9% |
| Class D | -0.48% | -0.74% | -0.73% | 103.7% | 104.6% |
| Class F | -0.10% | -0.29% | -0.27% | 103% | 103% |

Table 7 Simulation results of condition A in LDB compared to VTM-3.0

|  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.30% | -0.78% | -0.30% | 104.4% | 103.5% |
| Class C | -0.49% | -0.52% | -0.82% | 104.8% | 105.7% |
| Class E | -0.26% | -0.42% | -0.50% | 103.8% | 103.6% |
| Overall | -0.35% | -0.61% | -0.52% | 104.4% | 104.3% |
| Class D | -0.57% | -0.78% | -0.46% | 104.9% | 106.5% |
| Class F | 0.01% | -0.32% | 0.35% | 104% | 102% |

Condition B

Table 8 and Table 9show the simulation results of condition B in RA and LDB respectively.

Table 8 Simulation results of condition B in RA compared to VTM-3.0

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.38% | -0.65% | 102.6% | 101.9% |
| Class A2 | -0.44% | -0.88% | -0.82% | 103.3% | 103.1% |
| Class B | -0.33% | -0.75% | -0.79% | 103.4% | 103.4% |
| Class C | -0.66% | -1.08% | -1.25% | 104.0% | 105.5% |
| Class E |  |  |  |  |  |
| Overall | -0.42% | -0.79% | -0.89% | 103.4% | 103.6% |
| Class D | -0.61% | -1.37% | -1.12% | 104.1% | 106.9% |
| Class F | -0.06% | -0.28% | -0.25% | 104% | 104% |

Table 9 Simulation results of condition B in LDB compared to VTM-3.0

|  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.33% | -0.90% | -0.52% | 104.7% | 103.8% |
| Class C | -0.65% | -0.71% | -1.04% | 105.4% | 107.4% |
| Class E | -0.32% | -0.42% | -0.95% | 103.8% | 103.5% |
| Overall | -0.43% | -0.72% | -0.80% | 104.7% | 104.9% |
| Class D | -0.71% | -0.68% | -0.95% | 106.1% | 109.3% |
| Class F | 0.05% | -0.30% | -0.25% | 104% | 103% |

Conclusion

In this proposal, two modifications of the OBMC applicable condition were proposed to reduce memory bandwidth and operation number. To evaluate the proposed method, two simulation were conducted under different OBMC applicable conditions. Simulation results under condition A show the BD-rate saving for luma by -0.37% in RA and -0.35% in LD and, on the other hand, the simulation results under condition B show the BD-rate saving for luma by -0.42% in RA and -0.43% in LD without any exceedance of the worst-case memory bandwidth and operation number than bi-directional MC. Furthermore, the coding gains of the proposed method are improved up to -0.15% in RA compared to that of CE10.2.1. So, it is recommended that this proposed OBMC is adopted into WD.

- Acknowledgment

We would like to thank MediaTek for crosschecking our proposal.

References

- J. Chen, E. Alshina, G. J. Sullivan, J. R. Ohm, J. Boyce, “Algorithm description of Joint Exploration Test Model 7 (JEM7)”, JVET-G1001, Torino, IT, July 2018.

- C.-W. Hsu, M. Winken, “Description of Core Experiment 10 (CE10): Combined and multi-hypothesis prediction”, JVET-L1030, Macao, CN, October 2018.

- Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, C.-C. Chen, Y.-C. Lin, Y.-W. Huang, S.-M. Lei, X. Xiu and Y. He, “CE10.2.1: Uni-prediction-based CU-boundary-only OBMC”, JVET-M0178, Marrakech, MA, January. 2019

- C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei “CE-related: OBMC complexity reduction and parallel blending”, JVET-K0258, Slovenia, SI, July 2018.

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3)”, JVET-L1002, Macao, CN, October 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, Macao, CN, October 2018.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix

Table 10 and Table 11 show the simulation results of CE10.2.1 in RA and LDB respectively.

Table 10 Simulation results of CE10.2.1 in RA compared to VTM-3.0

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.42% | -0.44% | 103.6% | 102.0% |
| Class A2 | -0.31% | -0.61% | -0.59% | 104.2% | 103.2% |
| Class B | -0.20% | -0.53% | -0.65% | 104.0% | 103.8% |
| Class C | -0.46% | -0.73% | -0.74% | 104.3% | 104.7% |
| Class E |  |  |  |  |  |
| Overall | -0.27% | -0.58% | -0.62% | 104.0% | 103.5% |
| Class D | -0.38% | -0.83% | -0.75% | 104.3% | 105.5% |
| Class F | -0.04% | -0.31% | -0.18% | -0.06% | 103.4% |

Table 11 Simulation results of CE10.2.1 in LDB compared to VTM-3.0

|  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.26% | -0.76% | -0.49% | 105.4% | 104.5% |
| Class C | -0.50% | -0.49% | -0.65% | 105.4% | 106.4% |
| Class E | -0.36% | -0.10% | -0.35% | 104.8% | 104.4% |
| Overall | -0.36% | -0.50% | -0.51% | 105.2% | 105.1% |
| Class D | -0.52% | -0.80% | -0.31% | 105.6% | 107.3% |
| Class F | 0.03% | -0.36% | -0.34% | -0.02% | 104.5% |

Figure 6 shows WC block pattern. Table 12 show the analyses of the WC memory bandwidth derived from the calculation method in JVET-M0178.

Figure 6 WC block pattern

Table 12 Analysis of the WC memory bandwidth derived from the calculation method in JVET-M0178.

| WC pattern | Non-OBMC bi-pred. | CE10.2.1 | Condition A | Condition B |
| --- | --- | --- | --- | --- |
| #0 | 1.00(X) | ‐ | ‐ | ‐ |
| #1 | ‐ | 0.98 (A) | 0.94 (C) | 0.94 (C) |
| #2 | ‐ | 0.90 (B) | 0.69 (D) | 0.69 (D) |

X): 15x11x2x8 = 2640

2640/2640 = 1.0

A): 15x11x2x4 + 15x15x2 + 11x11x4 + 15x11x2 = 2584

2584/2640 = 0.98

(B): 15x15x2 + 11x11x8 + 11x11x8 =2386

2386/2640 = 0.90

(C): 15x11x2x4 + 15x15x2 + 7x7x4 + 11x7x2 = 2470

2470/2640 = 0.94

(C): 15x15x2 + 11x11x8 + 7x7x8 = 1810

1810/2640 = 0.69