| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0359-v2 |
| --- | --- |

| Title: | Non-CE4: Modification of merge data syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geonjung Ko, Dongcheol Kim, Jason Jung, Ju-Hyung (John) Son, Jin Sam Kwak | Tel: Email: | +82-31-712-0523 greg.ko@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

# Abstract

This contribution presents a modification of merge data syntax. The current VTM-3.0 has many merge schemes such as subblock merge modes, merge mode with motion vector difference (MMVD), the combined inter-picture merge and intra-picture prediction, and triangular shape-based motion compensation. When merge_flag is equal to 1, mmvd_flag is always parsed since it is the first syntax in merge data syntax. However, the current MMVD uses only the first or second merge candidate as a base candidate. Therefore, in the proposed syntax structure, mmvd_flag is parsed only when the merge candidate index is equal to 0 or 1. It reportedly shows -0.06%/-0.14%/-0.15% (Y/U/V) average BD-rate change with RA configuration and -0.17%/-0.22%/-0.30% (Y/U/V) average BD-rate change with LDB configuration. The proposal with removal of redundant triangle flag reports -0.08%/-0.13%/-0.19% (Y/U/V) average BD-rate change with RA configuration and -0.19%/-0.23%/-0.16% (Y/U/V) average BD-rate change with LDB configuration. Also the proposed method could remove the mmvd_merge_flag and its context model.

# Introduction

VVC has adopted several merge (or skip) schemes, such as subblock merge modes, merge mode with motion vector difference (MMVD), the combined inter-picture merge and intra-picture prediction, and triangular shape based motion compensation [1]. If merge_flag is equal to 1, mmvd_flag is always parsed since it is the first syntax in merge data syntax. Therefore, when the coding unit uses the newly adopted merge mode listed above, or when it uses the conventional merge mode with the candidate index equal to or greater than 2, mmvd_flag is parsed.

# Proposed method

It is proposed that mmvd_flag is parsed when merge_idx is equal to 0 or 1. This is because the current MMVD uses the first or second merge candidate as a base candidate. Table 1 shows the modified merge data syntax. By this modification, mmvd_flag is not parsed when a merge mode other than MMVD is used. The changes are highlighted in yellow and strikethrough.

Table 1. The modified merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mh_intra_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| mh_intra_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| mh_intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| mh_intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( sps_triangle_enabled_flag && slice_type = = B && cbWidth * cbHeight >= 16 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) |  |
| merge_triangle_idx[ x0 ][ y0 ] | ae(v) |
| else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( merge_idx[ x0 ][ y0 ] < 2 && !mh_intra_flag[ x0 ][ y0 ] && !merge_triangle_flag[ x0 ][ y0 ]) { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

Instead of mmvd_merge_flag, merge_idx specifies which merge candidate is used as an MMVD base candidate. Thus, there is no need for separate mmvd_merge_flag and its context model. The context initial value of mmvd_merge_flag is used for that of merge_idx.

# Experimental results

## Proposed method

The proposed method was implemented on top of VTM-3.0 and tested under the common test conditions [2]. Table 2 and Table 3 summarize experimental results for random access and low delay B configurations, respectively.

Table 2. Experimental results of the proposed method with RA configuration

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.25% | -0.18% | 100% | 101% |
| Class A2 | -0.04% | -0.13% | -0.07% | 100% | 101% |
| Class B | -0.06% | -0.08% | -0.13% | 100% | 100% |
| Class C | -0.07% | -0.15% | -0.21% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.14% | -0.15% | 100% | 101% |
| Class D | -0.08% | -0.19% | -0.23% | 100% | 100% |
| Class F | -0.04% | -0.11% | -0.12% | 100% | 99% |

Table 3. Experimental results of the proposed method with LDB configuration

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | -0.28% | -0.20% | 100% | 100% |
| Class C | -0.14% | -0.16% | -0.28% | 100% | 102% |
| Class E | -0.28% | -0.21% | -0.50% | 100% | 103% |
| Overall | -0.17% | -0.22% | -0.30% | 100% | 101% |
| Class D | -0.13% | -0.34% | 0.16% | 100% | 103% |
| Class F | -0.03% | -0.50% | 0.06% | 101% | 105% |

## Proposed method + Redundant triangle flag fix

In the current VTM, when mh_intra_flag is equal to 1, merge_triangle_flag is parsed, even though it is always equal to 0. Therefore, merge_triangle_flag can be parsed only when mh_intra_flag is equal to 0 as shown in Table 4.

Table 4. Removal of redundant triangle flag

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| // … |  |
| if( sps_triangle_enabled_flag && slice_type = = B && cbWidth * cbHeight >= 16 && !mh_intra_flag[ x0 ][ y0 ] ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| // … |  |
| } |  |

Table 5 and Table 6 summarize experimental results of the proposed method with redundant triangle flag fix for random access and low delay B configurations, respectively.

Table 5. Experiemental results of the proposed method with redundant triangle flag fix for RA configuration

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.18% | -0.22% | 100% | #DIV/0! |
| Class A2 | -0.09% | -0.18% | -0.13% | 100% | #DIV/0! |
| Class B | -0.08% | -0.09% | -0.17% | 100% | #DIV/0! |
| Class C | -0.06% | -0.11% | -0.21% | 100% | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.13% | -0.19% | 100% | #DIV/0! |
| Class D | -0.12% | -0.27% | -0.22% | 100% | #DIV/0! |
| Class F | -0.05% | -0.12% | -0.11% | 99% | #DIV/0! |

Table 6. Experiemental results of the proposed method with redundant triangle flag fix for LDB configuration

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.14% | -0.28% | -0.03% | 100% | #DIV/0! |
| Class C | -0.15% | -0.11% | -0.10% | 100% | #DIV/0! |
| Class E | -0.32% | -0.32% | -0.45% | 100% | #DIV/0! |
| Overall | -0.19% | -0.23% | -0.16% | 100% | #DIV/0! |
| Class D | -0.17% | -0.13% | 0.27% | 100% | #DIV/0! |
| Class F | -0.16% | -0.43% | -0.51% | 100% | #DIV/0! |

# Conclusion

This contribution proposes to modify merge data syntax to reduce signaling overhead when MMVD is not used. The proposal shows -0.06%/-0.14%/-0.15% (Y/U/V) average BD-rate change with RA configuration and -0.17%/-0.22%/-0.30% (Y/U/V) average BD-rate change with LDB configuration. The proposal with removal of redundant triangle flag shows -0.08%/-0.13%/-0.19% (Y/U/V) average BD-rate change with RA configuration and -0.19%/-0.23%/-0.16% (Y/U/V) average BD-rate change with LDB configuration. Furthermore, the proposal reduces the number of syntax elements and context models. It is suggested to adopt the proposed change in the next version of VVC draft text and the reference software.

# References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” JVET-L1001, 12th meeting, Macao, CN, 3–12 Oct.2018.

[2] F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th meeting, Macao, CN, 3–12 Oct.2018.

# Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).