| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0361 |
| --- | --- |

| Title: | Non-CE6: Mismatch between text specification and reference software on the signalling root CBF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jason Jung, Dongcheol Kim, Geonjung Ko, Ju-Hyung (John) Son, Jin Sam Kwak | Tel: Email: | +82-31-712-0523 jason.jung@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

# Abstract

In this contribution, it is reported that there is a mismatch between VVC Draft 3 and VTM-3.0 on signalling syntax element cu_cbf. In VVC Draft 3, cu_cbf is signalled for inter-coded blocks when cu_skip_flag is equal to 0. However, in VTM-3.0, cu_cbf is signalled for inter-coded blocks when merge_flag is equal to 0. It is reportedly shown that SW change reflecting the method described in the specification text increases overall BD-rate by 0.25%, 0.44% with 101%, 100% overall encoding time and 10x%, 10x% overall decoding time under Random Access (RA) and Low Delay B (LDB) configurations, respectively. Therefore, it is recommended to change the specification text to the method used in the reference software.

# Proposed Method

In VVC Draft 3 [1], syntax element cu_cbf can be signalled within coding unit syntax structure as described in Table 1.

Table 1 – 7.3.4.6 Coding unit syntax in VVC Draft 3

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { |  |
| --- | --- |
| … |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |
| } |  |

In the coding unit syntax structure, syntax element cu_cbf is signalled if CuPredMode[ x0 ][ y0 ] is not equal to MODE_INTRA and cu_skip_flag[ x0 ][ y0 ] is equal to 0, which means cu_cbf can be signalled for inter-coded blocks with non-skip mode.

On the other hand, in the VTM-3.0 [2], cu_cbf is signalled and parsed if CuPredMode[ x0 ][ y0 ] is not equal to MODE_INTRA and merge_flag[ x0 ][ y0 ] is equal to 0, which means cu_cbf can be signalled inter-coded blocks with non-merge mode.

cu_cbf for inter-coded blocks can be inferred according to cu_skip_flag[ x0 ][ y0 ] and merge_flag[ x0 ][ y0 ], which are signalled prior to cu_cbf. In the current specification, when merge_flag[ x0 ][ y0 ] is equal to 1 and cu_skip_flag[ x0 ][ y0 ] is equal to 0, cu_cbf is redundantly signalled even though it can be inferred as 1.

In order to remove redundant signalling related to cu_cbf, it is proposed to change the specification text to be aligned with VTM-3.0 as described in Table 2.

Table 2 – The proposed specification text change

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { |  |
| --- | --- |
| … |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && cu_skip_flag[ x0 ][ y0 ] = = 0 merge_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |
| } |  |

# Experimental Results

Signalling scheme for cu_cbf described in the current VVC Draft 3 is implemented on top of VTM-3.0 and tested under the common test condition (CTC) [3]. Detailed experimental results are shown in Table 3.

Table 3 – Comparison of the proposed method over VTM-3.0

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.21% | 0.14% | 0.33% | 101% | #DIV/0! |
| Class A2 | 0.25% | 0.32% | 0.38% | 100% | #DIV/0! |
| Class B | 0.23% | 0.26% | 0.19% | 101% | #DIV/0! |
| Class C | 0.30% | 0.20% | 0.23% | 100% | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | 0.25% | 0.23% | 0.27% | 101% | #DIV/0! |
| Class D | 0.30% | -0.03% | 0.10% | 100% | #DIV/0! |
| Class F | 0.18% | 0.13% | 0.14% | 100% | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.34% | 0.11% | 0.35% | 101% | #DIV/0! |
| Class C | 0.38% | 0.33% | 0.12% | 100% | #DIV/0! |
| Class E | 0.69% | 0.38% | 0.12% | 100% | #DIV/0! |
| Overall | 0.44% | 0.25% | 0.21% | 100% | #DIV/0! |
| Class D | 0.53% | -0.03% | 0.72% | 100% | #DIV/0! |
| Class F | 0.38% | 0.73% | -0.25% | 101% | #DIV/0! |

# Conclusion

In this contribution, a mismatch between VVC Draft 3 and the reference software regarding cu_cbf is reported. In the specification text, cu_cbf is signalled for inter-coded blocks with non-skip mode, while it is signalled for inter-coded blocks with non-merge mode in the VTM-3.0. It is reportedly shown that changing the reference software to be aligned with the method described in the specification text shows 0.25%, 0.44% overall BD-rate losses with 101%, 100% encoding time and 10x%, 10x% decoding time under RA and LDB configurations, respectively. Therefore, it is recommended to change the specification text to the method used in the reference software.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” JVET-L1001, 12th meeting, Macao, CN, 3–12 Oct. 2018.

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th meeting, Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).