| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0362 |
| --- | --- |

| Title: | CE13: In-loop filters disabled across face discontinuities (Test 1.1.a and Test1.1.b) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sheng-Yen Lin, Lin Liu, Jian-Liang Lin, Yung-Chang Chang, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Philippe Hanhart, Yuwen He 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 {Sheng-Yen.Lin, Lin.Liu, jl.lin, yc.zhang,cc.ju}@mediatek.com Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com |
| Source: | MediaTek Inc. InterDigital Communications, Inc. |  |  |

_____________________________

- Abstract

This contribution reports the experiment results of Test 1.1.a/b in CE13: in-loop filter disabled across face discontinuities on HEC/PHEC. In terms of end-to-end WS-PSNR-Y/U/V, the experimental results reportedly demonstrate that Test 1.1.a achieves an average -0.57%/-0.58%/-0.37% and -0.27%/-0.93%/-0.80% BD-rate saving compared to PHEC and HEC, respectively, and Test 1.1.b achieves an average -0.01%/-0.29%/-0.13% and 0.29%/-0.63%/-0.56% BD-rate saving compared to PHEC and HEC, respectively. Moreover, in terms of codec PSNR, Test 1.1.b achieves an average -0.32%/-1.41%/-1.23% BD-rate saving compared to PHEC. In addition, the subjective quality of Test 1.1.a and Test 1.1.b is improved by alleviating the seam artifacts compared to HEC and PHEC, respectively.

Introduction

In the last meeting, Core Experiment 13 (CE13) [1] was established to study removing subjective artifacts at face boundaries for the 360° video category. This contribution reports the results of in-loop filters disabled across face discontinuities implemented on HEC and PHEC proposed in JVET-L-0232 [2], JVET-L0347 [3], JVET-L0348 [4] and JVET-L0350 [5].

The in-loop filters in a conventional video coding system is used to remove undesired artifacts and enhance encoding quality. For a 360° video, the layout for a specific projection format usually has one or multiple discontinuous edges. Applying the in-loop filters on these discontinuous edges directly may result in poor visual quality and coding efficiency because the accessed pixels (to be referenced and/or filtered) cross discontinuous edges are jointly processed (filtered). To address this problem, the conventional in-loop filters are modified such that the discontinuous edge can be properly processed.

Proposed Method

In this contribution, the in-loop filters including the de-blocking filter, sample adaptive offset (SAO) filter and the adaptive loop filter (ALF), are performed necessarily changes to avoid inappropriately filtering at the discontinuous edge.

De-blocking Filter

The proposed de-blocking filtering method is a straightforward approach that disables the de-blocking filtering process when the filtered pixels cross the discontinuous edge as illustrated in Fig. 1.

Fig. 1. The de-blocking filtering is disabled when the filtered pixels cross the discontinuous edge

SAO Filter

As proposed in JVET-L0348 [4], when the referenced pixels of current pixel in the SAO filtering process cross the discontinuous edge, the proposed modification disables the SAO filtering process of the current pixel to avoid inadequate filtering process.

Fig. 2. The SAO filtering process is disabled when the referenced pixels cross the discontinuous edge

Adaptive Loop Filter

For ALF, when filtering a CU, reconstructed sample positions used in filtering are clipped to be within the same side of the face discontinuity to which the current CU belongs.

Experimental Results

The simulations of the Test 1.1.a and Test 1.1.b are conducted on HEC and PHEC, respectively. The software is implemented on 360Lib-8.0 [6] and VTM-3.0 [7] followed by the JVET common test conditions and evaluation procedures for 360° video [8]. The resolution of each face is set to 1280x1280, which yields the frame resolution is 3840x2560. As compared to PHEC and HEC, the BD-rate performance, in terms of end-to-end WS-PSNR, provided by Test 1.1.a and Test 1.1.b are listed in Table I and Table II, respectively. In addition, the BD-rate performance of codec PSNR for Test 1.1.b is listed in Table III.

Table I. BD-rate performance for Test 1.1.a compared to PHEC and HEC under VTM-3.0 in terms of end-to-end WS-PSNR

|  | Anchor : PHEC |  |  | Anchor : HEC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -0.22% | -0.33% | 0.54% | -0.22% | -1.21% | -0.69% |
| ChairLift | -0.47% | -0.36% | -0.39% | -0.36% | -0.54% | -0.41% |
| KiteFlite | -0.65% | -0.32% | -0.35% | -0.18% | -0.37% | -0.37% |
| Harbor | -0.57% | -0.17% | -0.29% | -0.70% | -1.30% | -1.69% |
| Trolley | -0.84% | -0.44% | -0.30% | -0.16% | -0.78% | -0.56% |
| GasLamp | -0.73% | -0.52% | -0.60% | -0.26% | -1.01% | -0.91% |
| Balboa | -0.37% | -0.57% | 0.15% | -0.31% | -0.60% | -0.33% |
| Broadway | -0.32% | -0.56% | -0.21% | -0.27% | -0.60% | -0.40% |
| Landing2 | -0.70% | -1.98% | -1.66% | -0.23% | -2.78% | -2.34% |
| BranCastle2 | -0.86% | -0.57% | -0.55% | -0.06% | -0.11% | -0.28% |
| Class S1 | -0.58% | -0.36% | -0.23% | -0.31% | -0.87% | -0.77% |
| Class S2 | -0.56% | -0.92% | -0.57% | -0.22% | -1.02% | -0.83% |
| Overall | -0.57% | -0.58% | -0.37% | -0.27% | -0.93% | -0.80% |

Table II. BD-rate performance for Test 1.1.b compared to PHEC and HEC under VTM-3.0 in terms of end-to-end WS-PSNR

|  | Anchor : PHEC |  |  | Anchor : HEC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | 0.13% | -0.49% | 0.35% | 0.13% | -1.37% | -0.88% |
| ChairLift | -0.07% | -0.12% | -0.10% | 0.05% | -0.29% | -0.12% |
| KiteFlite | -0.03% | 0.02% | -0.13% | 0.43% | -0.03% | -0.15% |
| Harbor | -0.10% | -0.32% | -0.09% | -0.23% | -1.45% | -1.49% |
| Trolley | -0.04% | -0.12% | -0.25% | 0.65% | -0.46% | -0.50% |
| GasLamp | -0.06% | -0.16% | -0.19% | 0.41% | -0.64% | -0.49% |
| Balboa | 0.04% | -0.42% | 0.06% | 0.10% | -0.44% | -0.41% |
| Broadway | 0.06% | -0.06% | -0.13% | 0.11% | -0.09% | -0.31% |
| Landing2 | -0.04% | -1.36% | -0.99% | 0.43% | -2.17% | -1.67% |
| BranCastle2 | 0.02% | 0.16% | 0.11% | 0.82% | 0.63% | 0.39% |
| Class S1 | -0.03% | -0.20% | -0.07% | 0.24% | -0.71% | -0.61% |
| Class S2 | 0.02% | -0.42% | -0.24% | 0.36% | -0.52% | -0.50% |
| Overall | -0.01% | -0.29% | -0.13% | 0.29% | -0.63% | -0.56% |

Table III. BD-rate performance for Test 1.1.b compared to PHEC under VTM-3.0 in terms of codec PSNR

|  | Anchor : PHEC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| SkateboardInLot | -0.14% | -2.02% | -1.49% |
| ChairLift | -0.41% | -0.80% | -0.48% |
| KiteFlite | -0.25% | -0.49% | -0.59% |
| Harbor | -0.78% | -1.92% | -2.37% |
| Trolley | -0.21% | -1.11% | -0.81% |
| GasLamp | -0.30% | -1.49% | -1.45% |
| Balboa | -0.38% | -1.20% | -0.63% |
| Broadway | -0.35% | -0.91% | -0.92% |
| Landing2 | -0.29% | -3.82% | -3.21% |
| BranCastle2 | -0.08% | -0.31% | -0.34% |
| Class S1 | -0.35% | -1.30% | -1.20% |
| Class S2 | -0.27% | -1.56% | -1.27% |
| Overall | -0.32% | -1.41% | -1.23% |

Subjective Quality Testing

In Fig. 3 to Fig. 10, the cropped viewport images of HEC, PHEC, Test 1.1.a and Test 1.1.b are demonstrated (under VTM-3.0 with QP = 37) in order to evaluate the seam artifacts appeared at  discontinuous edges.

Fig. 3. The subjective quality of Broadway (frame 231) in HEC. The image is cropped from the viewport centered in degree (45, -35).

Fig. 4. The subjective quality of Broadway (frame 231) in Test 1.1.a. The image is cropped from the viewport centered in degree (45, -35).

Fig. 5. The subjective quality of Broadway (frame 231) in PHEC. The image is cropped from the viewport centered in degree (45, -35).

Fig. 6. The subjective quality of Broadway (frame 231) in Test 1.1.b. The image is cropped from the viewport centered in degree (45, -35).

Fig. 7. The subjective quality of Landing2 (frame 59) in HEC. The image is cropped from the viewport centered in degree (45, -35).

Fig. 8. The subjective quality of Landing2 (frame 59) in Test 1.1.a. The image is cropped from the viewport centered in degree (45, -35).

Fig. 9. The subjective quality of Landing2 (frame 59) in PHEC. The image is cropped from the viewport centered in degree (45, -35).

Fig. 10. The subjective quality of Landing2 (frame 59) in Test 1.1.b. The image is cropped from the viewport centered in degree (45, -35).

Conclusion

In this contribution, the performance of the in-loop filters disabled across face discontinuities on HEC and PHEC is investigated. The conventional in-loop filters are modified to avoid improper filtering at the discontinuous edge. The experimental results demonstrate that Test 1.1.a and Test 1.1.b reduce the visible seam artifacts with slightly improved compression efficiency compared to HEC and PHEC, respectively.

References

- P. Hanhart, J.-L. Lin, and C. Pujara, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1033, Oct. 2018, Macau, CN.

- P. Hanhart, Y. He and Y. Ye, “CE13: HEC with deblocking and ALF disabled across face discontinuities (Test 4.1.c),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L0232, Oct. 2018, Macao, CN.

- S.-Y. Lin, J-L Lin, H-C Lin, S-K Chang, Y-C Chang, C-C Ju, “CE13: De-blocking filter disabled across face discontinuities (Test 4.1.a),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L0347, Oct. 2018, Macao, CN.

- S.-Y. Lin, L. Liu, J.-L. Lin, H.-C. Lin, S.-K. Chang, Y-C Chang and C.-C. Ju, “CE13: In-loop filters disabled across face discontinuities (Test 4.1.b and Test 4.1.d),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L0348, Oct. 2018, Macau, CN.

- S.-Y. Lin, L. Liu, J.-L. Lin, S.-K. Chang, Y-C Chang and C.-C. Ju, “CE13: Padding and in-loop filters disabled across face discontinuities (Test 7.1),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L0350, Oct. 2018, Macau, CN.

- 360Lib, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

- Versatile Video Coding Test Model, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- P. Hanhart, J. Boyce, K. Choi and J.-L. Lin, “JVET common test conditions and evaluation procedures for 360° video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1012, Oct. 2018, Macau, CN.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Annex A: Specification

The modifications of the in-loop filters disabled across face discontinuities are highlighted in green.

- 7.3.2.1 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| vr360_cubemap_3x2_layout_flag | u(1) |
| … |  |

- 7.4.3.1 Sequence parameter set RBSP semantics

…

vr360_cubemap_3x2_layout_flag equal to 1 specifies that the cubemap projection format with 3x2 layout is applied for coding of VR360 video. vr360_cubemap_3x2_layout_flag equal to 0 specifies that the cubemap projection format with 3x2 layout is not applied for coding of VR360 video.

…

  - In-loop Filter Process

…

    - Deblocking filter process

General

Inputs to this process are the reconstructed picture prior to deblocking, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr.

Outputs of this process are the modified reconstructed picture after deblocking, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr.

The vertical edges in a picture are filtered first. Then the horizontal edges in a picture are filtered with samples modified by the vertical edge filtering process as input. The vertical and horizontal edges in the CTBs of each CTU are processed separately on a coding unit basis. The vertical edges of the coding blocks in a coding unit are filtered starting with the edge on the left-hand side of the coding blocks proceeding through the edges towards the right-hand side of the coding blocks in their geometrical order. The horizontal edges of the coding blocks in a coding unit are filtered starting with the edge on the top of the coding blocks proceeding through the edges towards the bottom of the coding blocks in their geometrical order.

NOTE – Although the filtering process is specified on a picture basis in this Specification, the filtering process can be implemented on a coding unit basis with an equivalent result, provided the decoder properly accounts for the processing dependency order so as to produce the same output values.

The deblocking filter process is applied to all coding subblock edges and transform block edges of a picture, except the following types of edges:

–	vr360_cubemap_3x2_layout_flag is equal to 1 and edges that coincide with the discontinuous edges of the picture,

–	Edges that are at the boundary of the picture,

–	Edges that coincide with tile boundaries when loop_filter_across_tiles_enabled_flag is equal to 0,

–	Edges that coincide with upper or left boundaries of slices with slice_loop_filter_across_slices_enabled_flag equal to 0 or slice_deblocking_filter_disabled_flag equal to 1,

–	Edges within slices with slice_deblocking_filter_disabled_flag equal to 1,

–	Edges that do not correspond to 8x8 sample grid boundaries of the considered component,

–	Edges within chroma components for which both sides of the edge use inter prediction,

–	Edges of chroma transform blocks that are not edges of the associated transform unit.

[Ed. (BB): Adapt syntax once tiles are integrated.]

…

Deblocking filter process for one direction

Inputs to this process are:

- the variable treeType specifying whether a single tree (SINGLE_TREE) or a dual tree is used to partition the CTUs and, when a dual tree is used, whether the luma (DUAL_TREE_LUMA) or chroma components (DUAL_TREE_CHROMA) are currently processed,

- when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the reconstructed picture prior to deblocking, i.e., the array recPictureL,

- when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered.

Outputs of this process are the modified reconstructed picture after deblocking, i.e:

- when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the array recPictureL,

- when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr.

For each coding unit with coding block width log2CbW, coding block height log2CbH and location of top-left sample of the coding block ( xCb, yCb ), when edgeType is equal to EDGE_VER and xCb % 8 is equal 0 or when edgeType is equal to EDGE_HOR and yCb % 8 is equal to 0, the edges are filtered by the following ordered steps:

- The coding block width nCbW is set equal to 1  <<  log2CbW and the coding block height nCbH is set equal to 1  <<  log2CbH

- The variable filterEdgeFlag is derived as follows:

- If edgeType is equal to EDGE_VER and one or more of the following conditions are true, filterEdgeFlag is set equal to 0:

- The left boundary of the current coding block is the left boundary of the picture.

- The left boundary of the current coding block is the left boundary of the tile and loop_filter_across_tiles_enabled_flag is equal to 0.

- The left boundary of the current coding block is the left boundary of the slice and slice_loop_filter_across_slices_enabled_flag is equal to 0.

- Otherwise if edgeType is equal to EDGE_HOR and one or more of the following conditions are true, the variable filterEdgeFlag is set equal to 0:

- The top boundary of the current luma coding block is the top boundary of the picture.

- The top boundary of the current coding block is the top boundary of the tile and loop_filter_across_tiles_enabled_flag is equal to 0.

- The top boundary of the current coding block is the top boundary of the slice and slice_loop_filter_across_slices_enabled_flag is equal to 0.

- Otherwise, filterEdgeFlag is set equal to 1.

[Ed. (BB): Adapt syntax once tiles are integrated.]

- All elements of the two-dimensional (nCbW)x(nCbH) array edgeFlags are initialized to be equal to zero.

- The derivation process of transform block boundary specified in clause 8.5.2.3 is invoked with the location ( xCb, yCb ), the location ( xB0, yB0 ) set equal to ( 0, 0 ), the block width nTbW set equal to nCbW, the block height nTbH set equal to nCbH, the variable treeType, the variable filterEdgeFlag, the array edgeFlags, and the variable edgeType as inputs, and the modified array edgeFlags as output.

- The derivation process of coding subblock boundary specified in clause 8.5.2.4 is invoked with the location ( xCb, yCb ), the coding block width nCbW, the coding block height nCbH, the array edgeFlags, and the variable edgeType as inputs, and the modified array edgeFlags as output.

- The picture sample array recPicture is derived as follows:

- If treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, recPicture is set equal to the reconstructed luma picture sample array prior to deblocking recPictureL.

- Otherwise (treeType is equal to DUAL_TREE_CHROMA), recPicture is set equal to the reconstructed chroma picture sample array prior to deblocking recPictureCb.

- The derivation process of the boundary filtering strength specified in clause 8.5.2.5 is invoked with the picture sample array recPicture, the luma location ( xCb, yCb ), the coding block width nCbW, the coding block height nCbH, the variable edgeType, and the array edgeFlags as inputs, and an (nCbW)x(nCbH) array verBs as output.

…

      - Derivation process of transform block boundary

Inputs to this process are:

- a location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a location ( xB0, yB0 ) specifying the top-left sample of the current block relative to the top-left sample of the current coding block,

- a variable nTbW specifying the width of the current block,

- a variable nTbH specifying the height of the current block,

- a variable treeType specifying whether a single tree (SINGLE_TREE) or a dual tree is used to partition the CTUs and, when a dual tree is used, whether the luma (DUAL_TREE_LUMA) or chroma components (DUAL_TREE_CHROMA) are currently processed,

- a variable filterEdgeFlag,

- a two-dimensional (nCbW)x(nCbH) array edgeFlags,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered.

Output of this process is the modified two-dimensional (nCbW)x(nCbH) array edgeFlags.

The maximum transform block size maxTbSize is derived as follows:

maxTbSize = ( treeType  = =  DUAL_TREE_CHROMA ) ? MaxTbSizeY / 2 : MaxTbSizeY	(8-835)

Depending on maxTbSize, the following applies:

- If nTbW is greater than maxTbSize or nTbH is greater than maxTbSize, the following ordered steps apply.

- The variables newTbW and newTbH are derived as follows:

newTbW = ( nTbW  >  maxTbSize ) ? ( nTbW / 2 ) : nTbW	(8-836)

newTbH = ( nTbH   >  maxTbSize ) ? ( nTbH / 2 ) :  nTbH	(8-837)

- The derivation process of transform block boundary as specified in this clause is invoked with the location ( xB0, yB0 ), the variables nTbW set equal to newTbW and nTbH set equal to newTbH, the variable filterEdgeFlag, the array edgeFlags, and the variable edgeType as inputs, and the output is the modified version of array edgeFlags.

- If nTbW is greater than maxTbSize, the derivation process of transform block boundary as specified in this clause is invoked with the luma location ( xTb0, yTb0 ) set equal to ( xTb0 + newTbW, yTb0 ), the variables nTbW set equal to newTbW and nTbH set equal to newTbH, the variable filterEdgeFlag, the array edgeFlags and the variable edgeType as inputs, and the output is the modified version of array edgeFlags.

- If nTbH is greater than maxTbSize, the derivation process of transform block boundary as specified in this clause is invoked with the luma location ( xTb0, yTb0 ) set equal to ( xTb0, yTb0 + newTbH ), the variables nTbW set equal to newTbW and nTbH set equal to newTbH, the variable filterEdgeFlag, the array edgeFlags and the variable edgeType as inputs, and the output is the modified version of array edgeFlags.

- If nTbW is greater than maxTbSize and nTbH is greater than maxTbSize, the derivation process of transform block boundary as specified in this clause is invoked with the luma location ( xTb0, yTb0 ) set equal to ( xTb0 + newTbW, yTb0 + newTbH ), the variables nTbW set equal to newTbW and nTbH set equal to newTbH, the variable filterEdgeFlag, the array edgeFlags and the variable edgeType as inputs, and the output is the modified version of array edgeFlags.

–	Otherwise, the following applies:

- If edgeType is equal to EDGE_VER, the value of edgeFlags[ xB0 ][ yB0 + k ] for k = 0..nTbH − 1 is derived as follows:

- If xB0 is equal to 0, edgeFlags[ xB0 ][ yB0 + k ] is set equal to filterEdgeFlag.

- Otherwise, edgeFlags[ xB0 ][ yB0 + k ] is set equal to 1.

- Otherwise (edgeType is equal to EDGE_HOR), the value of edgeFlags[ xB0 + k ][ yB0 ] for k = 0..nTbW − 1 is derived as follows:

- If vr360_cubemap_3x2_layout_flag is equal to 1 and ( yCb + yB0 ) is equal to ( pic_height_in_luma_samples >> 1 ), edgeFlags[ xB0 + k ][ yB0 ] is set 0.

- Else if If yB0 is equal to 0, edgeFlags[ xB0 + k ][ yB0 ] is set equal to filterEdgeFlag.

- Otherwise, edgeFlags[ xB0 + k ][ yB0 ] is set equal to 1.

      - Derivation process of coding subblock boundary

Inputs to this process are:

- a location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a variable nCbW specifying the width of the current coding block,

- a variable nCbH specifying the height of the current coding block,

- a two-dimensional (nCbW)x(nCbH) array edgeFlags,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered.

Output of this process is the modified two-dimensional (nCbW)x(nCbH) array edgeFlags.

The number of coding subblock in horizontal direction numSbX and in vertical direction numSbY are derived as follows:

- If CuPredMode[ xCb ][ yCb ]  = =  MODE_INTRA, numSbX and numSbY are both set equal to 1.

- Otherwise, numSbX and numSbY are set equal to NumSbX[ xCb ][ yCb ] and NumSbY[ xCb ][ yCb ], respectively.

Depending on the value of edgeType the following applies:

- If edgeType is equal to EDGE_VER and numSbX is greater than 1, the following applies for i = 1..min( ( nCbW / 8 ) − 1, numSbX − 1), k = 0..nCbH − 1:

edgeFlags[ i * Max( 8, nCbW / numSbX ) ][ k ] = 1	(8-838)

- Otherwise if edgeType is equal to EDGE_HOR and numSbY is greater than 1, the following applies for j = 1..min( ( nCbH / 8 ) − 1, numSbY − 1 ), k = 0..nCbW − 1:

- if vr360_cubemap_3x2_layout_flag is equal to 1 and ( j * Max( 8, nCbH / numSbY ) is equal to ( pic_height_in_luma_samples >> 1 ):

edgeFlags[ k ][ j * Max( 8, nCbH / numSbY ) ] = 0

- else :

edgeFlags[ k ][ j * Max( 8, nCbH / numSbY ) ] = 1	(8-839)

…

    - Sample adaptive offset process

General

Inputs to this process are the reconstructed picture sample array prior to sample adaptive offset recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr.

Outputs of this process are the modified reconstructed picture sample array after sample adaptive offset saoPictureL and, when ChromaArrayType is not equal to 0, the arrays saoPictureCb and saoPictureCr.

This process is performed on a CTB basis after the completion of the deblocking filter process for the decoded picture.

The sample values in the modified reconstructed picture sample array saoPictureL and, when ChromaArrayType is not equal to 0, the arrays saoPictureCb and saoPictureCr are initially set equal to the sample values in the reconstructed picture sample array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr, respectively.

For every CTU with CTB location ( rx, ry ), where rx = 0..PicWidthInCtbsY − 1 and ry = 0..PicHeightInCtbsY − 1, the following applies:

–	When slice_sao_luma_flag of the current slice is equal to 1, the CTB modification process as specified in clause 8.5.3.2 is invoked with recPicture set equal to recPictureL, cIdx set equal to 0, ( rx, ry ), and both nCtbSw and nCtbSh set equal to CtbSizeY as inputs, and the modified luma picture sample array saoPictureL as output.

–	When ChromaArrayType is not equal to 0 and slice_sao_chroma_flag of the current slice is equal to 1, the CTB modification process as specified in clause 8.5.3.2 is invoked with recPicture set equal to recPictureCb, cIdx set equal to 1, ( rx, ry ), nCtbSw set equal to ( 1  <<  CtbLog2SizeY ) / SubWidthC and nCtbSh set equal to ( 1  <<  CtbLog2SizeY ) / SubHeightC as inputs, and the modified chroma picture sample array saoPictureCb as output.

–	When ChromaArrayType is not equal to 0 and slice_sao_chroma_flag of the current slice is equal to 1, the CTB modification process as specified in clause 8.5.3.2 is invoked with recPicture set equal to recPictureCr, cIdx set equal to 2, ( rx, ry ), nCtbSw set equal to ( 1  <<  CtbLog2SizeY ) / SubWidthC and nCtbSh set equal to ( 1  <<  CtbLog2SizeY ) / SubHeightC as inputs, and the modified chroma picture sample array saoPictureCr as output.

CTB modification process

Inputs to this process are:

–	the picture sample array recPicture for the colour component cIdx,

–	a variable cIdx specifying the colour component index,

–	a pair of variables ( rx, ry ) specifying the CTB location,

–	the CTB width nCtbSw and height nCtbSh.

Output of this process is a modified picture sample array saoPicture for the colour component cIdx.

The variable bitDepth is derived as follows:

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

The picture height pic_height is derived as follows:

pic_height = ( cIdx == 0 ) ? ( pic_height_in_luma_samples ) : ( pic_height_in_luma_samples / SubHeightC )

The location ( xCtb, yCtb ), specifying the top-left sample of the current CTB for the colour component cIdx relative to the top-left sample of the current picture component cIdx, is derived as follows:

( xCtb, yCtb ) = ( rx * nCtbSw, ry * nCtbSh )		(8-905)

The sample locations inside the current CTB are derived as follows:

( xSi, ySj ) = ( xCtb + i, yCtb + j )		(8-906)

( xYi, yYj ) = ( cIdx  = =  0 ) ? ( xSi, ySj ) : ( xSi * SubWidthC, ySj * SubHeightC )	(8-907)

For all sample locations ( xSi, ySj ) and ( xYi, yYj ) with i = 0..nCtbSw − 1 and j = 0..nCtbSh − 1, depending on the values of pcm_loop_filter_disabled_flag, pcm_flag[ xYi ][ yYj ] and cu_transquant_bypass_flag of the coding unit which includes the coding block covering recPicture[ xSi ][ ySj ], the following applies:

–	If one or more of the following conditions are true, saoPicture[ xSi ][ ySj ] is not modified:

- pcm_loop_filter_disabled_flag and pcm_flag[ xYi ][ yYj ] are both equal to 1.

- cu_transquant_bypass_flag is equal to 1.

- SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 0.

- vr360_cubemap_3x2_layout_flag is equal to 1 and [ ySj ] is equal to ( ( pic_height >> 1 ) -1 ) and SaoTypeIdx[cIdx ][ rx ][ ry ] is equal to 2 and SaoEoClass[ cIdx ][ rx ][ ry ] is not equal to 0.

- vr360_cubemap_3x2_layout_flag is equal to 1 and [ ySj ] is equal to ( pic_height >> 1 ) and SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 2 and SaoEoClass[ cIdx ][ rx ][ ry ] is not equal to 0.

[Ed. (BB): Modify highlighted sections prending on future decision transform/quantizaion bypass.]

–	Otherwise, if SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 2, the following ordered steps apply:

- The values of hPos[ k ] and vPos[ k ] for k = 0..1 are specified in Table 8-14 based on SaoEoClass[ cIdx ][ rx ][ ry ].

- The variable edgeIdx is derived as follows:

- The modified sample locations ( xSik′, ySjk′ ) and ( xYik′, yYjk′ ) are derived as follows:

( xSik′, ySjk′ ) = ( xSi + hPos[ k ], ySj + vPos[ k ] )	(8-908)

( xYik′, yYjk′ ) = ( cIdx  = =  0 ) ? ( xSik′, ySjk′ ) : ( xSik′ * SubWidthC, ySjk′ * SubHeightC )	(8-909)

- If one or more of the following conditions for all sample locations ( xSik′, ySjk′ ) and ( xYik′, yYjk′ ) with k = 0..1 are true, edgeIdx is set equal to 0:

- The sample at location ( xSik′, ySjk′ ) is outside the picture boundaries.

- The sample at location ( xSik′, ySjk′ ) belongs to a different slice and one of the following two conditions is true:

- MinTbAddrZs[ xYik′  >>  MinTbLog2SizeY ][ yYjk′  >>  MinTbLog2SizeY ] is less than MinTbAddrZs[ xYi  >>  MinTbLog2SizeY ][ yYj  >>  MinTbLog2SizeY ] and slice_loop_filter_across_slices_enabled_flag in the slice which the sample recPicture[ xSi ][ ySj ] belongs to is equal to 0.

- MinTbAddrZs[ xYi  >>  MinTbLog2SizeY ][ yYj  >>  MinTbLog2SizeY ] is less than MinTbAddrZs[ xYik′  >>  MinTbLog2SizeY ][ yYjk′  >>  MinTbLog2SizeY ] and slice_loop_filter_across_slices_enabled_flag in the slice which the sample recPicture[ xSik′ ][ ySjk′ ] belongs to is equal to 0.

- loop_filter_across_tiles_enabled_flag is equal to 0 and the sample at location ( xSik′, ySjk′ ) belongs to a different tile.

[Ed. (BB): Modify highlighted sections when tiles without slices are incorporated]

- Otherwise, edgeIdx is derived as follows:

- The following applies:

edgeIdx = 2 + Sign( recPicture[ xSi ][ ySj ] − recPicture[ xSi + hPos[ 0 ] ][ ySj + vPos[ 0 ] ] ) +
		Sign( recPicture[ xSi ][ ySj ] − recPicture[ xSi + hPos[ 1 ] ][ ySj + vPos[ 1 ] ] )	(8-910)

- When edgeIdx is equal to 0, 1, or 2, edgeIdx is modified as follows:

edgeIdx = ( edgeIdx  = =  2 ) ? 0 : ( edgeIdx + 1 )	(8-911)

- The modified picture sample array saoPicture[ xSi ][ ySj ] is derived as follows:

saoPicture[ xSi ][ ySj ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, recPicture[ xSi ][ ySj ] +
		SaoOffsetVal[ cIdx ][ rx ][ ry ][ edgeIdx ] )	(8-912)

- Otherwise (SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 1), the following ordered steps apply:

- The variable bandShift is set equal to bitDepth − 5.

- The variable saoLeftClass is set equal to sao_band_position[ cIdx ][ rx ][ ry ].

- The list bandTable is defined with 32 elements and all elements are initially set equal to 0. Then, four of its elements (indicating the starting position of bands for explicit offsets) are modified as follows:

for( k = 0; k < 4; k++ )
	bandTable[ ( k + saoLeftClass ) & 31 ] = k + 1	(8-913)

- The variable bandIdx is set equal to bandTable[ recPicture[ xSi ][ ySj ]  >>  bandShift ].

- The modified picture sample array saoPicture[ xSi ][ ySj ] is derived as follows:

saoPicture[ xSi ][ ySj ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, recPicture[ xSi ][ ySj ] +
			SaoOffsetVal[ cIdx ][ rx ][ ry ][ bandIdx ] )	(8-914)

Table 8-19 – Specification of hPos and vPos according to the sample adaptive offset class

| SaoEoClass[ cIdx ][ rx ][ ry ] | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| hPos[ 0 ] | −1 | 0 | −1 | 1 |
| hPos[ 1 ] | 1 | 0 | 1 | −1 |
| vPos[ 0 ] | 0 | −1 | −1 | −1 |
| vPos[ 1 ] | 0 | 1 | 1 | 1 |

Adaptive loop filter process

General

Inputs of this process are the reconstructed picture sample arrays prior to adaptive loop filter recPictureL, recPictureCb and recPictureCr.

Outputs of this process are the modified reconstructed picture sample arrays after adaptive loop filter alfPictureL, alfPictureCb and alfPictureCr.

The sample values in the modified reconstructed picture sample arrays after adaptive loop filter alfPictureL, alfPictureCb and alfPictureCr are initially set equal to the sample values in the reconstructed picture sample arrays prior to adaptive loop filter recPictureL, recPictureCb and recPictureCr, respectively.

When tile_group_alf_enabled_flag is equal to 1, for every coding tree unit with luma coding tree block location ( rx, ry ), where rx = 0..PicWidthInCtbs − 1 and ry = 0..PicHeightInCtbs − 1, the following applies:

  - When alf_ctb_flag[ 0 ][ rx ][ ry ] is equal to 1, the coding tree block filtering process for luma samples as specified in clause 8.6.4.2 is invoked with recPictureL, alfPictureL, and the luma coding tree block location ( xCtb, yCtb ) set equal to ( rx  <<  CtbLog2SizeY, ry  <<  CtbLog2SizeY ) as inputs, and the output is the modified filtered picture alfPictureL.

  - When alf_ctb_flag[ 1 ][ rx ][ ry ] is equal to 1, the coding tree block filtering process for chroma samples as specified in clause 8.6.4.4 is invoked with recPicture set equal to recPictureCb, alfPicture set equal to alfPictureCb, and the chroma coding tree block location ( xCtbC, yCtbC ) set equal to ( rx  <<  ( CtbLog2SizeY − 1 ), ry  <<  ( CtbLog2SizeY − 1 ) ) as inputs, and the output is the modified filtered picture alfPictureCb.

  - When alf_ctb_flag[ 2 ][ rx ][ ry ] is equal to 1, the coding tree block filtering process for chroma samples as specified in clause 8.6.4.4 is invoked with recPicture set equal to recPictureCr, alfPicture set equal to alfPictureCr, and the chroma coding tree block location ( xCtbC, yCtbC ) set equal to ( rx  <<  ( CtbLog2SizeY − 1 ), ry  <<  ( CtbLog2SizeY − 1 ) ) as inputs, and the output is the modified filtered picture alfPictureCr.

Coding tree block filtering process for luma samples

Inputs of this process are:

- a reconstructed luma picture sample array recPictureL prior to the adaptive loop filtering process,

- a filtered reconstructed luma picture sample array alfPictureL,

- a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top left sample of the current picture.

Output of this process is the modified filtered reconstructed luma picture sample array alfPictureL.

The derivation process for filter index clause 8.6.4.3 is invoked with the location ( xCtb, yCtb ) and the reconstructed luma picture sample array recPictureL as inputs, and filtIdx[ x ][ y ] and transposeIdx[ x ][ y ] with x, y = 0..CtbSizeY − 1 as outputs.

For the derivation of the filtered reconstructed luma samples alfPictureL[ x ][ y ], each reconstructed luma sample inside the current luma coding tree block recPictureL[ x ][ y ] is filtered as follows with x, y = 0..CtbSizeY − 1:

  - The array of luma filter coefficients f[ j ] corresponding to the filter specified by filtIdx[ x ][ y ] is derived as follows with j = 0..12:

f[ j ] = AlfCoeffL[ filtIdx[ x ][ y ] ][ j ]		(8-942)

  - The luma filter coefficients filterCoeff are derived depending on transposeIdx[ x ][ y ] as follows:

  - If transposeIndex[ x ][ y ] = = 1,

filterCoeff[ ] = { f[9], f[4], f[10], f[8], f[1], f[5], f[11], f[7], f[3], f[0], f[2], f[6], f[12] }	(8-943)

  - Otherwise, if transposeIndex[ x ][ y ] = = 2,

filterCoeff[ ] = { f[0], f[3], f[2], f[1], f[8], f[7], f[6], f[5], f[4], f[9], f[10], f[11], f[12] }	(8-944)

  - Otherwise, if transposeIndex[ x ][ y ] = = 3,

filterCoeff[ ] = { f[9], f[8], f[10], f[4], f[3], f[7], f[11], f[5], f[1], f[0], f[2], f[6], f[12] }	(8-945)

  - Otherwise,

filterCoeff[ ] = { f[0], f[1], f[2], f[3], f[4], f[5], f[6], f[7], f[8], f[9], f[10], f[11], f[12] }	(8-946)

  - The locations ( hx, vy ) for each of the corresponding luma samples ( x, y ) inside the given array recPicture of luma samples are derived as follows:

hx = Clip3( 0, pic_width_in_luma_samples − 1, xCtb + x )	(8-947)

  - If vr360_cubemap_3x2_layout_flag is equal to 1,

    - If yCtb is smaller than (pic_height_in_luma_samples >> 1),

vy = Clip3( 0, (pic_height_in_luma_samples >> 1) − 1, yCtb + y )

    - Otherwise,

vy = Clip3( pic_height_in_luma_samples >> 1, pic_height_in_luma_samples − 1, yCtb + y )

  - Otherwise,

vy = Clip3( 0, pic_height_in_luma_samples − 1, yCtb + y )	(8-948)

  - The variable sum is derived as follows:

sum = filterCoeff[ 0 ]   * ( recPictureL[ hx, vy + 3 ] + recPictureL[ hx, vy − 3 ] ) + 
	filterCoeff[ 1 ]   * ( recPictureL[ hx + 1, vy + 2 ] + recPictureL[ hx − 1, vy − 2 ] ) + 
	filterCoeff[ 2 ]   * ( recPictureL[ hx, vy + 2 ] + recPictureL[ hx, vy − 2 ] ) + 
	filterCoeff[ 3 ]   * ( recPictureL[ hx − 1, vy + 2 ] + recPictureL[ hx + 1, vy − 2 ] ) + 
	filterCoeff[ 4 ]   * ( recPictureL[ hx + 2, vy + 1 ] + recPictureL[ hx − 2, vy − 1 ] ) + 
	filterCoeff[ 5 ]   * ( recPictureL[ hx + 1, vy + 1 ] + recPictureL[ hx − 1, vy − 1 ] ) + 
	filterCoeff[ 6 ]   * ( recPictureL[ hx, vy + 1 ] + recPictureL[ hx, vy − 1 ] ) + 	(8-949)
	filterCoeff[ 7 ]   * ( recPictureL[ hx − 1, vy + 1 ] + recPictureL[ hx + 1, vy − 1 ] ) + 
	filterCoeff[ 8 ]   * ( recPictureL[ hx − 2, vy + 1 ] + recPictureL[ hx + 2, vy − 1 ] ) + 
	filterCoeff[ 9 ]   * ( recPictureL[ hx + 3, vy ] + recPictureL[ hx − 3, vy ] ) + 
	filterCoeff[ 10 ] * ( recPictureL[ hx + 2, vy ] + recPictureL[ hx − 2, vy ] ) + 
	filterCoeff[ 11 ] * ( recPictureL[ hx + 1, vy ] + recPictureL[ hx − 1, vy ] ) + 
	filterCoeff[ 12 ] *   recPictureL[ hx, vy ]

sum = ( sum + 64 ) >> 7		(8-950)

  - The modified filtered reconstructed luma picture sample alfPictureL[ xCtb + x ][ yCtb + y ] is derived as follows:

alfPictureL[ xCtb + x ][ yCtb + y ] = Clip3( 0, ( 1 << BitDepthY ) − 1, sum )	(8-951)

Derivation process for ALF transpose and filter index for luma samples

Inputs of this process are:

- a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top left sample of the current picture,

- a reconstructed luma picture sample array recPictureL prior to the adaptive loop filtering process.

Outputs of this process are

- the classification filter index array filtIdx[ x ][ y ] with x, y = 0..CtbSizeY − 1,

- the transpose index array transposeIdx[ x ][ y ] with x, y = 0..CtbSizeY − 1.

The locations ( hx, vy ) for each of the corresponding luma samples ( x, y ) inside the given array recPicture of luma samples are derived as follows:

hx = Clip3( 0, pic_width_in_luma_samples − 1, x )		(8-952)

  - If vr360_cubemap_3x2_layout_flag is equal to 1,

    - If yCtb is smaller than (pic_height_in_luma_samples >> 1),

vy = Clip3( 0, (pic_height_in_luma_samples >> 1) − 1, y )

    - Otherwise,

vy = Clip3( pic_height_in_luma_samples >> 1, pic_height_in_luma_samples − 1, y )

  - Otherwise,

vy = Clip3( 0, pic_height_in_luma_samples − 1, y )		(8-953)

The classification filter index array filtIdx and the transpose index array transposeIdx are derived by the following ordered steps:

- The variables filtH[ x ][ y ], filtV[ x ][ y ], filtD0[ x ][ y ] and filtD1[ x ][ y ] with x, y = − 2..CtbSizeY + 1 are derived as follows:

- If both x and y are even numbers or both x and y are uneven numbers, the following applies:

filtH[ x ][ y ] = Abs(  ( recPicture[ hxCtb+x, vyCtb+y ] << 1 ) − recPicture[ hxCtb+x−1, vyCtb+y ] − 	(8-954)
	 recPicture[ hxCtb+x+1, vyCtb+y ]  )

filtV[ x ][ y ] = Abs(  ( recPicture[ hxCtb+x, vyCtb+y ] << 1 ) − recPicture[ hxCtb+x, vyCtb+y−1 ] − 	(8-955)
	 recPicture[ hxCtb+x, vyCtb+y+1 ]  )

filtD0[ x ][ y ] = Abs(  ( recPicture[ hxCtb+x, vyCtb+y ] << 1 ) − recPicture[ hxCtb+x−1, vyCtb+y−1 ] − 	(8-956)
	recPicture[ hxCtb+x+1, vyCtb+y+1 ]  )

filtD1[ x ][ y ] = Abs(  ( recPicture[ hxCtb+x, vyCtb+y ] << 1 ) − recPicture[ hxCtb+x+1, vyCtb+y−1 ] − 	(8-957)
	recPicture[ hxCtb+x−1, vyCtb+y+1 ]  )

- Otherwise, filtH[ x ][ y ], filtV[ x ][ y ], filtD0[ x ][ y ] and filtD1[ x ][ y ] are set equal to 0.

- The variables varTempH1[ x ][ y ], varTempV1[ x ][ y ], varTempD01[ x ][ y ], varTempD11[ x ][ y ] and varTemp[ x ][ y ] with x, y = 0..( CtbSizeY  − 1 ) >> 2 are derived as follows:

sumH[ x ][ y ] = ij filtH[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-958)

sumV[ x ][ y ] = ij filtV[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-959)

sumD0[ x ][ y ] = ij filtD0[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-960)

sumD1[ x ][ y ] = ij filtD1[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-961)

sumOfHV[ x ][ y ] = sumH[ x ][ y ] + sumV[ x ][ y ]	(8-962)

- The variables dir1[ x ][ y ], dir2[ x ][ y ] and dirS[ x ][ y ] with x, y = 0..CtbSizeY − 1 are derived as follows:

- The variables hv1, hv0 and dirHV are derived as follows:

- If sumV[ x >> 2 ][ y >> 2 ] is greater than sumH[ x >> 2 ][ y >> 2 ], the following applies:

hv1 = sumV[ x >> 2 ][ y >> 2 ]		(8-963)

hv0 = sumH[ x >> 2 ][ y >> 2 ] 		8-964)

dirHV = 1		(8-965)

- Otherwise, the following applies:

hv1 = sumH[ x >> 2 ][ y >> 2 ]		(8-966)

hv0 = sumV[ x >> 2 ][ y >> 2 ]		(8-967)

dirHV = 3		(8-968)

- The variables d1, d0 and dirD are derived as follows:

- If sumD0[ x >> 2 ][ y >> 2 ] is greater than sumD1[ x >> 2 ][ y >> 2 ], the following applies:

d1 = sumD0[ x >> 2 ][ y >> 2 ]	(8-969)

d0 = sumD1[ x >> 2 ][ y >> 2 ] 	(8-970)

dirD = 0		(8-971)

- Otherwise, the following applies:

d1 = sumD1[ x >> 2 ][ y >> 2 ]		(8-972)

d0 = sumD0[ x >> 2 ][ y >> 2 ]		(8-973)

dirD = 2		(8-974)

- The variables hvd1, hvd0, are derived as follows:

hvd1 = ( d1 * hv0 > hv1 * d0 )  ?  d1  :  hv1	(8-975)

hvd0 = ( d1 * hv0 > hv1 * d0 )  ?  d0  :  hv0	(8-976)

- The variables dirS[ x ][ y ], dir1[ x ][ y ] and dir2[ x ][ y ] derived as follows:

dir1[ x ][ y ] = ( d1 * hv0 > hv1 * d0 )  ?  dirD  :  dirHV	(8-977)

dir2[ x ][ y ] = ( d1 * hv0 > hv1 * d0 )  ?  dirHV  :  dirD	(8-978)

dirS[ x ][ y ] = ( hvd1 > 2 * hvd0 )  ?  1  :  ( ( hvd1 * 2 > 9 * hvd0 )  ?  2  :  0 )	(8-979)

- The variable avgVar[ x ][ y ] with x, y = 0..CtbSizeY − 1 is derived as follows:

varTab[ ] = { 0, 1, 2, 2, 2, 2, 2, 3, 3, 3, 3, 3, 3, 3, 3, 4 }	(8-980)

avgVar[ x ][ y ] = varTab[ Clip3( 0, 15, ( sumOfHV[ x >> 2 ][ y >> 2 ] * 64 ) >> ( 3 + BitDepthY ) ) ]	(8-981)

- The classification filter index array filtIdx[ x ][ y ] and the transpose index array transposeIdx[ x ][ y ] with x = y = 0..CtbSizeY − 1 are derived as follows:

transposeTable[ ] = { 0, 1, 0, 2, 2, 3, 1, 3 }

transposeIdx[ x ][ y ] = transposeTable[ dir1[ x ][ y ] * 2 + ( dir2[ x ][ y ] >> 1 ) ]

filtIdx[ x ][ y ] = avgVar[ x ][ y ]

When dirS[ x ][ y ] is not equal 0, filtIdx[ x ][ y ] is modified as follows:

filtIdx[ x ][ y ] += ( ( ( dir1[ x ][ y ] & 0x1 ) << 1 ) + dirS[ x ][ y ] ) * 5	(8-982)

Coding tree block filtering process for chroma samples

Inputs of this process are:

- a reconstructed chroma picture sample array recPicture prior to the adaptive loop filtering process,

- a filtered reconstructed chroma picture sample array alfPicture,

- a chroma location ( xCtbC, yCtbC ) specifying the top-left sample of the current chroma coding tree block relative to the top left sample of the current picture.

Output of this process is the modified filtered reconstructed chroma picture sample array alfPicture.

The size of the current chroma coding tree block ctbSizeC is derived as follows:

ctbSizeC = CtbSizeY / SubWidthC		(8-983)

For the derivation of the filtered reconstructed chroma samples alfPicture[ x ][ y ], each reconstructed chroma sample inside the current chroma coding tree block recPicture[ x ][ y ] is filtered as follows with x, y = 0..ctbSizeC − 1:

  - The locations ( hx, vy ) for each of the corresponding chroma samples ( x, y ) inside the given array recPicture of chroma samples are derived as follows:

hx = Clip3( 0, pic_width_in_luma_samples / SubWidthC − 1, xCtbC + x )	(8-984)

  - If vr360_cubemap_3x2_layout_flag is equal to 1,

    - If yCtbC is smaller than (pic_height_in_luma_samples / SubHeightC >> 1),

vy = Clip3( 0, (pic_height_in_luma_samples / SubHeightC >> 1) − 1, yCtbC + y )

    - Otherwise,

vy = Clip3( pic_height_in_luma_samples / SubHeightC >> 1, pic_height_in_luma_samples − 1, yCtbC + y )

  - Otherwise,

vy = Clip3( 0, pic_height_in_luma_samples / SubHeightC − 1, yCtbC + y )	(8-985)

  - The variable sum is derived as follows:

sum = AlfCoeffC[ 0 ] * ( recPicture[ hx, vy + 2 ] + recPicture[ hx, vy − 2 ] ) + 
	AlfCoeffC[ 1 ] * ( recPicture[ hx + 1, vy + 1 ] + recPicture[ hx − 1, vy − 1 ] ) + 
	AlfCoeffC[ 2 ] * ( recPicture[ hx, vy + 1 ] + recPicture[ hx, vy − 1 ] ) + 	(8-986)
	AlfCoeffC[ 3 ] * ( recPicture[ hx − 1, vy + 1 ] + recPicture[ hx + 1, vy − 1 ] ) + 
	AlfCoeffC[ 4 ] * ( recPicture[ hx + 2, vy ] + recPicture[ hx − 2, vy ] ) + 
	AlfCoeffC[ 5 ] * ( recPicture[ hx + 1, vy ] + recPicture[ hx − 1, vy ] ) + 
	AlfCoeffC[ 6 ] * recPicture[ hx, vy ]

sum = ( sum + 64 ) >> 7		(8-987)

  - The modified filtered reconstructed chroma picture sample alfPicture[ xCtbC + x ][ yCtbC + y ] is derived as follows:

alfPicture[ xCtbC + x ][ yCtbC + y ] = Clip3( 0, ( 1 << BitDepthC ) − 1, sum )	(8-988)