| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0369 |
| --- | --- |

| Title: | CE4-related: Syntax changes of merge data |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjo Ahn Donggyu Sim | Tel: Email: | +82-2-941-6470 yjahn@digitalinsights.co.kr dgsim@digitalinsights.co.kr |
| Source: | Digital Insights Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a clean-up of merge data syntaxes in order to maintain consistency according to prediction partitions. In the previous meeting, three kinds of merge list were adopted such as CU merge list, subblock merge list, and triangle merge list. More specifically, there are MMVD list and general CU merge list for CU-level. In the current draft, the order of syntax parsing for merge data is mmvd, subblock merge, multi hypothesis prediction, triangle prediction, and general CU-level merge. This contribution changes the order of merge data syntaxes in order to maintain consistency according to the type of prediction partitions. The proposed changes can achieve 0.1% and 0.2% BD-rate gain in random access configuration and low-delay B configuration, respectively.

# Introduction

In the current draft of VVC, there are three kinds of prediction partitions such as a block, subblocks, and two triangles. It is one of the key features that are different from conventional video coding standards. Figure 1 represents three kinds of prediction partitions for the current VVC.

(a) Block						(b) Subblock				(c) Triangle

Figure 1. Three kinds of prediction partitions in the current draft of VVC

From this point of view, the merge lists can be categorized as CU merge list, subblock merge list, and triangle merge list [1]. More specifically, there are MMVD list and general CU merge list for CU-level. In the current draft, the order of syntax parsing for merge data is mmvd, subblock merge, multi hypothesis prediction, triangle prediction, and general CU-level merge. Figure 2 represents the flowchart of the current parsing process of merge_data(). In Figure 2, syntaxes in green boxes are related to CU merge candidates, syntaxes in red boxes are relate to subblock merge candidates, and syntaxes in yellow boxes are related to triangle merge candidates.

Figure 2. Flowchart of the current parsing process of merge_data()

# Proposed method

In this contirubtion, it is proposed to adjust the order based on the type of prediction partition. By arranging the merge_subblock_flag and the merge_triangle_flag, which are syntaxes related to the type of prediction partition, sequentially, the proposed method can provide flexibility for further syntax changes. The reason that the merge_subblock_flag is positioned at the front is that the coding efficiency of affine prediction and sbTMVP is the highest. In addition, the merge_triangle_flag is placed after that to determine block-level merge and triangle-level merge.

In order to maintain consistency according to the type of prediction partitions, this contribution changes the order of the syntaxes as Table1. In addition, Figure 3 represents the flowchart of the proposed parsing process of merge_data().

Table 1. Proposed changes in Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_triangle_enabled_flag && slice_type = = B && cbWidth * cbHeight >= 64 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] = = 1 ) { |  |
| merge_triangle_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mh_intra_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) |  |
| mh_intra_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| mh_intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| mh_intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Figure 3. Flowchart of the proposed parsing process of merge_data()

# Performance evaluation

The proposed methods were evaluated under JVET common test conditions (CTC) [2]. The proposed methods were integrated on VTM-3.0. Table 2 shows BD-rate reduction of the proposed method. In this test, the proposed methods can achieve 0.1% and 0.2% BD-rate reduction in random access and low-delay B configuration, respectively.

Table 2. BD-rate reduction of the proposed method

|  | Over VTM-3.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.07% | -0.15% | 0.04% |  |  | -0.13% | -0.05% | -0.28% |  |  |
| Class E |  |  |  |  |  | -0.22% | -0.22% | -0.42% |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.12% | -0.13% | -0.22% |  |  | -0.25% | -0.13% | -0.51% |  |  |

# References

- B. Bross, J. Chen, and S. Liu, “Versatile video coding (Draft 3),” JVET-L1001, 12th JVET meeting, Macau, CN, Oct. 2018.

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th JVET meeting, Macau, CN, Oct. 2018.

# Patent rights declaration(s)

Digital Insights Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).